3GPP TSG-SA WG2 Meeting #154AH-E	S2-2300502

# E-meeting, January 16 - 20, 2023											 (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0176 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for Discovery integrated into PC5 unicast link establishment procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The terms used in the Discovery integrated into PC5 unicast link establishment procedure are not appropriate and need to be corrected. The selection of Layer-2 IDs is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the terms in the Discovery integrated into PC5 unicast link establishment procedure. Clarify the selection of Layer-2 IDs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect terms are used and selection of Layer-2 IDs is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.1, 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.7.3.1	General

5G ProSe Communication via 5G ProSe UE-to-UE Relay with Discovery integrated into PC5 unicast link establishment procedure is supported.

For Discovery integrated into PC5 unicast link establishment, when a 5G ProSe End UE allows a 5G ProSe UE-to-UE Relay to be involved in the Direct Communication Request to the other 5G ProSe End UEs, the 5G ProSe End UE indicates it by including a relay_indication in the broadcasted Direct Communication Request message.

When a 5G ProSe UE-to-UE Relay receives a Direct Communication Request including a relay_indication, it decides whether to forward the message according to e.g. Relay Service Code if there is any, Application ID, operator policy per Relay Service Code, signal strength, and local policy.

The procedures for 5G ProSe Discovery integrated into PC5 unicast link establishment via 5G Layer-3 UE-to-UE Relay and 5G ProSe Layer-2 UE-to-UE Relay are described in clause 6.7.3.2 and clause 6.7.3.3 respectively.

* * * * 2nd Change * * * *

6.7.3.2	5G ProSe Discovery integrated into PC5 unicast link establishment procedure via Layer-3 UE-to-UE Relay

Figure 6.7.3.2-1: 5G ProSe Discovery integrated into PC5 unicast link establishment procedure

0.	Service authorization and provisioning are performed for source 5G ProSe Layer-3 End UE, target 5G ProSe Layer-3 End UE and 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.2.

1.	The source 5G ProSe Layer-3 End UE wants to establish a unicast communication with target 5G ProSe Layer-3 End UE and broadcasts a Direct Communication Request. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The Source Layer-2 ID and Destination Layer-2 ID are selected as described in clause 5.8.5.

The relay_indication in the Direct Communication Request is used to indicate whether a 5G ProSe Layer-3 UE-to-UE Relay can forward the Direct Communication Request message or not. It is also used to limit the number of hops of 5G ProSe Layer-3 UE-to-UE Relay by removing relay_indication in the Direct Communication Request message from the 5G ProSe Layer-3 UE-to-UE Relay.

NOTE:	The data type of relay_indication can be determined in Stage 3.

2.	When receiving Direct Communication Request with relay_indication from source 5G ProSe Layer-3 End UE, the 5G ProSe Layer-3 UE-to-UE Relay(s)  may decide to participate in the procedure and broadcast a Direct Communication Request message in its proximity without relay_indication. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The Source Layer-2 ID and Destination Layer-2 ID are selected as described in clause 5.8.5.

3.	When target 5G ProSe Layer-3 End UE receives a Direct Communication Request from one or multiple 5G ProSe Layer-3 UE-to-UE Relays, the target 5G ProSe Layer-3 End UE select a 5G ProSe Layer-3 UE-to-UE Relay which the target 5G ProSe Layer-3 End UE will respond. The target 5G ProSe Layer-3 End UE may select a 5G ProSe Layer-3 UE-to-UE Relay according to the signal strength, local policy, operator policy per Relay Service Code if any.

4.	The security establishment happens between the 5G ProSe Layer-3 End UE and the selected 5G ProSe Layer-3 UE-to-UE Relay, if needed.

Editor's note:	It is FFS how to perform PC5 link modification between UE-2 and relay-1.

5.	The target 5G ProSe Layer-3 End UE replies Direct Communication Accept message to 5G ProSe Layer-3 UE-to-UE Relay. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

The Source Layer-2 ID and Destination Layer-2 ID are selected as described in clause 5.8.5.

6. For IP traffic, IPv6 prefix or IPv4 address is allocated for the target 5G ProSe Layer-3 End UE as defined in clause 5.5.1.3.

7.	The security establishment happens between the source 5G ProSe Layer-3 End UE and the 5G ProSe Layer-3 UE-to-UE Relay, if needed.

Editor's note:	It is FFS how to perform PC5 link modification between UE-1 and relay-1.

8.	The 5G ProSe Layer-3 UE-to-UE Relay responds with Direct Communication Accept to the source 5G ProSe Layer-3 End UE. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

The Source Layer-2 ID and Destination Layer-2 ID are selected as described in clause 5.8.5.

9. For IP traffic, IPv6 prefix or IPv4 address is allocated for the source 5G ProSe Layer-3 End UE as defined in clause 5.5.1.3.

Editor's note:	It is FFS how to support non-IP case.

* * * * End of Changes * * * *