3GPP TSG-WG SA2 Meeting #154-AH e-meeting  	S2-2300530

Elbonia, January 16th – 20th, 2023	(revision of S2-220xxxx)

Title:	[DRAFT] Reply LS on Handling of the Allowed PDU session status IE in Non-allowed service area

Response to:	LS (C1-227197) on Handling of the Allowed PDU session status IE in Non-allowed service area from CT1

Release:	Release18

Work Item:	5GProtoc18

Source:	[Huawei to be] SA2

To:	CT1

Cc:

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	raymond.limeng@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2300531, S2-2300532

1. Overall Description:

SA2 thanks CT1 for the “LS on Handling of the Allowed PDU session status IE in Non-allowed service area (C1-227197)”. SA2 notices that CT1 asks SA2 for clarifications on the following scenario:

- The UE is registered to an AMF both via 3GPP and non-3GPP access. The UE is currently in a non-allowed area. The registration area (TAI list) for 3GPP access includes both TAs from the UE's non-allowed area and TAs from the allowed area. The UE is currently in a non-allowed area. - The UE has at least 1 PDU session activated via non-3GPP access which cannot be transferred to 3GPP access. - The UE is in IDLE mode both for 3GPP and non-3GPP access, when it receives a paging via 3GPP access indicating non-3GPP access and responds with a Service Request.

SA2 thinks the bullet 1) may not be the case, as the Registration Area of a UE in the Non-Allowed Area should only consist of a set of TAs which belongs to a Non-Allowed Area of the UE. SA2 then assumes the case that CT1 is asking is that the UE is currently in a non-allowed area and the registration area (TAI list) for 3GPP access only includes TAs from the UE's non-allowed area.

Given that the PDU session cannot be activated in IDLE mode, SA2 assumes that bullet 2) should be "The UE has at least 1 PDU session established (instead of activated) via non-3GPP access which can be (instead of cannot be, otherwise it is not reasonable for the UE to include the List Of Allowed PDU Sessions) transferred to 3GPP access."

With the assumptions for bullet 1) and 2) indicated above, SA2 thinks that it may not be reasonable for the AMF to trigger the paging. Instead, the AMF shall reject the request from SMF holding the Non-3GPP PDU session context, as the AMF knows the UE now is in the Non-Allowed Area and so the user plane cannot be activated via 3GPP access even if the PDU session of Non-3GPP access type is transferred to 3GPP access type.

Since the paging is not triggered, there is no need to specify the UE side.

2. Actions:

To CT1 group.

ACTION: 	SA2 respectfully requests CT1 to take the above answers into account in the further work.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting Adhoc   	Jan 16 – 20, 2023				Elbonia

TSG-SA2 Meeting #155	Feb 20 – 24, 2023				Athens