3GPP TSG-WG SA2 Meeting #154-AH-E	S2-2300540

# Jan 16 – 20, 2023, Elbonia

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0639 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ML Models Accuracy Monitoring procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add procedures of ML Models Accuracy Monitoring |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedures for AnLF-assisited ML Models Accuracy Monitoring based on conclusion of Key Issue #1 in T 23.700-81. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures for AnLF-assisited MTLF ML Models Accuracy Monitoring and MTLF based ML Models Accuracy Monitoring are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.1(new), 6.2E.2(new), 6.2E.3(new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** Start of Changes ***************

6.2E	MTLF ML Models Accuracy Monitoring procedures

6.2E.1	General

MTLF based ML Models Accuracy Monitoring procedures, using procedure as specified in clause 6.2E.2.

AnLF-assisited MTLF ML Models Accuracy Monitoring procedures, using procedure as specified in clause 6.2E.3.

6.2E.2	Procedure for MTLF ML Models Accuracy Mornitoring

xxxx

6.2E.3	Procedure for AnLF-assisited MTLF ML Models Accuracy Monitoring

Figure 6.2E.3 depicts the procedure for the proposed solution that the AnLF collect input data and ground truth data to calculate Analytics Accuracy Information by comparing the ground truth data and the corresponding inference and provides Analytics Accuracy Information and the corresponding input data and ground truth data to the MTLF, which are used by MTLF to determine ML Model degradation and retrain the existing ML Model.

Figure 6.2.E.3-1: AnLF-asisited MTLF ML Models Accuracy Monitoring

1.	The NWDAF containing AnLF subscribes to a NWDAF conataining MTLF a trained ML Model by invoking the Nnwdaf_MLModelProvision (Analytics ID, ML Model Filter Information) service operation

2-3.	The MTLF trains the model according to the requirement from AnLF and notifies the AnLF with the ML model information by invoking Nnwdaf_MLModelProvision_Notify (Analytic ID, the file address of the trained ML model, ML Model Accuracy Information) service operation.

The ML Model Accuracy Information provided by the NWDAF containing MTLF is to indicate the ML Model Accuracy Information of the trained ML Model in training phase.

NOTE:	The call flow only shows a subscription/notification model for the simplicity, however both request-response and subscription-notification models should be supported.

4.	When receiving subscribtion or request from analytics consumer, the AnLF collects the input data from data providers, e.g. OAM, NFs, to make predictions based on the trained ML Model provided by the MTLF. The AnLF also collects the ground truth data, which is the actual measured data corresponding to the prediction calculated by AnLF.

The AnLF calculates the Analytics Accuracy Infomation, which reflects the performance of ML model used in live network by comparing prediction with the corresponding ground truth data retrieved from the live network by the AnLF. Optionally, the AnLF may take the feedback information received from the consumer NF into account to improve Accuracy Calculating.

Editor’s Note: Whether and how the AnLF store the data (e.g. input data, ground truth and/or predictions) in the ADRF is FFS.

5-7.	The AnLF decides whether or not to send notification to MTLF to retrain the ML model e.g. by determining whether Analytics Accuracy Information cross a threshold (e.g. which is locally configutured in AnLF or is equal to ML model Accuracy Information provided by the MTLF).

When the Analytics Accuracy Information crosses the threshold and the analytics accuracy is degrading, the AnLF notifies the MTLF the Analytics Accuracy Information and the corresponding data e.g. input data and ground truth data, which are used by MTLF to determine ML Model degradation and retrain the existing ML Model.

The MTLF provides the AnLF with the retrained model and ML Model Accuracy Information.

********** End of Changes ***************