3GPP TSG-WG SA2 Meeting #154-AH-e 	S2-2300550

Elbonia, January 16 – 20, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#4 Conclusion update

Document for:	Approval

Agenda Item:	9.5.1

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: This paper proposes the conclusion update for KI#4 Control of Operations for Ranging/Sidelink positioning.

1. Introduction/Discussion

This paper proposes to update the conclusion and handle the ENs for KI#4, based on the RAN LS feedback (R2-2213131).

- Editor's note: Whether group management is supported by the Ranging/SL Positioning layer or the Application layer is FFS.

Discussion: In both ProSe 23.304 and V2X 23.287, the group discovery/formation/management can be carried out in the Application layer. The same logic can be used for ranging service.

Proposal: It proposes to group management is supported by application layer.

- Editor's note: Whether the Ranging/SL Positioning layer is over V2X/ProSe layer or AS layer is up to RAN WG to decide. Whether Layer 2 ID or Application Layer ID is exchanged between Ranging/SL Positioning layer and lower layer depends if the lower layer includes V2X/ProSe layer.

Discussion: RAN WG did not discuss whether Ranging/SL Positioning layer is over V2X/ProSe layer or AS layer yet.

Proposal: It proposes to change the EN to NOTE, and further coordinate with RAN WG during normative phase.

- Editor's note: Whether RSPP is over PC5-S or PC5-U or (partially) over PC5-D is up to RAN WG to decide.

Discussion: In R2-2213131, RAN2 concluded that the transport layer of SLPP is down selected between PDCP and PC5-U.

Proposal: It proposes to update the description aligned with the RAN2 conclusion, and change it into a NOTE.

- Editor's note: Whether the SL Positioning Server functionalities can support more functionalities, e.g. SL Positioning/Ranging method determination, operation coordination, resource coordination and scheduling, in addition to result calculation is FFS.

Discussion: In R2-2213131, RAN2 thinks that, for out-of-coverage scenario, the functionalities of method determination, assistant data distribution and anchor UE selection can be performed by SL positioning server UE.

Proposal: It proposes to update the description aligned with the RAN2 conclusion, and remove the EN.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change* * * *

## 8.4	Key Issue #4: Control of Operations for Ranging/Sidelink positioning

For Key Issue #4 "Control of Operations for Ranging/Sidelink positioning", the followings are taken as conclusions:

-	A Ranging/SL Positioning layer is introduced on the Target UE/Reference UE/SL Positioning Server UE under Application layer and above AS layer to handle service request received from application layer and to control the Sidelink Positioning and Ranging operation:

-	Functionalities supported by the Ranging/SL Positioning layer include discovery of the UE(s) in proximity that can participate in Sidelink Positioning and Ranging service sessions and control signalling between UEs or among a group of UEs or between UE and LMF to manage and coordinate the Sidelink Positioning and Ranging operations.

-	Group management is supported by Application layer.

-	Information exchanged between Ranging/SL Positioning layer and lower layer includes one time or periodic ranging, ranging for distance or direction measurement or both.

NOTE:	Whether the Ranging/SL Positioning layer is over V2X/ProSe layer or AS layer will be decided in normative phase with RAN WG. Whether Layer 2 ID or Application Layer ID is exchanged between Ranging/SL Positioning layer and lower layer depends if the lower layer includes V2X/ProSe layer.

-	Ranging/Sidelink Positioning Protocol (RSPP) is introduced for SR5 over the PC5 reference point between the UEs (i.e. Target UE, Reference UE, Assistant UE, Located UE and SL Positioning Server UE), which includes the following procedures (to be decided by RAN WGs):

-	exchange the Ranging/Sidelink Positioning capability.

-	exchange the Ranging/Sidelink Positioning assistant data.

NOTE 1:	What assistant data are required to be exchanged will be coordinated with RAN WGs.

-	exchange Ranging/Sidelink positioning measurement data/result.

-	V2X Communication procedures as defined in TS 23.287 [3] and 5G ProSe Direct Communication procedures as defined in TS 23.304 [4] are used for the RSPP communication between UEs over PC5.

NOTE:	Whether RSPP is over PDCP or PC5-U is up to RAN WGs to decide.

-	A LMF may be involved when at least one of the Target UE and the Reference UE are in the network coverage. In that case, a suitable LMF needs to be selected by the AMF based on the LMF Ranging/SL Positioning capabilities defined in clause 6.27.2 and the service requests. The protocol used between UE and LMF will be decided by RAN and could be an extension of LPP, a new protocol or both, including the following procedures (to be confirmed by RAN WGs):

-	Triggering Ranging/Sidelink positioning.

-	exchange the Ranging/Sidelink Positioning capability.

-	exchange the Ranging/Sidelink Positioning assistant data.

NOTE 2: what assistant data are required to be exchanged will be coordinated with RAN WGs.

-	exchange Ranging/Sidelink positioning signal measurement data/result.

-	deliver Ranging/Sidelink positioning service request/response.

NOTE 3:	The full support of LPP is not required for operating only SL Positioning/Ranging; in that case only the extension of LPP, a new protocol or both is required.

-	A SL Positioning Server UE can be discovered and selected for result calculation for the case of out of coverage. A SL Positioning Server UE also supports the functionalities of method determination, assistant data distribution and anchor UE selection, as defined by RAN WG.

-	Multiple UEs may be involved in a single Ranging/Sidelink Positioning session.

-	Whether measurement results or Ranging/Sidelink Positioning results are exchanged over the SR5 depends on the negotiation during the control signalling.

# * * * * End of changes * * * *