3GPP TSG-WG SA2 Meeting #154-AH e-meeting  	S2-2300552

Elbonia, January 16th – 20th, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Deletion of EN

Document for:	Approval

Agenda Item:	9.5.1

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: this contribution is to solve the Editor's note on QoS parameters for KI#1 in TR 23.700-86.

1. Introduction/Discussion

There is an Editor's note on QoS parameters for KI#1 in clause 8.1, i.e. “Editor's note: Whether and what QoS parameters used by NG-RAN will be authorised is FFS, and will be checked with RAN WGs.” And in RAN2’s reply LS(R2-2213131), RAN2 requires more information about the meaning of QoS parameters for Service Authorization, i.e. whether it is LCS QoS information or PQI like QoS.

It may need more time for both RAN2 and SA2 to clarify the QoS parameters of Ranging/SL positioning, but it does not impact the overall conclusion of Service Authorization, so it proposes to transfer the Editor’s note to a NOTE to determine the QoS parameters during normative phase.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change (All text new)* * * *

8.1	Key Issue #1: Support of authorisation and policy/parameter provisioning to UE and Key Issue #8: Service Authorization to NG-RAN

For KI #1 and KI #8, the followings are taken as conclusions:

-	The procedures and signalling specified for ProSe in clause 6.2 of TS 23.304 [4] and for V2X in clause 6.2 of TS 23.287 [3] are used to provision the Ranging/SL positioning service authorization and policy/parameter provisioning to the UE.

-	The specific Ranging/SL positioning parameters to UE include:

-	When the UE is served by NG-RAN, the PLMNs in which the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	When the UE is not served by NG-RAN, indicates whether the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and Ranging/SL positioning QoS parameters.

NOTE 1:	The QoS parameters will be coordinated with RAN WGs during the normative phase.

-	The authorized Ranging/SL positioning role per PLMN, e.g. target UE, reference UE, assistant UE, and located UE.

-	When the UE is not served by NG-RAN, indicates the authorized Ranging/SL positioning role, e.g. target UE, reference UE, assistant UE, and located UE.

-	The discovery parameters for Ranging/SL positioning service depends on the conclusions of KI#3.

NOTE 2:	These parameters can be preconfigured in ME/UICC. If same policy/parameters are provided by PCF, the policy/parameters provided by PCF have higher priority than preconfigured parameters.

-	The procedures and signalling specified for ProSe in clause 6.6 of TS 23.304 [4] and for V2X in clause 6.5 of TS 23.287 [3] are used to provision the Ranging/SL positioning service authorization to RAN.

-	The specific Ranging/SL positioning service authorization to RAN include:

-	The "Ranging/SL positioning service authorized" indication based on UE subscription from UDM, indicating the UE is authorized to use Ranging/SL positioning service in the serving PLMN.

NOTE 3:	Whether and what QoS parameters of Ranging/SL positioning used by NG-RAN will be specified during normative phase.

NOTE 4:	The parameters list is not exhausted. Additional parameters can be added during normative phase.

# * * * * End of changes * * * *