3GPP TSG-SA WG2 Meeting #154AH-E	S2-2300561

# Elbonia, January 16 – 20, 2023	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3933 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing cell ID/TAC of MBSR for services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to FS_VMR KI#3 and KI#6 conclusion in TR 23.700-05, the NG-RAN provides an additional ULI for the IAB-UE including TAI/NR CGI to the AMF based on MBSR's donor gNB information when a UE connects to the 5GC via a MBSR. The AMF may use the additional ULI together with the existing ULI to apply the mobility management and Warning Area List management. Also the AMF may determine UE's location by taking ULI for the IAB-UE ULI into account. This contribution addresses how to provide cell ID/TAC of MBSR for services as per the following RAN WG3 agreement(S2-2300057). Static TAC solution is not pursued. RAN3 assumes that dynamic TAC solution should be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe how cell ID/TAC of MBSR is provided for services with dynamic TAC provisioning as per conclusion of TR 23.700-5 and RAN WG3 agreement and provide ULI of IAB-UE to the CN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear whether and how the cell ID/TAC of MBSR is configured and used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.35A.6	Providing cell ID/TAC of MBSR for services

The TAC broadcasted by the MBSR cell(s) is configured by the IAB-donor gNB with one of the cell serving the MBSR where IAB-UE is located. Upon reception of the mobile IAB indication during F1 Setup procedure from the MBSR (i.e. IAB-DU), the IAB-donor gNB shall determine the TA where the IAB-UE is located and send TAC configuration.

The cell broadcasting information is updated accordingly as following:

-	When the MBSR enters to a new TA supported by the same IAB-donor gNB, the IAB-donor gNB provisions the new TAC towards MBSR using GNB CU Configuration Update as specified in TS 38.401 [42].

-	When the MBSR enters to a new TA supported by a different IAB-donor gNB, the IAB-donor gNB configures the cell's TAC of MBSR during F1 Setup procedure.

If the mobile IAB indication is received included in F1 Setup Request message, the IAB-donor gNB shall provide the User Location Information for the IAB-UE including TAI/NR CGI based on a TAC/Cell managed by the IAB-donor gNB to the AMF. The AMF serving the UE may take ULI of IAB-UE together with ULI of UE into account to determine the UE location for services that use cell ID/TAC as location reference (e.g Mobility Management, Registration Area management, etc).

Editor's note:	For the case that IAB-UE and IAB-DU are connected to different IAB-donors, the details are subject to RAN WGs, and will be coordinated with RAN WGs.

* * * * End of Changes * * * *