SA WG2 Meeting #154-AH-e	S2-2300577

# January 16th – 20th, 2023; Elbonia               		     (Revision of S2-230xxxx)

Source: 	China Mobile

Title: 	Update to conclusions on KI#2

Document for: 	Approval

Agenda Item: 	9.16.1

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: Update to resolve the editor’s note in conclusions on KI#2.

1 Discussion

There is one Editor’s Note on clause 8.2 that should be resolved:

Editor's note:	how to support following case is FFS: the case where the consumer of NWDAF service doesn't provide the target DNN, S-NSSAI, but provides the application server IP address/FQDN.

For this EN, according to recent study in FS_EDGE_ph2, the NWDAF may obtain the target DNAI from 5GC to trigger the SMFs discovery in NRF for UPF event exposure if the consumer of NWDAF service doesn't provide the target DNN, S-NSSAI but provides the application server IP address/FQDN. This is for any UE scenario. For single UE scenario, the SMFs discovery is better from UDM since the SUPI is provided. These two scenarios are described separately to remove the EN.

2 Proposal

It is proposed to update TR 23.700-62 v2.0.0 as follows.

Start of change

8.2	Conclusions for KI#2

The following interim conclusions are proposed for KI#2:

1.	Subscription to UPF events via SMF is the rule except for the cases listed in bullet 2; Subscription via SMF means the final consumer of UPF event notifications sends the subscription request to the SMF and then the SMF is doing a third-party subscription onto UPF on behalf of this final consumer. Conversely the notifications are directly sent by the UPF to the final consumer of UPF notifications.

NOTE 1:	Optimizing notifications is more important than optimizing subscriptions.

NOTE 2:	Subscriptions related with AoI are handled by SMF that subscribe/unsubscribe to the relevant UPF(s) on behalf of the final consumer based on whether the UE is in the target AoI (for example, the SMF can first get the UE list within the AoI and keep being updated about the UE list by subscribing to AMF, and then subscribe/unsubscribe to the relevant UPF(s) on behalf of the final consumer). This allows the UPF not having to determine the AMF where to subscribe for UE presence in the AoI.

NOTE 3:	For event subscriptions requiring interactions with 5G AN, a solution where the UPF event consumer would directly subscribe to UPF and then UPF would ask SMF to send N2 SM signalling to 5G AN would be more complex and not bring advantage.

2.	Direct subscription to UPF (i.e. not requiring third party subscription to UPF via SMF) shall be possible for data collection where UPF is the source as defined in TS 23.288 [5], i.e. the following cases:

A.	TS 23.288 [5] Table 6.5.2-2: Data collected by NWDAF for UPF load analytics recalled in item 2 of Annex A of the TR.

B.	For analytics targeting "any UE" (possibly for specific DNN and or slices) and not related with an AoI or BSSID/SSID or with a specific data flow.

NOTE 4:	This can relate to use cases such as Data collected by NWDAF for UPF load analytics, User Data Congestion Analytics, Data Volume dispersion analytics, WLAN performance analytics.

C.	The case described in solution 20 (UE IP address mapping information exposure by UPF) where the 5GC NF (e.g., NEF, BSF) can directly discover the UPF that performs NAT (based on the NATed IP address) and then invoke directly the NAT related API at the UPF. This is an optional feature.

Normative work will be based on NEF contacting NRF and UPF before invoking BSF.

NOTE 5:	Which 5GC NF can directly discover the UPF and invoke the NAT related API will be confirmed during normative phase.

3.	In Rel18:

A.	the only defined consumers of UPF event SUBSCRIBE are SMF, and NWDAF.

B.	the only defined consumers of UPF event notifications are AF/NEF, TSNAF/TSCTSF and NWDAF.

4.	UPF event exposure Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on UPFs.

The following events may be subscribed by a NF consumer:

-	Event: QoS monitoring. This event provides QoS Flow level performance information (information listed in Solution #8, clause 6.8.2).

-	Event: UserDataUsageMeasures. This event provides information of user data usage of the User PDU Session (information listed in Solution #7, clause 6.7.2).

-	Event: UserDataUsageTrends. This event provides statistical measurements (information listed in Solution #7, clause 6.7.2).

When the UPF provides notifications related to UserDataUsageMeasures or UserDataUsageTrends events the notifications may indicate the time stamps for the measures and also refer to the Application Id or Packet Filter Set indicated in the consumer subscription.

5.	The subscription mechanisms for the defined UPF event exposure events are as follows:

-	Event: QoS monitoring. Subscription is always indirect via SMF. UPF and SMF interact using PFCP.

-	Event: UserDataUsageMeasures. SBI based subscription operation shall be used by SMF and other allowed direct consumers.

-	Event: UserDataUsageTrends. SBI based subscription operation shall be used by SMF and other allowed direct consumers.

6.	To determine which SMF to contact the final consumer of UPF events proceeds as follows:

-	If the event targets any UE, the final consumer of UPF events looks up the NRF to discover all suitable SMF(s) (e.g. SMF(s) that serve the target combination of DNN and S-NSSAI).

-	If the event targets a unique UE identified by its SUPI, the final consumer of UPF events sends Nudm_UECM_Get_Request (SUPI, type of requested information set to SMF Registration Info and the S-NSSAI and DNN) to UDM to get the SMF ID serving the target UE.

-	If the event target are UEs identified by a Group Identifier, the final consumer of UPF events sends Nudm_SDM_Get_Request to UDM and requests the list of SUPIs that correspond to the Group ID. Final consumer then proceeds with these SUPIs as described above to get SMF ID serving each UE identified by one of received SUPIs.

If the consumer of NWDAF service doesn't provide the target DNN, S-NSSAI (these parameters are optional for NWDAF service defined in TS 23.288 [5]), and the event targets a unique UE identified by its SUPI, the NWDAF gets the list of all PDU Session for that SUPI via Nudm_UECM_Get_Request and then determine which (DNN, S-NSSAI) it will consider.

If the consumer of NWDAF service doesn't provide the target DNN, S-NSSAI (these parameters are optional for NWDAF service defined in TS 23.288 [5]), but provides the application server IP address/FQDN, and the event targets a unique UE identified by its SUPI or any UE, the NWDAF may obtain the target DNAI from 5GC by the mapping table between Application IP range/address and DNAI as described in clause 8.7 conclusion of TR 23.700-48 [10]. After obtaining DNAI from the NEF by providing AS IP/IP range and/or FQDN, the NWDAF may

-	use the DNAI to trigger the SMFs discovery in NRF in any UE situation, and then send the subscription request to each of these SMFs for UPF event exposure providing Application IP address/FQDN and DNAI.

-	use the SUPI to trigger the SMFs discovery from UDM in single UE situation, and then send the subscription request to each of these SMFs for UPF event exposure providing SUPI, Application IP address/FQDN and DNAI.

7.	For the UPF data collection, the event subscription includes Reporting suggestion information as described in Sol#14, which is used to assist the UPF event notification. Per Reporting suggestion information UPF can concatenate several notification messages to the same notification endpoint in one notification message.

End of change