3GPP TSG-WG SA2 Meeting #154-AH e-meeting  	S2-2300596

Elbonia, January 16th – 20th, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Authorisation and provisioning for Ranging/Sidelink positioning

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: this contribution is to solve the Editor's note on the LMF selection for KI#7 in TR 23.700-86.

1. Introduction/Discussion

The authorisation and provisioning for Ranging/Sidelink positioning is necessary and shall be specified in the normative phase, based on the conclusion of KI#1. From the perspective of high level functionality and feature, it adds the authorisation and provisioning for Ranging/Sidelink positioning as one basic functionality.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change (All text new)* * * *

X	High level functionality and features

X.Y	Authorization and Provisioning for Ranging/SL positioning service

In 5GS, the parameters for Ranging/SL positioning service may be made available to the UE in following ways:

-	provisioned in the ME; or

-	configured in the UICC; or

-	provisioned in the ME and configured in the UICC; or

-	provided or updated by the Ranging/SL positioning Application Server via PCF and/or PC1 reference point; or

-	provided or updated by the PCF to the UE.

If the same parameters are provided by different sources, the UE shall consider them in the following priority order:

-	provided or updated by the PCF (including parameters determined by the PCF itself and parameters provided by the Ranging/SL positioning Application Server to the PCF);

-	provided or updated by the Ranging/SL positioning Application Server via SR1 reference point;

-	configured in the UICC;

-	provisioned in the ME.

The parameters provided or updated by the Ranging/SL positioning Application Server via SR1 reference point may need to be complemented with configuration data from other sources listed above.

NOTE:	The Ranging/SL positioning Application Server can provision the same Ranging/SL positioning parameters via 5GC or directly to the UE via SR1 reference point, and can revoke (e.g. delete) the Ranging/SL positioning parameters via 5GC in order for the provisioning via SR1 reference point to take effect.

The basic principles of service authorization and provisioning for Ranging/SL positioning service are as follows:

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorized to use Ranging/SL positioning.

-	The PCF in the HPLMN merges authorization information from home and other PLMNs and provides the UE with the final authorization information.

-	The PCF in the VPLMN or HPLMN may revoke the authorization (via H-PCF when roaming) at any time by using the UE Configuration Update procedure for transparent UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [X].

-	The Ranging/SL positioning Policy/parameters provisioning to UE is controlled by the PCF and may be triggered by UE. The PCF provisions one or more of the following Policy/parameters:

-	When the UE is served by NG-RAN, the PLMNs in which the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	When the UE is not served by NG-RAN, indicates whether the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and Ranging/SL positioning QoS parameters.

-	The authorized Ranging/SL positioning role per PLMN, e.g. target UE, reference UE, assistant UE, and located UE.

-	When the UE is not served by NG-RAN, indicates the authorized Ranging/SL positioning role, e.g. target UE, reference UE, assistant UE, and located UE.

# * * * * End of changes * * * *