3GPP TSG-SA WG2 Meeting #154AH-e	S2-2300604

# Electronic meeting, 16 – 20 January 2023

Title:	Reply LS on Handling of the Allowed PDU session status IE in Non-allowed service area

Response to:	LS on Handling of the Allowed PDU session status IE in Non-allowed service area (S2-2300029/C1-227197)

Release:	Rel-18

Work Item:	5GProtoc18

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Fei Lu

E-mail Address:	lufei2 at OPPO dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

SA2 thanks CT1 on the LS on Handling of the Allowed PDU session status IE in Non-allowed service area (S2-2300029/C1-227197).

SA2 would like to answer CT1’s question as following:

Question 1: Does the UE need to include the Allowed PDU session status IE also if it responds to the paging when it is in a non-allowed area?

SA2 Answer of Question 1: No, the UE is not allowed to include Allowed PDU session status IE also if it responds to the paging when it is in a non-allowed area (Please refer to clause 5.3.4.1.1 and clause 5.6.8 of TS 23.501).

Question 2: Is the UE allowed to include the Uplink data status IE if it has uplink data pending if it responds to the paging when it is in a non-allowed area?

SA2 Answer of Question 1: No, the UE is not allowed to include the Uplink data status IE if it has uplink data pending if it responds to the paging when it is in a non-allowed area (Please refer to clause 5.3.4.1.1 and clause 5.6.8 of TS 23.501).

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take above into account.

3. Date of Next SA2 Meetings:

SA2#155	20- 24 February 2023		Athens, Greece