3GPP TSG-SA WG2 Meeting #154AH-e	S2-2300605

# Electronic meeting, 16 – 20 January 2023

Title:	Reply LS on Differentiation of Layer2 ID and Coexistence of U2N/U2U

Response to:	LS on Differentiation of Layer2 ID and Coexistence of U2N/U2U (S2-2300052/R2-2213328)

Release:	Rel-18

Work Item:	5G_ProSe_Ph2, NR_SL_relay_enh-Core

Source:	SA2

To:	RAN2

Cc:

Contact Person:

Name:	Fei Lu

E-mail Address:	lufei2 at OPPO.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks RAN2 for the LS on Differentiation of Layer2 ID and Coexistence of U2N/U2U (S2-2300052/R2-2213328).

SA2 would like to answer RAN2’s questions as following:

Question1:

Can a UE be involved in acting as both U2N (as U2N Relay or Remote UE) and U2U (as U2U Relay or Remote UE) at the same time?

SA2 Answer of Question 1:

Yes. There is no restriction for the UE to be acting as both U2N (as U2N Relay or Remote UE) and U2U (as U2U Relay or Remote UE) at the same time.

Question2:

Can the L2ID used for U2U communication be the same as the L2ID for U2N services?

SA2 Answer of Question 2:

No. L2 ID is not shared for different PC5 links.

Question3a:

Whether the Layer2 ID of the relay UE can be the same for U2U Relay service and Non-relay service?

SA2 Answer of Question 3a:

L2 ID is not shared for different PC5 links. However the same PC5 link may be used for both U2U relay service and non-relay service.

Question3b:

Whether the Layer2 ID of the remote UE can be the same for U2U Relay service and Non-relay service?

SA2 Answer of Question 3b:

L2 ID is not shared for different PC5 links. However the same PC5 link may be used for both U2U relay service and non-relay service.

Question4a:

Whether the Layer2 ID of the relay UE can be the same for U2U Relay communication and U2U Relay discovery transmission?

SA2 Answer of Question 4a:

No. Destination Layer2 ID of the relay UE can not the same for U2U Relay communication including with integrity discovery procedure and U2U Relay discovery transmission.

Question4b:

Whether the Layer2 ID of the remote UE can be the same for U2U Relay communication and U2U Relay discovery transmission?

SA2 Answer of Question 4b:

No. Source Layer2 ID of the remote UE can not the same for U2U Relay communication including with integrity discovery procedure and U2U Relay discovery transmission.

2. Actions:

To RAN2:

ACTION: 	SA2 kindly asks RAN2 to take above into account.

3. Date of Next SA2 Meetings:

SA2#155	20- 24 February 2023		Athens, Greece