SA WG2 Meeting #154AHe	S2-2300608

# January 16th – 20th, 2023; Elbonia               	  	 (revision of S2-230)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	KI1 conclusions: AUN3 devices

Document for: 	Approval

Agenda Item: 	9.20.1

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution: KI1 conclusions: AUN3 devices

# 1	Discussion

The conclusions take into account the feedback from BBF (LS in S2-2300038) and from CabeLabs (LS in S2-230xxxx)

# 2 Proposal

It is proposed to update TR 23700-17 as follows (all text is new)

### 8.1.3	Conclusion for AUN3 devices

-	Only 5G RG are considered.

-	following principles related with Solution#25 are endorsed where:

-	each AUN3 device has its own UDM/UDR subscription data and policy control subscription data including its own SUPI and security credential in UDM/UDR (SIDF)

-	the 5G RG may issue a NAS register on behalf of an AUN3 device that it is willing to serve and relay EAP between the AUN3 device and the 5GC.

-	the AMF and the 5G RG maintain a NAS connection per AUN3 device. This includes maintaining a GUTI and NAS security context per AUN3 device

-	A 5G RG shall be registered to serve an AUN3 device

-	there is a N2 connection per AUN3 device

-	AUN3 devices are only allowed to register on some 5G RG (s)

- 	once an AUN3 device has been registered, the 5G RG may request to establish one PDU session to the 5GC for this AUN3 device.

-	QoS differentiation on this PDU session is supported per existing specifications but AUN3 traffic cannot benefit from a better service/QoS than the 5G RG hosting it