3GPP TSG-WG SA2 Meeting #154-AH e-Meeting	S2-2300625

# Elbonia, Online, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3950 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Optimize TR conclusion to introduce UE unreachability period information is provided to UE and AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18 FS_5GSAT_Ph2, the support of mobility management and power saving with discontinuous coverage is studied and concluded. PSM parameters and NAS timer should be determined taking into account UE unreachability period information. Current conclusion includes coverage information can be provided to the UE and UE generate the UE unreachability period information based on the coverage information while UE unreachability period information is directly provided to AMF. The ways of coverage information provision to UE and AMF are different which will increase solution complexity. To make more concisely to the solution, this CR introduces a unified coverage related information (UE unreachability period information) is provided to UE and AMF based on the TR conclusion and DP S2-2300624 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support of UE unreachability period information can be provided to UE and AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If Rel18 only support TR conclusion, it will make complicate to the solution which support of mobility management and power saving with discontinuous coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X (new), 5.4.X.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new) * * * *

5.4.X Support of discontinuous network coverage for satellite access

5.4.X.1	Mobility Management and Power Saving Optimization

In case of LEO, MEO or a satellite constellation that provides discontinuous network coverage, UE unreachability period may be determined, which indicates the timing information for when a UE moves out/in of coverage. The UE unreachability period may consider current and potential future locations of the UE. The determined UE unreachability period information can be provided to both the UE and the AMF.

# * * * * End of changes * * * *