3GPP TSG-SA WG2 Meeting #154-AH-E (e-meeting)	S2-2300650

# January 16th – 20th, 2023, Electronic Meeting

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3957 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.501 PDU session relocation for slice service continuity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusion of KI#1 in TR 23.700-41, this CR aims to add the feature of network slice service continuity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the feature of network slice service continuity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This feature is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.502 CR3765 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of change ***

5.15.x	Support of Network Slice Service Continuity

AMF is responsible to replace a currently used S-NSSAI with an Alternative S-NSSAI either based on local configuration (e.g. based on trigger from OAM) or based on AM PCF/NSSF notification. The Alternative S-NSSAI is determined by the AMF by interaction with AM PCF and/or NSSF during initial registration procedure when AMF query NSSF. The alternative S-NSSAI may or may not be part of the Subscribed S-NSSAIs of the UE. AMF stores the mapping between the Alternative S-NSSAI and Allowed NSSAI UE can access in current registration area, and it will not need to re-visit the NSSF next time unless the access type changes, the mapping information stored in the AMF is missing or UE has moved out of current registration area, etc.

When an old S-NSSAI is determined to be replaced by an Alternative S-NSSAI, UE MM configuration update (e.g. UCU procedure) is performed to include the alternative S-NSSAI in the Allowed NSSAI and/or in the Configured NSSAI. The AMF may provide information that the Alternative S-NSSAI is associated with a replaced/mapped value to the Allowed S-NSSAI in both roaming or non-roaming case. It is independent whether the alternative S-NSSAI is part or not part of the Subscribed S-NSSAIs.

In case of PDU session relocation from the old S-NSSAI to the Alternative S-NSSAI:

-  If UE has no existed PDU session with the replaced S-NSSAI, AMF indicates the alternative S-NSSAI to the SMF and the SMF proceeds with the PDU Session establishment on the alternative S-NSSAI. The alternative S-NSSAI is included in the PDU Session Establishment Accept message, make sure the UE knows the PDU session is associated with both the alternative S-NSSAI and old S-NSSAI.

-  If UE has existing PDU session with the replaced S-NSSAI, AMF indicates the current SMF by triggering Nsmf_PDUSession_UpdateSMContext service operation, then PDU Session is transferred according to different SSC mode. The alternative S-NSSAI is included in the PDU Session Establishment Accept message, make sure the UE knows the PDU session is associated with both the alternative S-NSSAI and old S-NSSAI.

*** End of change ***