3GPP TSG-SA2 #154AH-e	S2-2300661

# Elbonia, January 16th – 20th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3961 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Slice control based on UE inactivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion in TR 23.700-41, the changes listed below are proposed: - Introduce new sub-clause 5.15.x summarizing how to enable selective removal of a given S-NSSAI from an Allowed NSSAI of a UE and release of a PDU Session based on inactivity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add a new clause for enabling selective deregistration and PDU Session release based on inactivity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Selective deregistration and PDU Session release based on UE inacitvity is not supported in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR3770 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5.15.x Slice usage control based on inactivity

Slice usage control based on inactivity is used by the network to ensure efficient slice resource usage. This is required for the purpose of deciding on the following:

- when to release a PDU session, and/or

- when to deregister an S-NSSAI from an Allowed NSSAI of a UE.

When an UE gets successfully registered for an S-NSSAI, the AMF determines whether the indicated S-NSSAI is subject to inactivity-based slice control based on the subscription data received from UDM or network configuration. If yes, the AMF monitors the S-NSSAI inactivity time for Allowed NSSAI control, i.e. when to remove the S-NSSAI from the Allowed NSSAI if no establishment PDU Session associated with the indicated S-NSSAI has happened before the expiration of S-NSSAI Inactivity time for Allowed NSSAI control. In case it is determined that the UE has not any established PDU Sessions associated with the indicated S-NSSAI after the expiry of a configured S-NSSAI Inactivity time, the AMF deregisters the given S-NSSAI by removing it from the Allowed NSSAI of that UE. The AMF also triggers the UE Configuration Update procedure to notify the UE for the removal of that S-NSSAI

In a similar way, for an already established PDU sessions, the SMF determines whether the indicated S-NSSAI is subject to inactivity-based slice control based on the subscription data received from UDM or network configuration. If yes, after having configured the UPF with an appropriate S-NSSAI inactivity time for PDU Session control, when an SMF receives an indication from UPF on expiry of a configured PDU Session Inactivity time that UE has not been transferring any user plane data and the established PDU Session has been inactive for longer than the configured S-NSSAI inactivity time for PDU Session control, the SMF releases that inactive PDU session.

In order to ensure that slice is not removed from an Allowed NSSAI prematurely or a PDU session is not released abruptly, S-NSSAI inactivity for Allowed NSSAI control or S-NSSAI inactivity for PDU Session control parameters can be provisioned by an authorized AF as part of external parameter provision procedure as defined in clause 4.15.6.2 of TS 23.502 [3].

NOTE: 	If S-NSSAI inactivity for Allowed NSSAI control or S-NSSAI inactivity for PDU Session control parameters are not provisioned by the AF but the S-NSSAI is subject to inactivity-based slice control, corresponding times need to be configured at the network.

* * * End of Changes * * *