SA WG2 Meeting #S2-154AH-e	S2-2300687

16 - 20 January 2023, Electronic, Elbonia	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0176 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2, adding MOCN network sharing for the same content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Accoring to the the KI#1 conclusion of TR 23.700-047, the MOCN network sharing for the same content for Broadcast MBS session is adopt for the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause on MOCN sharing optimization desciption. Adding new clause on the procedure enhancement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion for the FS_5MBS_Ph2 is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new), 7.3.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 6.X	MOCN sharing optimization for the same content in Broadcast MBS session

In the clause 5.18 of TS 23.501 [2], multiple CNs share the same NG-RAN, i.e. 5G Multi-Operator Core Network (5G MOCN).

When the same broadcast content is to be delivered to multiple CNs, the AF will set up multiple broadcast MBS sessions towards those CNs, each CN delivering the same content towards the same shared NG-RAN node. There are two metnhods to avoid allocating more radio resource for the same content.

-	Method a): The AF may provide associated session identifier (SSM used by AF) additionally to the NG-RAN nodes via 5GC so that the shared NG-RAN nodes can determine that the multiple broadcast MBS sessions are transmitting same content for the same MBS service, or

-	Method b): The association of MBS session identifiers may be configured in NG-RAN, so that the shared NG-RAN nodes can determine that the multiple broadcast MBS sessions are transmitting same content for the same MBS service.

NOTE:	Depend on the operator policy, all or some the shared delivery tunnels to the same NG RAN from different PLMNs for the same MBS service are established.

Next CHANGE

### 7.3.Y	MOCN sharing optimization

To support MOCN sharing optimization for the same content using method a) defined in the 6.X,

-	MBS session creation procedure is performed as defined in clause 7.1.1.2 and 7.1.1.3 with the following additions: The AF provides associated session identifier (i.e. SSM used by AF) additionally to MB-SMF via NEF/MBSF.

-	MBS Session Start for Broadcast procedure is performed as defined in clause 7.3.1 with the following additions: The MB-SMF provides received associated session identifier to NG-RAN.

-	NG-RAN nodes determine that the multiple broadcast MBS sessions are transmitting same content for the same MBS service according to the associated session identifier.

To support MOCN sharing optimization for the same content using method b) defined in the 6.X,

-	 The association of MBS session identifiers are configured in NG-RAN, so that the shared NG-RAN nodes can determine that the multiple broadcast MBS sessions are transmitting same content for the same MBS service.

END OF CHANGES