3GPP TSG-SA WG2 Meeting #154AHE (e-meeting)	S2-2300707

# Jan 16 – 20, 2023,												 (revision of)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0189 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Negotiated path switching between PC5 and Uu |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This paper tries to realize the conclusion of R18 ProSe ph2 KI#3 (Support direct communication path switching between PC5 and Uu (i.e. non-relay case)) in TR23.700-33. In normative work for the following NOTE: NOTE 1: Whether the path switching policy can be combined into the path selection policy as defined in TS 23.304 [3] will be determined in normative phase. The Path selection policy is considered as path preference (i.e. PC5 preferred, Uu preferred, or no preference), while the path switching policy is consisdered as path permission. It can be understood that the path switching policy is different from the Path selection policy in R17 TS23.304. So it would be better to separate the 2 things with different descriptions. Considering the path switching negotiation process has its own dedicated meaning to determine whether and which ProSe services are to be switched, currrent PC5-S messages seem not suitable to be reused for this process, therefore it would be better to re-define a dedicated PC5-S message for this path switching negotiation process. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Realize the conclusion of R18 ProSe ph2 KI#3 (Support direct communication path switching between PC5 and Uu (i.e. non-relay case)) in TR23.700-33, incouding: - Define the Path switching policy - Define the path switching negotiation process |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion of R18 ProSe ph2 KI#3 (Support direct communication path switching between PC5 and Uu (i.e. non-relay case)) in TR23.700-33 still needs to be reflected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.1, 5.xx (new), 6.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.1.3.1	Policy/Parameter provisioning for 5G ProSe Direct Communication

The following sets of information for 5G ProSe Direct Communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "served by NG-RAN".

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "not served by NG-RAN".

NOTE 1:	In this specification, [When the UE is "served by NG-RAN"] and [When the UE is "not served by NG-RAN"] are relevant to 5G ProSe Direct Communications over NR PC5 reference point.

2)	Groupcast mode 5G ProSe Direct Communication policy/parameters:

-	For each application layer group supported include the parameters that enable the UE to perform Groupcast mode 5G ProSe Direct Communication when provided by PCF or provisioned in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to.

-	ProSe Layer-2 Group ID: Destination Layer-2 ID applicable only when the Application Layer Group ID is provided by the application layer

-	ProSe Group IP multicast address

-	Indication whether the UE should use IPv4 or IPv6 for that group

-	For a specific Group configured to operate using IPv4, optionally an IPv4 address to be used by the UE as a source address. If none is provisioned, then the UE shall use Dynamic Configuration of IPv4 Link-Local Addresses RFC 3927 [18] to obtain a link local address for the Group.

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 2:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

4)	Policy/parameters related to privacy:

-	The list of ProSe services (i.e. ProSe identifiers) with Geographical Area(s) that require privacy support.

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

5)	Policy/parameters when NR PC5 is selected:

-	The mapping of ProSe services (i.e. ProSe identifiers) to radio frequencies with Geographical Area(s).

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for broadcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for groupcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 3:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one ProSe service. In the case where different ProSe services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one ProSe service, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

NOTE 4:	Security policies for Unicast mode 5G ProSe Direct Communication can be provisioned by PCF as defined in TS 33.503 [29].

-	The mapping of ProSe services (i.e. ProSe identifiers) to PC5 QoS parameters defined in clause 5.6.1 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	The mapping of ProSe services (i.e. ProSe identifiers) to the corresponding NR Tx Profiles for broadcast and groupcast (see TS 38.300 [12] and TS 38.331 [16] for further information).

-	AS layer configurations (see TS 38.331 [16]), i.e. the mapping of PC5 QoS profile(s) to radio bearer(s), when the UE is "not served by NG-RAN".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2 of TS 23.287 [2], and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1 of TS 23.287 [2] and in Table 5.6.1-1.

-	For broadcast, groupcast and initial signalling to establish unicast connection, PC5 DRX configuration (see TS 38.331 [16]), e.g. the mapping of PC5 QoS profile(s) to PC5 DRX cycle(s), default PC5 DRX configuration, when the UE is "not served by NG-RAN".

6)	Path selection policy:

-	The mapping of ProSe services (i.e. ProSe identifiers) to path preference (i.e. PC5 preferred, Uu preferred, or no preference) as defined in clause 5.11.

NOTE 5:	The path selection policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

7)	Path switching policy:

-	The mapping of ProSe services (i.e. ProSe identifiers) to path switching permission (i.e. PC5 permitted, or Uu permitted, or both PC5 and Uu permitted) as defined in clause 5.xx.

NOTE X:	The Path switching policy can be a one mapping for all ProSe services, i.e. same path switching permission for all ProSe services.

8)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Direct Communication.

The above parameter sets from bullet 2) to 7) may be provided or updated to the UE by the ProSe Application Server.

* * * * Start of 2nd Change * * * *

5.xx	Communication path switch between PC5 and Uu reference points

The "communication path switch between PC5 and Uu reference points" refers to the procedure on whether a UE is allowed to perform the path switch between the communication path over PC5 reference point and Uu reference point for a ProSe service.

Path switching policy is provided to the UE to indicate which path(s) is permitted to be switched for all or specific ProSe services (i.e. PC5 permitted, or Uu permitted, or both PC5 and Uu permitted) as specified in clause 5.1.3.1.

The ProSe Application Server can provide a path permission for ProSe Services to UDR as specified in clause 6.2.5 and this may be used by PCF for path selection policy generation and update as specified in clause 6.2.2.

The UE may use the provisioned path switching policy to determine perform the path switch for a ProSe service.

UE operation related to the path switch for ProSe service is as follows:

-	The UE evaluates the path switching policy in the policy and parameters as specified in clause 5.1.3.1 to determine the permission of the path switching for the ProSe service as below:

-	If PC5 permitted is indicated, the UE is permitted to switch to the PC5 for communication path for the ProSe service.

-	If Uu permitted is indicated, the UE is permitted to switch to the Uu for communication path for the ProSe service.

-	If no permitted is indicated or no path switching policy is provisioned, the UE is not permitted to perform the path switching.

* * * * Start of Next Change * * * *

6.X	Negotiated path switching between PC5 and Uu

6.X.1	General

In order to have unified understanding for performing the path switching between PC5 communication path and direct Uu communication path, a 5G ProSe enabled UE needs to negotiate with its peer UE over the existing PC5 connection to determine whether and which ProSe services are to be switched based on e.g. the path switching policy as specified in clause 5.1.3.1, availability of Uu path etc.

6.X.2	Path switching negotiation procedure for PC5 switching to Uu path

After the procedure for the unicast mode of ProSe Direct communication over PC5 reference point as specified in clause 6.4.3.1, one of the UEs initiates the path switching negotiation procedure for PC5 path switching to Uu path with peer UE as depicted in the following Figure 6.X.2-1.

Figure 6.X.2-1: Procedure for path switch from PC5 to Uu

1.	Service authorization and provisioning are performed for the UE as described in clause 6.2 with including the Path selection policy as described in clause 5.1.3.1.

2.	UE#1 and UE#2 have an established connection for unicast mode communication as described in clause 6.4.3.6.

3.	UE#1 initiates the path switching negotiation process for PC5 path switching to Uu path by sending a Path switching negotiation request message to UE#2. In this request message, it includes the ProSe identifier(s) that the UE#1 accepts to switch based on e.g. the Path switching policy as specified in clause 5.1.3.1, availability of Uu path etc. If there is not any ProSe identifier(s) in this request message, it means the UE#1 is not able to perform path switching from PC5 path to Uu path (e.g., under congestion control or mobility restriction).

4.	Upon reception of the Path switching negotiation request message, the UE#2 responds with a Path switching negotiation accept message, which includes the ProSe identifier(s) that the UE#2 confirms to be switched among the ProSe identifier(s) received in Path switching negotiation request message based on e.g. the Path switching policy as specified in clause 5.1.3.1, availability of Uu path etc. If there is not any ProSe identifier(s) in the received Path switching negotiation request message, the UE#2 decides that the UE#1 will not perform the path switching from PC5 to Uu path for the ProSe services over this established PC5 unicast connection. The response message doesn’t include any ProSe identifier(s) either.

5.	[optional] After step 4, the two UEs may prepare the Uu path communication, e.g., by performing the PDU session establishment/modification procedure for the negotiated ProSe identifier(s).

-	The QoS parameters over Uu path requested by UE may be based on PC5 QoS parameters.

6~7.	When the path switching triggers over PC5 are satisfied, e.g., the QoS requirements of the negotiated ProSe(s) can not be satisfied, the two UEs switch the corresponding traffic to the Uu path. If the step5 is not performed before, then in this step7 the 2 UEs perform the PDU session establishment/modification procedure for the corresponding traffic. If there isn’t any negotiated ProSe services to be switched during step3 and step4, the two UEs will not perform the path switching from PC5 to Uu path for the ProSe services over this established PC5 unicast connection.

8.	The two UEs may release the PC5 unicast connection, e.g., all the ProSe(s) are transferred to Uu path.

6.X.3	Path switching negotiation procedure for Uu switching to PC5 path

After the procedure for the unicast mode of ProSe Direct communication over PC5 reference point as specified in clause 6.4.3.1, one of the UEs initiates the path switching negotiation process for Uu path switching to PC5 path with peer UE as depicted in the following Figure 6.X.3-1.

Figure 6.X.3-1: Procedure for path switch from Uu to PC5

1.	Service authorization and provisioning are performed for the UE as described in clause 6.2 with including the Path selection policy as described in clause 5.1.3.1.

2.	UE#1 and UE#2 due to proximity have established a connection for unicast mode communication as described in clause 6.4.3.6.

3.	UE#1 initiates the path switching negotiation process for Uu path switching to PC5 path by sending a Path switching negotiation request message to UE#2. In this request message, it includes the ProSe identifier(s) that the UE#1 accepts to switch based on e.g. the Path switching policy as specified in clause 5.1.3.1 or the willing of offloading Uu traffic to PC5 etc.

4.	UE#2 responds with a Path switching negotiation accept message, which includes the ProSe identifier(s) that the UE#2 accepts to switch based on e.g. the Path switching policy as specified in clause 5.1.3.1, availability of Uu path etc.

5.	The two UEs initiate the PC5 unicast establishment/modification(i.e., add to the existing PC5 connection) procedure for the negotiated ProSe identifier(s).

6.	The two UEs may release corresponding Uu path (e.g., PDU session modification/release procedure).

* * * * End of Changes * * * *