3GPP TSG-WG SA2 Meeting #154-AH-E	S2-2300749

# Electronic Meeting, Jan 16th – 20th

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0091 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DNS Query transmission to HPLMN for HR-SBO PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is unclear how to handle the DNS query which needs to be sent to HPLMN, while the UE has only the V-EASDF address as the DNS server address and DNS query can only be transmitted to V-EASDF. In this paper, it is proposed to let the ULCL UPF or V-UPF modify the DNS query target IP address to the H-DNS server address. - for ULCL UPF, based on the offloading policy, the ULCL UPF will find which DNS query should be sent to HPLMN. ULCL UPF update the DNS query target IP address and send the DNS query to H-UPF through V-UPF. - for V-UPF, based on r17 method, optionD, it is PSA UPF to do the address update, so we propose to use V-UPF to update the DNS query target IP address, and send the DNS query to H-UPF through N9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two methods to support the DNS query which intent to HPLMN sent to V-UPF and H-UPF through GTP-U tunnel in HR-SBO PDU session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Currently it is not clear how the DNS query transmitted to HPLMN, while which should be handled by H-DNS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ….CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Changes (all new text) * * * *

6.7.2.x	DNS Query routing to HPLMN for HR-SBO PDU session

Figure 6.7.2.X-1. Procedure for DNS Query routing for HPLMN services for HR-SBO

- The DNS query sent by the UE reaches the UL CL/BP. Before this procedure, the HR-SBO PDU session has been established and ULCL/BP is also inserted. The related offloading policies, H-DNS address, are sent from H-SMF to V-SMF and configured in ULCL/BP UPF or V-UPF.

2a.		If the offloading policies, H-DNS address configured in ULCL/BP UPF, when ULCL/BP UPF detects the target FQDN of the DNS query is not part of the FQDN(s) which authorized by H-SMF to do local traffic routing in VPLMN, the ULCL/BP UPF modifies the the DNS Query’s destination IP address (corresponding to V-EASDF) to the address of H-DNS server.

3a.		The UL CL/BP UPF sends the DNS request to the DNS server of HPLMN via V-UPF and H-UPF(through N9).

2b.		If the offloading policies configured in ULCL/BP UPF, and H-DNS address configured inV-UPF, when the ULCL/BP UPF detects the target FQDN of the DNS query is not part of the FQDN(s) which authorized by H-SMF to do local traffic routing in VPLMN, the ULCL/BP UPF sends the DNS Query to V-UPF.

3b. 	V-UPF sends the DNS Query to the DNS server of HPLMN via H-UPF (through N9), with modifying the DNS Query 's destination IP address (corresponding to V-EASDF) to the address of H-DNS server.

4. 		DNS server in HPLMN sends the DNS response back to UE.

# * * * * End Changes * * * *