3GPP TSG-SA2 Meeting #154AH-E	S2-2300752

# Elbonia, January 16 –20, 2023																(Revision of S2-xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0652 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE latency performance analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the conclusion of KI#7, is has been agreed: AF can request NWDAFto provide existing, enhanced or new analytics to assist with federated learning operation. NWDAF Analytics services will be enhanced as follows: - New latency performance analytics are introduced for NWDAF. This CR will introduce the new UE latency performance analytics to TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing UE latency performance related analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE latency performance analytics as concluded in FS_AIMLsys cannot be supported to assist AIML-based services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1 , 6.7.x (new), 6.7.x.1 (new), 6.7.x.2 (new), 6.7.x.3 (new), 6.7.x.4 (new), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Change * * *

### 6.7.1	General

This clause specifies the UE related analytics which can be provided by NWDAF:

-	UE mobility analytics;

-	UE communication analytics;

-	Expected UE behavioural parameters related network data analytics;

-	Abnormal behaviour related network data analytics;

-	UE latency performance analytics.

The NWDAF service consumer may request for these analytics separately, or in a combined way. As an example, an NWDAF service consumer may learn from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3 of TS 23.502 [3] for a group of UEs or a specific UE, by requesting analytics for both UE mobility (see clause 6.7.2) and for UE communication (see clause 6.7.3).

Depending on local regulations, the NWDAF retrieves user consent for the UE with UDM prior to data collection as defined in clause 6.2.2.2 or clause 6.2.2.3. If user consent to collect data is not granted by the UE, the NWDAF rejects/cancels any analytics subscriptions to any of the UE related analytics with target for analytics set to the SUPI or GPSI of that UE. If the target for analytics is either an Internal or External Group Id or a list of SUPIs or "any UE", the NWDAF skips those SUPIs that do not grant user consent for the purpose of analytics or model training.

NOTE:	Possible uses of such analytics is for the AMF to learn about expected UE behaviour to derive appropriate MICO mode configuration, or for an AF to learn about expected UE behaviour to further provision 5GC with appropriate UE parameters.

* * *Next Change * * * (all new text)

## 6.7.x	UE Latency Performance Analytics

### 6.7.x.1	General

NWDAF provides latency performance statistics and predictions and shall be able to collect latency performance related service data from 5GC NFs and AF. The consumer can either subscribe to analytics notifications (i.e. a Subscribe-Notify model) or request a single notification (i.e. a Request-Response model). The latency performance analytics may be used to assist an AF hosting AI/ML-based services, e.g. for member selection of federated learning, etc.

Latency Performance may be provided as defined in clause 6.7.x.3 for a UE individually or group of UEs.

The service consumer may be an NF (e.g. NEF, AF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "UE Latency Performance".

-	Target of Analytics Reporting: a single UE or a group of UEs.

-	Analytics Filter Information optionally containing:

-	Application ID;

-	an optional list of analytics subsets that are requested (see clause 6.7.x.3);

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, Reporting Threshold(s), which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF; e.g. NWDAF may provide the percentage of UEs that have reached certain Reporting Threshold(s).

-	A matching direction may be provided such as crossed (default value), below, or above.

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested.

### 6.7.x.2	Input data

For the purpose of UE latency performance analytics, the NWDAF may collect the information as listed in Table 6.7.x.2-1 from AF directly (for trusted AF) or via NEF (for untrusted AF). Table 6.7.x.2-2 lists input data collected from relevant 5GC NFs (i.e. UPF, SMF, AMF) or OAM.

Table 6.7.x.2-1: Service Data from AF for UE latency performance analytics

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | Identifying the application providing this information. To identify the service and support analytics per type of service (the desired level of service) |
| Model size(s) | AF | The size of the model(s) to be trained or transferred. |
| Number of iterations | AF | The number of expected iterations associated with the model(s). |
| UE ID | AF | A UE ID or a list of UE ID(s) that are associated with the model(s). When the AF is untrusted, GPSI(s) will be provided. When the AF is trusted, SUPI(s) will be provided. |
| Group ID | AF | Application-layer identifier for the UE group if available |
| Training time | AF | Time required for training the model associated with the model size and iterations by the UE, if available. |
| Timestamp | AF | A timestamp associated with the latency performance related service data provided by the AF. |

Table 6.7.x.2-2: Service Data from 5GC NFs/OAM for UE latency performance analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF or OAM | A time stamp associated with the collected information. |
| UE location | AMF, GMLC | The UE location information |
| UE ID | AMF | (list of) SUPI(s). |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow. |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow. |
| QFI | SMF | QoS Flow Identifier. |
| QoS flow Packet Delay | SMF, UPF | The observed Packet delay for UL/DL/round trip directions; |
| Distribution of UL/DL/round trip packed delay between PSA UPF and UE | OAM | The distribution of UL/DL/round trip packet delay between PSA UPF and UE as captured in TS 28.552 [8]. |

### 6.7.x.3	Output analytics

UE latency performance statistics information is defined in Table 6.7.x.3-1. UE latency performance prediction information is defined in Table 6.7.x.3-2.

Table 6.7.x.3-1: UE Latency performance statistics

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs or Group ID | Identifies a UE or a group of UEs. |
| UE latency performance (1…max) | List of observed UE latencies. Max. is the number of UEs, if applicable. |
| > Application ID | Identification of the application. |
| > UE location | Indicating the UE location information when the UE service is delivered. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Model size | The average size of the application A/ML model(s) to be trained or transferred. |
| > Number of iterations | The number of iterations for application AI/ML model training. |
| > Validity period | The validity period for the UE latency performance statistics as defined in clause 6.1.3. |
| > Spatial validity | Area where the UE latency performance statistics applies |
| > UE Latency (NOTE 1) | Statistics of UE latency performance over the Analytics target period (e.g. average, variance) for a single UE or a group of UEs. |
| >> UL packet delay | The UL packet delay for the UE. |
| >> DL packet delay | The DL packet delay for the UE. |
| >> Round trip packet delay | The round trip packet delay for the UE. |
| Aggregate latency performances (NOTE 1) | Aggregated latency performance statistics for multiple UEs |
| > Latency classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of latency performance |
| >> Ratio/percentage of UEs per latency class | Percentage or ratio of UEs |
| >> Aggregate UL/DL/round trip packet delay per latency class | Aggregated statistical values of the corresponding UE latency statistics in the latency class (e.g. average, variance). |
| > Validity period | The validity period for the UE latency performance statistics as defined in clause 6.1.3. |
| > Spatial validity | Area where the UE latency performance statistics applies. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 2: The number of latency classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

Table 6.7.x.3-2: UE Latency performance predictions

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs or Group ID | Identifies a UE or a group of UEs. |
| UE latency performance (1…max) | List of observed UE latencies. Max. is the number of UEs, if applicable. |
| > Application ID | Identification of the application. |
| > UE location | Indicating the UE location information when the UE service is delivered. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Model size | The average size of the application A/ML model(s) to be trained or transferred. |
| > Number of iterations | The number of iterations for application AI/ML model training. |
| > Validity period | The validity period for the UE latency performance statistics as defined in clause 6.1.3. |
| > Spatial validity | Area where the UE latency performance statistics applies |
| > UE Latency (NOTE 1) | Predictions of UE latency performance over the Analytics target period (e.g. average, variance) for a single UE or a group of UEs. |
| >> UL packet delay | The UL packet delay for the UE |
| >> DL packet delay | The DL packet delay for the UE |
| >> Round trip packet delay | The round trip packet delay for the UE |
| > Confidence | Confidence of the prediction. |
| Aggregate latency performances (NOTE 1) | Aggregated latency performance predictions for multiple UEs |
| > Latency classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of latency performance |
| >> Ratio/percentage of UEs per latency class | Percentage or ratio of UEs |
| >> Aggregate UL/DL/round trip packet delay per latency class | Aggregated statistical values of the corresponding UE latency statistics in the latency class (e.g. average, variance). |
| > Validity period | The validity period for the UE latency performance statistics as defined in clause 6.1.3. |
| > Spatial validity | Area where the UE latency performance statistics applies. |
| > Confidence | Confidence of the prediction. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 2: The number of latency classes may be pre-defined by the operator or provided by the service consumer via reporting thresholds. |  |

### 6.7.x.4	Procedures

The NWDAF can provide UE latency performance analytics, in the form of statistics or predictions or both, to a 5GC NF or AF.

Figure 6.7.x.4-1: Procedure for UE latency performance analytics

- The consumer NF (e.g. NEF or and AF) subscribes to/requests a NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation. The Analytics ID is set to "UE Latency Performance". The Target of Analytics Reporting, Analytics Filters and Analytics Reporting Information are set according to clause 6.7.x.1.

2a-b.	The NWDAF subscribes the service data from AMF in Table 6.7.x.2-2 using Namf_EventExposure_Subscribe service for collecting UE location(s) for a UE or a group of UEs.

NOTE:	If NWDAF requires UE location information with finer granularity than TA/cell, then NWDAF collects the location data from GMLC instead of AMF.

2c.	NWDAF subscribes to service data from SMF in Table 6.7.x.2-2 by invoking Nsmf_EventExposure_Subscribe (Event ID, SUPI(s) or group ID, Application ID).

In order to provide the requested analytics, the NWDAF subscribes to information of the UE and may subscribe to N4 Session related input data from SMFs as defined in Table 6.7.x.2-2.

2d-e.	N4 related input data is provided by UPF to SMF.

2f.	SMF provides the requested input data to NWDAF.

2g-h. NWDAF subscribes the service data from AF in Table 6.7.x.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service as defined in TS 23.502 [3].

3.	The NWDAF derives requested analytics, in the form of latency performance statistics or predictions or both.

4.	The NWDAF provides requested latency performance analytics to the NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-7.	If the NF subscribed to UE latency performance analytics at step 1, when the NWDAF generates new analytics, it notifies the new generated analytics to the consumer.

* * *Next Change * * *

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| UE Latency Performance | Analytics ID: UE Latency Performance | Analytics on UE latency performance |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |

* * *End of Changes * * *