3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300758

# Janurary 16 – 20, 2023, Elbonia		(revision of S2-2xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3978 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (KI #4&5) modification on QoS description for PDU set based mechanism |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS23.501 needs to be updated to reflect the conclusion for KI#4&5 in TR23.700-60. On top of penholder’s version, the followings are added. - QoS flow description is updated to add PDU Set based handling. - It is claflaried that if PDU set based hanlding is applied, PSDB is derived from PDB(which is assigned to QoS flow) and maximum duration of a PDU set(which is provided by SLA or configuration or AF). - PSER is not used in this release because PER can be used for link layer adaptation as it is but error rate of set of PDUs can not give correct input for link layer configuration. - PDU set size and PDU set importance may need further information from AF in order that UPF may identify the PDU set information from RTP header because to know if RTP overhead is included in PDU set size and how the importance value is securely delivered against security attack. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.7 is updated to include PDU Set level QoS Clause 5.8.2 is updated to include PDU Set level user plane handling Clause 5.37.x is added to reflect approach adopted for XRM KI#4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDU set based handling solutions for XRM is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.1,5.7.1.2,5.7.1.5,5.7.2.1,5.7.3.x,5.7.3.y,5.7.3.z, 5.7.4, 5.8.2.x, 6.2.5.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.7	QoS model

5.7.1	General Overview

5.7.1.1	QoS Flow

The 5G QoS model is based on QoS Flows. The 5G QoS model supports both QoS Flows that require guaranteed flow bit rate (GBR QoS Flows) and QoS Flows that do not require guaranteed flow bit rate (Non-GBR QoS Flows). The 5G QoS model also supports Reflective QoS (see clause 5.7.5).

The QoS Flow is the finest granularity of QoS differentiation in the PDU Session. A QoS Flow ID (QFI) is used to identify a QoS Flow in the 5G System. User Plane traffic with the same QFI within a PDU Session receives the same traffic forwarding treatment (e.g. scheduling, admission threshold). The QFI is carried in an encapsulation header on N3 (and N9) i.e. without any changes to the e2e packet header. QFI shall be used for all PDU Session Types. The QFI shall be unique within a PDU Session. The QFI may be dynamically assigned or may be equal to the 5QI (see clause 5.7.2.1).

Moreover, when PDU Set based handing is applied to a QoS flow, User Plane traffic with the same QFI can be handled further differentiatedly per their PDU Sets. The PDU Set information(i.e. PDU Set sequence number, End PDU of the PDU set, PDU sequence number within a PDU Set, PDU Set size in bytes, PDU Set importance) as well as QFI is carried in an encapsulation header on N3 (and N9) i.e. without any changes to the e2e packet header.

Within the 5GS, a QoS Flow is controlled by the SMF and may be preconfigured, or established via the PDU Session Establishment procedure (see clause 4.3.2 of TS 23.502 [3]), or the PDU Session Modification procedure (see clause 4.3.3 of TS 23.502 [3].

Any QoS Flow is characterised by:

-	A QoS profile provided by the SMF to the AN via the AMF over the N2 reference point or preconfigured in the AN;

-	One or more QoS rule(s) and optionally QoS Flow level QoS parameters (as specified in TS 24.501 [47]) associated with these QoS rule(s) which can be provided by the SMF to the UE via the AMF over the N1 reference point and/or derived by the UE by applying Reflective QoS control; and

-	One or more UL and DL PDR(s) provided by the SMF to the UPF.

Within the 5GS, a QoS Flow associated with the default QoS rule is required to be established for a PDU Session and remains established throughout the lifetime of the PDU Session. This QoS Flow should be a Non-GBR QoS Flow (further details are described in clause 5.7.2.7).

A QoS Flow is associated with QoS requirements as specified by QoS parameters and QoS characteristics.

NOTE:	The QoS Flow associated with the default QoS rule provides the UE with connectivity throughout the lifetime of the PDU Session. Possible interworking with EPS motivates the recommendation for this QoS Flow to be of type Non-GBR.

5.7.1.2	QoS Profile

A QoS Flow may either be 'GBR' or 'Non-GBR' depending on its QoS profile. The QoS profile of a QoS Flow is sent to the (R)AN and it contains QoS parameters as described below (details of QoS parameters are described in clause 5.7.2):

-	For each QoS Flow, the QoS profile shall include the QoS parameters:

-	5G QoS Identifier (5QI); and

-	Allocation and Retention Priority (ARP).

-	For each Non-GBR QoS Flow only, the QoS profile may also include the QoS parameter:

-	Reflective QoS Attribute (RQA).

-	For each GBR QoS Flow only, the QoS profile shall also include the QoS parameters:

-	Guaranteed Flow Bit Rate (GFBR) - UL and DL; and

-	Maximum Flow Bit Rate (MFBR) - UL and DL; and

-	In the case of a GBR QoS Flow only, the QoS profile may also include one or more of the QoS parameters:

-	Notification control;

-	Maximum Packet Loss Rate - UL and DL.

NOTE:	In this Release of the specification, the Maximum Packet Loss Rate (UL, DL) is only provided for a GBR QoS Flow belonging to voice media.

-	For a QoS Flow which PDU Set based handling is applied, the QoS profile shall also include the QoS parameters:

-	PDU Set based handling indication;

-	Maximun duration of a PDU Set(i.e. maximum duration from when a packet of the PDU Set arrives at 5GS to when all the packets of the PDU Set arrive); and

-     PDU Set Integrated Indication.

NOTE:	Based on Maximum duration of a PDU Set and PDB of the QoS Flow, PDU Set delay budget is calculated as sum of  Maximum duration of a PDU Set and PDB) and applied in AN for PDU set based handling.

Each QoS profile has one corresponding QoS Flow identifier (QFI) which is not included in the QoS profile itself.

The usage of a dynamically assigned 5QI for a QoS Flow requires in addition the signalling of the complete 5G QoS characteristics (described in clause 5.7.3) as part of the QoS profile.

When a standardized or pre-configured 5QI is used for a QoS Flow, some of the 5G QoS characteristics may be signalled as part of the QoS profile (as described in clause 5.7.3).

# * * * *2nd change * * * *

5.7.1.5	QoS Flow mapping

The SMF performs the binding of PCC rules to QoS Flows based on the QoS and service requirements (as defined in TS 23.503 [45]). The SMF assigns the QFI for a new QoS Flow and derives its QoS profile, corresponding UPF instructions and QoS Rule(s) from the PCC rule(s) bound to the QoS Flow and other information provided by the PCF.

When applicable, the SMF provides the following information for the QoS Flow to the (R)AN:

-	QFI;

-	QoS profile as described in clause 5.7.1.2.

-	optionally, Alternative QoS Profile(s) as described in clause 5.7.1.2a;

For each PCC rule bound to a QoS Flow, the SMF provides the following information to the UPF enabling classification, bandwidth enforcement and marking of User Plane traffic (the details are described in clause 5.8):

-	a DL PDR containing the DL part of the SDF template;

-	an UL PDR containing the UL part of the SDF template;

NOTE 1:	If a DL PDR for an bidirectional SDF is associated with a QoS Flow other than the one associated with the default QoS rule and the UE has not received any instruction to use this QoS Flow for the SDF in uplink direction (i.e. neither a corresponding QoS rule is sent to the UE nor the Reflective QoS Indication is set in the PCC rule), it means that the UL PDR for the same SDF has to be associated with the QoS Flow associated with the default QoS rule.

-	the PDR precedence value (see clause 5.7.1.9) for both PDRs is set to the precedence value of the PCC rule;

-	QoS related information (e.g. MBR for an SDF, GFBR and MFBR for a GBR QoS Flow) as described in clause 5.8.2;

-	the corresponding packet marking information (e.g. the QFI, PDU Set information, the transport level packet marking value (e.g. the DSCP value of the outer IP header), RTP/SRTP header or extended header and payload(RFC 3550/3711/6184/7798/draft-ietf-avtcore-rtp-vvc/draft-ietf-avtext-framemarking/ AV1 RTP payload format));

-	optionally, the Reflective QoS Indication is included in the QER associated with the DL PDR (as described in clause 5.7.5.3).

For each PCC rule bound to a QoS Flow, when applicable, the SMF generates an explicitly signalled QoS rule (see clause 5.7.1.4) according to the following principles and provides it to the UE together with an add operation:

-	A unique (for the PDU Session) QoS rule identifier is assigned;

-	The QFI in the QoS rule is set to the QFI of the QoS Flow to which the PCC rule is bound;

-	The Packet Filter Set of the QoS rule is generated from the UL SDF filters and optionally the DL SDF filters of the PCC rule (but only from those SDF filters that have an indication for being signalled to the UE, as defined in TS 23.503 [45]);

-	The QoS rule precedence value is set to the precedence value of the PCC rule for which the QoS rule is generated;

-	for a dynamically assigned QFI, the QoS Flow level QoS parameters (e.g. 5QI, GFBR, MFBR, Averaging Window, see TS 24.501 [47]) are signalled to UE in addition to the QoS rule(s) associated to the QoS Flow. The QoS Flow level QoS parameters of an existing QoS Flow may be updated based on the MBR and GBR information received in the PCC rule (MBR and GBR per SDF are however not provided to UE over N1 in the case of more than one SDF) or, if the PCF has not indicated differently, when Notification control or handover related signalling indicates that the QoS parameter the NG-RAN is currently fulfilling for the QoS Flow have changed (see clause 5.7.2.4).

Changes in the binding of PCC rules to QoS Flows as well as changes in the PCC rules or other information provided by the PCF can require QoS Flow changes which the SMF has to provide to (R)AN, UPF and/or UE. In the case of changes in the explicitly signalled QoS rules associated to a QoS Flow, the SMF provides the explicitly signalled QoS rules and their operation (i.e. add/modify/delete) to the UE.

NOTE 2:	The SMF cannot provide, update or remove pre-configured QoS rules or UE derived QoS rules.

The principle for classification and marking of User Plane traffic and mapping of QoS Flows to AN resources is illustrated in Figure 5.7.1.5-1.

Figure 5.7.1.5-1: The principle for classification and User Plane marking for QoS Flows and mapping to AN Resources

In DL, incoming data packets are classified by the UPF based on the Packet Filter Sets of the DL PDRs in the order of their precedence (without initiating additional N4 signalling). The UPF conveys the classification of the User Plane traffic belonging to a QoS Flow and (if applied) its belonging PDU Set through an N3 (and N9) User Plane marking using a QFI and PDU Set information. The AN binds QoS Flows to AN resources (i.e. Data Radio Bearers of in the case of 3GPP RAN). There is no strict 1:1 relation between QoS Flows and AN resources. It is up to the AN to establish the necessary AN resources that QoS Flows can be mapped to, and to release them. It is up to the AN to treat the AN resources as PDU Sets of the QoS Flow. The AN shall indicate to the SMF when the AN resources onto which a QoS Flow is mapped are released.

If no matching DL PDR is found, the UPF shall discard the DL data packet.

In UL:

-	For a PDU Session of Type IP or Ethernet, the UE evaluates UL packets against the UL Packet Filters in the Packet Filter Set in the QoS rules based on the precedence value of QoS rules in increasing order until a matching QoS rule (i.e. whose Packet Filter matches the UL packet) is found.

-	If no matching QoS rule is found, the UE shall discard the UL data packet.

-	For a PDU Session of Type Unstructured, the default QoS rule does not contain a Packet Filter Set and allows all UL packets.

NOTE 3:	Only the default QoS rule exist for a PDU Session of Type Unstructured.

The UE uses the QFI in the corresponding matching QoS rule to bind the UL packet to a QoS Flow. The UE then binds QoS Flows to AN resources.

# * * * * 3rd change * * * *

5.7.2	5G QoS Parameters

5.7.2.1	5QI

A 5QI is a scalar that is used as a reference to 5G QoS characteristics defined in clause 5.7.4, i.e. access node-specific parameters that control QoS forwarding treatment for the QoS Flow (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.).

Standardized 5QI values have one-to-one mapping to a standardized combination of 5G QoS characteristics as specified in Table 5.7.4-1.

The 5G QoS characteristics for pre-configured 5QI values are pre-configured in the AN.

When the PDU Set based handling is indicated, the 5G QoS characteristics for pre-configured 5QI values are interpreted as per PDU Set based values(i.e. PSDB) in the AN as specified in clauses 5.7.3.4, 5.7.3.x, 5.7.3.y, and 5.7.3.z

Standardized or pre-configured 5G QoS characteristics, are indicated through the 5QI value, and are not signalled on any interface, unless certain 5G QoS characteristics are modified as specified in clauses 5.7.3.3, 5.7.3.4, 5.7.3.6, 5.7.3.7, 5.7.3.x, 5.7.3.y, and 5.7.3.z.

The 5G QoS characteristics for QoS Flows with dynamically assigned 5QI are signalled as part of the QoS profile.

NOTE:	On N3, each PDU (i.e. in the tunnel used for the PDU Session) is associated with one 5QI via the QFI carried in the encapsulation header.

# * * * * 4th change * * * *

5.7.3	5G QoS characteristics

5.7.3.1	General

This clause specifies the 5G QoS characteristics associated with 5QI. The characteristics describe the packet forwarding treatment that a QoS Flow receives edge-to-edge between the UE and the UPF in terms of the following performance characteristics:

1	Resource type (Non-GBR, GBR, Delay-critical GBR);

2	Priority Level;

3	Packet Delay Budget (including Core Network Packet Delay Budget);

4	Packet Error Rate;

5	Averaging window (for GBR and Delay-critical GBR resource type only);

6	Maximum Data Burst Volume (for Delay-critical GBR resource type only).

The 5G QoS characteristics should be understood as guidelines for setting node specific parameters for each QoS Flow e.g. for 3GPP radio access link layer protocol configurations.

Standardized or pre-configured 5G QoS characteristics, are indicated through the 5QI value, and are not signalled on any interface, unless certain 5G QoS characteristics are modified as specified in clauses 5.7.3.3, 5.7.3.4, 5.7.3.6, and 5.7.3.7.

NOTE:	As there are no default values specified, pre-configured 5G QoS characteristics have to include all of the characteristics listed above.

Signalled 5G QoS characteristics are provided as part of the QoS profile and shall include all of the characteristics listed above.

5.7.3.2	Resource Type

The resource type determines if dedicated network resources related to a QoS Flow-level Guaranteed Flow Bit Rate (GFBR) value are permanently allocated (e.g. by an admission control function in a radio base station).

GBR QoS Flows are therefore typically authorized "on demand" which requires dynamic policy and charging control. A GBR QoS Flow uses either the GBR resource type or the Delay-critical GBR resource type. The definition of PDB and PER are different for GBR and Delay-critical GBR resource types, and the MDBV parameter applies only to the Delay-critical GBR resource type.

A Non-GBR QoS Flow may be pre-authorized through static policy and charging control. A Non-GBR QoS Flow uses only the Non-GBR resource type.

5.7.3.3	Priority Level

The Priority Level associated with 5G QoS characteristics indicates a priority in scheduling resources among QoS Flows. The lowest Priority Level value corresponds to the highest priority.

The Priority Level shall be used to differentiate between QoS Flows of the same UE, and it shall also be used to differentiate between QoS Flows from different UEs.

In the case of congestion, when all QoS requirements cannot be fulfilled for one or more QoS Flows, the Priority Level shall be used to select for which QoS Flows the QoS requirements are prioritised such that a QoS Flow with Priority Level value N is priorized over QoS Flows with higher Priority Level values (i.e. N+1, N+2, etc).In the case of no congestion, the Priority Level should be used to define the resource distribution between QoS Flows. In addition, the scheduler may prioritize QoS Flows based on other parameters (e.g. resource type, radio condition) in order to optimize application performance and network capacity.

Every standardized 5QI is associated with a default value for the Priority Level -specified in QoS characteristics Table 5.7.4.1). Priority Level may also be signalled together with a standardized 5QI to the -R)AN, and if it is received, it shall be used instead of the default value.

Priority Level may also be signalled together with a pre-configured 5QI to the (R)AN, and if it is received, it shall be used instead of the pre-configured value.

5.7.3.4	Packet Delay Budget

The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF. The PDB applies to the DL packet received by the UPF over the N6 interface, and to the UL packet sent by the UE. For a certain 5QI the value of the PDB is the same in UL and DL. In the case of 3GPP access, the PDB is used to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points). For GBR QoS Flows using the Delay-critical resource type, a packet delayed more than PDB is counted as lost if the data burst is not exceeding the MDBV within the period of PDB and the QoS Flow is not exceeding the GFBR. For GBR QoS Flows with GBR resource type not exceeding GFBR, 98 percent of the packets shall not experience a delay exceeding the 5QI's PDB.

The 5G Access Network Packet Delay Budget (5G-AN PDB) is determined by subtracting a static value for the Core Network Packet Delay Budget (CN PDB), which represents the delay between any N6 termination point at the UPF (for any UPF that may possibly be selected for the PDU Session) and the 5G-AN from a given PDB.

NOTE 1:	For a standardized 5QI, the static value for the CN PDB is specified in the QoS characteristics Table 5.7.4-1.

NOTE 2:	For a non-standardized 5QI, the static value for the CN PDB is homogeneously configured in the network.

For GBR QoS Flows using the Delay-critical resource type, in order to obtain a more accurate delay budget PDB available for the NG-RAN, a dynamic value for the CN PDB, which represents the delay between the UPF terminating N6 for the QoS Flow and the 5G-AN, can be used. If used for a QoS Flow, the NG-RAN shall apply the dynamic value for the CN PDB instead of the static value for the CN PDB (which is only related to the 5QI). Different dynamic value for CN PDB may be configured per uplink and downlink direction.

NOTE 3:	The configuration of transport network on CN tunnel can be different per UL and DL, which can be different value for CN PDB per UL and DL.

NOTE 4:	It is expected that the UPF deployment ensures that the dynamic value for the CN PDB is not larger than the static value for the CN PDB. This avoids that the functionality that is based on the 5G-AN PDB (e.g. MDBV, NG-RAN scheduler) has to handle an unexpected value.

The dynamic value for the CN PDB of a Delay-critical GBR 5QI may be configured in the network in two ways:

-	Configured in each NG-RAN node, based on a variety of inputs such as different IP address(es) or TEID range of UPF terminating the N3 tunnel and based on different combinations of PSA UPF to NG-RAN under consideration of any potential I-UPF, etc;

-	Configured in the SMF, based on different combinations of PSA UPF to NG-RAN under consideration of any potential I-UPF. The dynamic value for the CN PDB for a particular QoS Flow shall be signalled to NG-RAN (during PDU Session Establishment, PDU Session Modification, Xn/N2 handover and the Service Request procedures) when the QoS Flow is established or the dynamic value for the CN PDB of a QoS Flow changes, e.g. when an I-UPF is inserted by the SMF.

If the NG-RAN node is configured locally with a dynamic value for the CN PDB for a Delay-critical GBR 5QI, and receives a different value via N2 signalling for a QoS Flow with the same 5QI, local configuration in RAN node determines which value takes precedence.

Services using a GBR QoS Flow and sending at a rate smaller than or equal to the GFBR can in general assume that congestion related packet drops will not occur.

NOTE 5:	Exceptions (e.g. transient link outages) can always occur in a radio access system which may then lead to congestion related packet drops. Packets surviving congestion related packet dropping may still be subject to non-congestion related packet losses (see PER below).

Services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB.

The PDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. However, for a Delay-critical GBR resource type, packets delayed more than the PDB are added to the PER and can be discarded or delivered depending on local decision.

NOTE: When the PDU Set based handling is applied, PSDB(PDU Set Delay Budget) is calculated as sum of Maximum duration of a PDU Set and PDB, and used instead of PDB in the NG-RAN.

5.7.3.5	Packet Error Rate

The Packet Error Rate (PER) defines an upper bound for the rate of PDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PER defines an upper bound for a rate of non-congestion related packet losses. The purpose of the PER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). For every 5QI the value of the PER is the same in UL and DL. For GBR QoS Flows with Delay-critical GBR resource type, a packet which is delayed more than PDB is counted as lost, and included in the PER unless the data burst is exceeding the MDBV within the period of PDB or the QoS Flow is exceeding the GFBR.

5.7.3.6	Averaging Window

Each GBR QoS Flow shall be associated with an Averaging window. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. in the (R)AN, UPF, UE).

Every standardized 5QI (of GBR and Delay-critical GBR resource type) is associated with a default value for the Averaging window (specified in QoS characteristics Table 5.7.4.1). The averaging window may also be signalled together with a standardized 5QI to the (R)AN and UPF, and if it is received, it shall be used instead of the default value.

The Averaging window may also be signalled together with a pre-configured 5QI to the (R)AN, and if it is received, it shall be used instead of the pre-configured value.

5.7.3.7	Maximum Data Burst Volume

Each GBR QoS Flow with Delay-critical resource type shall be associated with a Maximum Data Burst Volume (MDBV).

MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB.

Every standardized 5QI (of Delay-critical GBR resource type) is associated with a default value for the MDBV (specified in QoS characteristics Table 5.7.4.1). The MDBV may also be signalled together with a standardized 5QI to the (R)AN, and if it is received, it shall be used instead of the default value.

The MDBV may also be signalled together with a pre-configured 5QI to the (R)AN, and if it is received, it shall be used instead of the pre-configured value.

5.7.3.x	PDU Set Delay Budget

The PDU Set Delay Budget (PSDB) defines an upper bound for the time that all packets of a PDU Set may be delayed between the UE and the N6 termination point at the UPF(i.e. from when a packet of the PDU Set arrives to when all the packets of the PDU Set are transmitted between UE and PSA UPF) when the PDU Set based handling is applied. The PSDB applies to the DL packet received by the UPF over the N6 interface, and to the UL packet sent by the UE. PSDB is calculated as sum of Maximum duration of a PDU Set and PDB of the QoS Flow so that PSDB value is not directly delivered to NG-RAN but used instead of PDB if the PDU Set based handling is applied, where Maximun duration of a PDU Set is defined as maximum duration from when a packet of the PDU Set arrives at 5GS to when all the packets of the PDU Set arrive at 5GS)

NOTE: When the PDU Set based handling is applied, Maximum duration of a PDU Set as well as PDB is delivered to NG-RAN but PSDB may not need to be delivered.

5.7.3.y	PDU Set Error Rate

If the PDU Set based handling is applied, PER is used so that PSER(PDU Set Error Rate) does not need definition in this release.

5.7.3.z	PDU Set Integrated Indication

When the PDU Set based handling is applied, PDU Set Integrated Indication implies whether all PDUs of the PDU Set are needed for the usage of PDU Set by application layer.

5.7.4	Standardized 5QI to QoS characteristics mapping

Standardized 5QI values are specified for services that are assumed to be frequently used and thus benefit from optimized signalling by using standardized QoS characteristics. Dynamically assigned 5QI values (which require a signalling of QoS characteristics as part of the QoS profile) can be used for services for which standardized 5QI values are not defined. The one-to-one mapping of standardized 5QI values to 5G QoS characteristics is specified in table 5.7.4-1.

Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms (NOTE 11, NOTE 13) | 10-2 | N/A | 2000 ms | Conversational Voice |
| 2 | (NOTE 1) | 40 | 150 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |
| 3 |  | 30 | 50 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Real Time Gaming, V2X messages (see TS 23.287 [121]). Electricity distribution – medium voltage, Process automation monitoring |
| 4 |  | 50 | 300 ms (NOTE 11, NOTE 13) | 10-6 | N/A | 2000 ms | Non-Conversational Video (Buffered Streaming) |
| 65 (NOTE 9, NOTE 12) |  | 7 | 75 ms (NOTE 7, NOTE 8) | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 66 (NOTE 12) |  | 20 | 100 ms (NOTE 10, NOTE 13) | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 67 (NOTE 12) |  | 15 | 100 ms (NOTE 10, NOTE 13) | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 75 (NOTE 14) |  |  |  |  |  |  |  |
| 71 |  | 56 | 150 ms (NOTE 11, NOTE 13, NOTE 15) | 10-6 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 72 |  | 56 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | 10-4 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 73 |  | 56 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | 10-8 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 74 |  | 56 | 500 ms (NOTE 11, NOTE 15) | 10-8 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 76 |  | 56 | 500 ms (NOTE 11, NOTE 13, NOTE 15) | 10-4 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 5 | Non-GBR | 10 | 100 ms NOTE 10, NOTE 13) | 10-6 | N/A | N/A | IMS Signalling |
| 6 | (NOTE 1) | 60 | 300 ms (NOTE 10, NOTE 13) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 |  | 70 | 100 ms (NOTE 10, NOTE 13) | 10-3 | N/A | N/A | Voice, Video (Live Streaming) Interactive Gaming |
| 8 |  | 80 | 300 ms (NOTE 13) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive |
| 9 |  | 90 |  |  |  |  | video, etc.) |
| 10 |  | 90 | 1100ms (NOTE 13) (NOTE 17) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |
| 69 (NOTE 9, NOTE 12) |  | 5 | 60 ms (NOTE 7, NOTE 8) | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 70 (NOTE 12) |  | 55 | 200 ms (NOTE 7, NOTE 10) | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |
| 79 |  | 65 | 50 ms (NOTE 10, NOTE 13) | 10-2 | N/A | N/A | V2X messages (see TS 23.287 [121]) |
| 80 |  | 68 | 10 ms (NOTE 5, NOTE 10) | 10-6 | N/A | N/A | Low Latency eMBB applications Augmented Reality |
| 82 | Delay-critical GBR | 19 | 10 ms (NOTE 4) | 10-4 | 255 bytes | 2000 ms | Discrete Automation (see TS 22.261 [2]) |
| 83 |  | 22 | 10 ms (NOTE 4) | 10-4 | 1354 bytes (NOTE 3) | 2000 ms | Discrete Automation (see TS 22.261 [2]); V2X messages (UE - RSU Platooning, Advanced Driving: Cooperative Lane Change with low LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 84 |  | 24 | 30 ms (NOTE 6) | 10-5 | 1354 bytes (NOTE 3) | 2000 ms | Intelligent transport systems (see TS 22.261 [2]) |
| 85 |  | 21 | 5 ms (NOTE 5) | 10-5 | 255 bytes | 2000 ms | Electricity Distribution- high voltage (see TS 22.261 [2]). V2X messages (Remote Driving. See TS 22.186 [111], NOTE 16, see TS 23.287 [121]) |
| 86 |  | 18 | 5 ms (NOTE 5) | 10-4 | 1354 bytes | 2000 ms | V2X messages (Advanced Driving: Collision Avoidance, Platooning with high LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 87 |  | 25 | 5 ms (NOTE 4) | 10-3 | 500 bytes | 2000 ms | Interactive Service - Motion tracking data, (see TS 22.261 [2]) |
| 88 |  | 25 | 10 ms (NOTE 4) | 10-3 | 1125 bytes | 2000 ms | Interactive Service - Motion tracking data, (see TS 22.261 [2]) |
| 89 |  | 25 | 15 ms (NOTE 4) | 10-4 | 17000 bytes | 2000 ms | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |
| 90 |  | 25 | 20 ms (NOTE 4) | 10-4 | 63000 bytes | 2000 ms | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |
| NOTE 1: A packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: It is required that default MDBV is supported by a PLMN supporting the related 5QIs. NOTE 3: The Maximum Transfer Unit (MTU) size considerations in clause 9.3 and Annex C of TS 23.060 [56] are also applicable. IP fragmentation may have impacts to CN PDB, and details are provided in clause 5.6.10. NOTE 4: A static value for the CN PDB of 1 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 5: A static value for the CN PDB of 2 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 6: A static value for the CN PDB of 5 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 7: For Mission Critical services, it may be assumed that the UPF terminating N6 is located "close" to the 5G_AN (roughly 10 ms) and is not normally used in a long distance, home routed roaming situation. Hence a static value for the CN PDB of 10 ms for the delay between a UPF terminating N6 and a 5G_AN should be subtracted from this PDB to derive the packet delay budget that applies to the radio interface. NOTE 8: In both RRC Idle and RRC Connected mode, the PDB requirement for these 5QIs can be relaxed (but not to a value greater than 320 ms) for the first packet(s) in a downlink data or signalling burst in order to permit reasonable battery saving (DRX) techniques. NOTE 9: It is expected that 5QI-65 and 5QI-69 are used together to provide Mission Critical Push to Talk service (e.g. 5QI-5 is not used for signalling). It is expected that the amount of traffic per UE will be similar or less compared to the IMS signalling. NOTE 10: In both RRC Idle and RRC Connected mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 11: In RRC Idle mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 12: This 5QI value can only be assigned upon request from the network side. The UE and any application running on the UE is not allowed to request this 5QI value. NOTE 13: A static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 14: This 5QI is not supported in this Release of the specification as it is only used for transmission of V2X messages over MBMS bearers as defined in TS 23.285 [72] but the value is reserved for future use. NOTE 15: For "live" uplink streaming (see TS 26.238 [76]), guidelines for PDB values of the different 5QIs correspond to the latency configurations defined in TR 26.939 [77]. In order to support higher latency reliable streaming services (above 500ms PDB), if different PDB and PER combinations are needed these configurations will have to use non-standardised 5QIs. NOTE 16: These services are expected to need much larger MDBV values to be signalled to the RAN. Support for such larger MDBV values with low latency and high reliability is likely to require a suitable RAN configuration, for which, the simulation scenarios in TR 38.824 [112] may contain some guidance. NOTE 17: The worst case one way propagation delay for GEO satellite is expected to be ~270ms, ,~ 21 ms for LEO at 1200km, and 13 ms for LEO at 600km. The UL scheduling delay that needs to be added is also typically two way propagation delay e.g. ~540ms for GEO, ~42ms for LEO at 1200km, and ~26 ms for LEO at 600km. Based on that, the 5G-AN Packet delay budget is not applicable for 5QIs that require 5G-AN PDB lower than the sum of these values when the specific types of satellite access are used (see TS 38.300 [27]). 5QI-10 can accommodate the worst case PDB for GEO satellite type. |  |  |  |  |  |  |  |

NOTE1:	It is preferred that a value less than 64 is allocated for any new standardised 5QI of Non-GBR resource type. This is to allow for option 1 to be used as described in clause 5.7.1.3 (as the QFI is limited to less than 64).

NOTE2:	When the PDU Set based handling is indicated, the standardized 5QI values are interpreted as per PDU Set based values(i.e. PSDB) as specified in clauses 5.7.3.4, 5.7.3.x, 5.7.3.y, and 5.7.3.z

# * * * * 5th change * * * *

5.8.2	Functional Description

5.8.2.1	General

This clause contains detailed functional descriptions for some of the functions provided by the UPF. It is described how the SMF instructs it's corresponding UP function and which control parameters are used.

5.8.2.x	PDU Set Identification

The UPF identifies PDUs that belong to PDU Sets and determines PDU Set Information which it sends to the RAN in the GTP-U header of each PDU identified as belonging to a PDU Set. The PDU Set information is used by the RAN for PDU Set Integrated and Differentiated Handling as described in clause 5.37.x.

The SMF may specify the Service Protocol to be used for identifying PDU Sets in a service flow. The Service Protocol indicates the header (e.g. RTP/SRTP) or payload type to be used by the UPF for PDU Set Identification. The SMF may determine the Service Protocol based on PDU Set related assistance information provided by the AF as described in clause 5.37.x.  Alternatively the Service Protocol or UPF implementation specific information for identifying PDU Sets may be provisioned locally at the UPF.

NOTE xx: Since more than one service flow may be mapped to a QoS Flow, the SMF or local provisioning at the UPF may specify more than one Service Protocol to be detected within a QoS Flow.

For each Service Protocol supported by a UPF, local configuration at the UPF specifies PDU Set identification rules that map the attributes of the Service Protocol (e.g. RTP header fields and values) to PDU Sets. For each of these PDU Sets, local configuration also specifies an Importance Value ranging from 0 (most important) to 3 (least important).

For each DL PDU received on N6 or N19 for which a Service Protocol has been determined, the UPF applies the rules for PDU Set identificastion and determines PDU Set Information which it provides to the RAN in the GTP-U header. The PDU Set Information comprises:

-	PDU Set Sequence Number, which is a cyclical counter incremented for each PDU Set instance.

-	End PDU of the PDU Set, which is a flag that indicates the last PDU of a PDU Set instance

-	PDU Sequence Number within a PDU Set, which is a cyclical counter incremented for each PDU within a PDU Set and is reset at PDU Set boundaries.

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the importance of a PDU Set within a QoS flow.

NOTE yy: The RAN may use the PDU Set Importance for PDU Set level packet discarding in presence of congestion.

NOTE zz: PDU set size related info of the RTP header can include RTP overhead or not, which depends on AS implementation. PDU set importace related info of the RTP header can be securely delivered because if a PDU set is higher value then it is vunerable against attacks.

# * * * *6th change * * * *

6.2.5	NEF

6.2.5.0	NEF functionality

The Network Exposure Function (NEF) supports the following independent functionality:

-	Exposure of capabilities and events:

NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing as described in clause 5.13.

NEF stores/retrieves information as structured data using a standardized interface (Nudr) to the Unified Data Repository (UDR).

-	Secure provision of information from external application to 3GPP network:

It provides a means for the Application Functions to securely provide information to 3GPP network, e.g. Expected UE Behaviour, 5G-VN group information, time synchronization service information and service specific information and PDU Set Integrated Indication and descrition of service protocol, which is used for UPF’s traffic inspection in PDU Set level(e.g. encoding schemes, RTP extended header’s input factors). In that case the NEF may authenticate and authorize and assist in throttling the Application Functions.

-	Translation of internal-external information:

It translates between information exchanged with the AF and information exchanged with the internal network function. For example, it translates between an AF-Service-Identifier and internal 5G Core information such as DNN, S-NSSAI, as described in clause 5.6.7.

In particular, NEF handles masking of network and user sensitive information to external AF's according to the network policy.

-	Redirecting the AF to a more suitable NEF/L-NEF e.g. when serving an AF request for local information exposure and detecting there is a more appropriate NEF instance to serve the AF's request.

-	The Network Exposure Function receives information from other network functions (based on exposed capabilities of other network functions). NEF stores the received information as structured data using a standardized interface to a Unified Data Repository (UDR). The stored information can be accessed and "re-exposed" by the NEF to other network functions and Application Functions, and used for other purposes such as analytics.

-	A NEF may also support a PFD Function: The PFD Function in the NEF may store and retrieve PFD(s) in the UDR and shall provide PFD(s) to the SMF on the request of SMF (pull mode) or on the request of PFD management from NEF (push mode), as described in TS 23.503 [45].

-	A NEF may also support a 5G-VN Group Management Function: The 5G-VN Group Management Function in the NEF may store the 5G-VN group information in the UDR via UDM as described in TS 23.502 [3].

-	Exposure of analytics:

NWDAF analytics may be securely exposed by NEF for external party, as specified in TS 23.288 [86].

-	Retrieval of data from external party by NWDAF:

Data provided by the external party may be collected by NWDAF via NEF for analytics generation purpose. NEF handles and forwards requests and notifications between NWDAF and AF, as specified in TS 23.288 [86].

-	Support of Non-IP Data Delivery:

NEF provides a means for management of NIDD configuration and delivery of MO/MT unstructured data by exposing the NIDD APIs as described in TS 23.502 [3] on the N33/Nnef reference point. See clause 5.31.5.

-	Charging data collection and support of charging interfaces.

-	Support of UAS NF functionality:

Details are defined in TS 23.256 [136].

-	Support of EAS deployment functionality:

Details are defined in TS 23.548 [130].

-	Support of SBI-based MO SM transmit for MSISDN-less MO SMS:

Details are defined in TS 23.540 [142].

A specific NEF instance may support one or more of the functionalities described above and consequently an individual NEF may support a subset of the APIs specified for capability exposure.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

The services provided by the NEF are specified in clause 7.2.8.

For external exposure of services related to specific UE(s), the NEF resides in the HPLMN. Depending on operator agreements, the NEF in the HPLMN may have interface(s) with NF(s) in the VPLMN.

When a UE is capable of switching between EPC and 5GC, an SCEF+NEF is used for service exposure. See clause 5.17.5 for a description of the SCEF+NEF.

# * * * * 6th change * * * *

5.37	Support for high data rate low latency services

5.37.1	General

Interactive services that require high data rate and low latency communication, e.g. cloud gaming and AR/VR/XR services are documented in TS 22.261 [2]. Standardized 5QI characteristics for such services are provided in Table 5.7.4-1 and TSCAI can be used to describe the related traffic characteristics as defined in clause 5.27.2.

5.37.x	PDU Set Integrated and Differentiated Handling for Extended Reality (XR)

Integrated and differentiated handling is provided in the NG-RAN for those PDUs within a QoS flow identified as belonging to a PDU Set. A PDU Set is comprised of one or more PDUs carrying a common application layer payload such as a video frame or video slice. Integrated PDU set handling constitutes common handling of PDUs associated with the same PDU Set instance while differentiated PDU set handling constitutes differentiated handling of the groups of PDUs associated with different PDU Set instances. The handing is determined by PDU Set QoS parameters specified in clause 5.7.3X and PDU Set information received by the RAN in GTP-U header markings provided by the UPF as described in clause 5.8.2.x.

NOTE xx: In this release of the specification, PDU Set integrated and differentiated handling is supported only on the downlink.

The AF may provide PDU Set related assistance information for dynamic PCC. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS23.502[3].

-	QoS parameters for PDU Set handlinging described in clause 5.7.1.2

-	Burst periodicity.

-	Service Protocol: Indicates RTP/SRTP header type to be used for PDU Set Identification at the UPF.

AF provided PDU Set QoS Parameters and Service Protocol may be used in determining the QoS Profile while Burst Periodicity may be used to determine the TSCAI Periodicity parameter.

NOTE zz: PDU set size related info of the RTP header can include RTP overhead or not, which depends on AS implementation. PDU set importance related info of the RTP header can be securely delivered because if a PDU set is higher value then it is vulnerable against attacks.

# * * * * End of changes * * * *