3GPP TSG-SA WG2 Meeting #154AHE	S2-2300764

Electronic meeting, Jan 16 - 20, 2023                   	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0192 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for Communication Path Switching between PC5 and Uu |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FS_5G_ProSe_Ph2 studies the key issue of communication path switching between PC5 and Uu; and it concuded the path switching based on policy will be included in Release 18 of TS 23.304. This CR is submitted to introduce the procedures for communication path switching between PC5 and Uu. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures for communication path switching between PC5 and Uu. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for communication path switching between PC5 and Uu are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** FIRST CHANGE (All New Text) ******************

6.X	Procedures for communication path switching between PC5 and Uu

6.X.1	General

The communication path switching between PC5 and Uu is based on the path permission policy as specified in clause 5.X.

The procedure for communication path switching from PC5 to Uu is described in clause 6.X.2; and the procedure for communication path switching from Uu to PC5 is described in clause 6.X.3.

6.X.2	Procedure for communication path switching from PC5 to Uu

Figure 6.X.2-1 shows the procedure for communication path switching from PC5 to Uu.

Figure 6.X.2-1: Path switching from PC5 to Uu procedure

0.	Service authorization and provisioning are performed for the UE-1 and UE-2 as described in clause 6.2.

1. 	UE-1 and UE-2 have an established PC5 connection which are transferring service data with each other.

2. 	The PC5 communication path quality for the ongoing ProSe services cannot satisfy the service requirements.

Whether the PC5 communication path quality satisfies the requirements of the ProSe service will be based on the traffic loss rate and delay for the service which is measured at application layer, or by performing the measurement of the signal strength of PC5 unicast link between two UEs.

3. 	UE-1 checks the path permission policy and finds the policy for the served ProSe services is both PC5 and Uu path permitted. Then UE-1 decides to switch the ongoing ProSe services to Uu path and sends a path switching negotiation request which includes the ProSe services information for path switching.

4. 	UE-2 receives path switching negotiation request and checks the path permission policy to determine the ProSe services for path switching and responds the ProSe services information to UE-1 in negotiation response.

5. 	UE1 and UE2 triggers PDU Session establishment/modification procedure for the corresponding ProSe services.

The PDU Session is established as defined in clause 4.3.2 of TS 23.502 [5] or modified as defined in clause 4.3.3 of TS 23.502 [5] to transfer the ProSe service data.

6. 	UE-1 and UE-2 transfer service data for the switched ProSe services with each other on Uu communication path.

7. 	UE-1 and UE-2 may release or modify the PC5 connection.

The used PC5 unicast link is released as defined in clause 6.4.3.3 or modified as defined in clause 6.4.3.4.

6.X.3	Procedure for communication path switching from Uu to PC5

Figure 6.X.3-1 shows the procedure for communication path switching from Uu to PC5.

Figure 6.X.3-1: Path switching from Uu to PC5 procedure

0.	Service authorization and provisioning are performed for the UE-1 and UE-2 as described in clause 6.2.

1. 	UE-1 and UE-2 transfer service data with each other on their Uu communication paths.

2. 	The Uu communication path quality for the ongoing ProSe services cannot satisfy the service requirements.

Whether the Uu communication path quality satisfies the requirements of the ProSe service will be based on the traffic loss rate and delay for the service which is measured at application layer.

3. 	UE-1 checks the path permission policy and finds the policy for the served ProSe services is both PC5 and Uu path permitted. Then UE-1 decides to switch the ongoing ProSe services to Uu path and establishes the PC5 unicast link with UE-2.

The PC5 unicast link is established as defined in clause 6.4.3.1

4. 	UE-1 sends a path switching negotiation request which includes the ProSe services information for path switching through PC5 connection.

5. 	UE-2 receives path switching negotiation request and checks the path permission policy to determine the ProSe services for path switching and responds the information to UE-1 in negotiation response.

6. 	UE-1 and UE-2 transfer service data for the switched ProSe services with each other on PC5 communication path.

7. 	UE-1 and UE-2 may release or modify the PDU Session.

The PDU session is released as defined in clause 4.3.4 of TS 23.502 [5] or modified as defined in clause 4.3.3 of TS 23.502 [5].

****************** END OF CHANGES ***************