3GPP TSG-SA2 Meeting #154AHE e-meeting	S2-2300786

# Elbonia, Jan 16 – 20, 2023																			(was 2210555)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0573 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Procedures to monitor ML model accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-81, it was concluded to extend ML Model Provision service for triggering ML accuracy check in MTLF as follows. “- When requesting an ML model via the MLModelProvision service, the AnLF can specify in the request the additional parameters indicating the need for ML model accuracy check.” Also, it was concluded that the MTLF may subscribe some events (i.e., change in the policy, a change in the subscription data, etc.) to trigger ML model accuracy check as follows. “- MTLF with accuracy checking capability of ML models can trigger the analytics accuracy checking based on its internal logic or configuration which may require to subscribe events, e.g. a change in the policy and/or a change in the subscription data for Target of ML Model Reporting, etc.” Once MTLF decides to check ML model accuracy as described above, the MTLF can collect data for monitoring and for building a new test data, and subscribe to AnLF for getting notification of accuracy degradation. “- MTLF determining ML model degradation by collecting new test data (including input data, ground truth data and the corresponding inference) and testing the ML model accuracy. MTLF can compute accuracy by comparing the predictions and the corresponding ground truth data. - MTLF can collect data for monitoring purposes from AnLF, ADRF or other NF. When ADRF is used, the MTLF can retrieve the data by specifying in the request the DataSetTag. - MTLF subscribes to AnLF, that is registered in MTLF with its accuracy monitoring for a model provided by that MTLF, for getting notifications of the accuracy degradation of the analytics generated by the model, where the AnLF determines accuracy information based on any of the following:…” So, this CR tries to implement the principle by adding procedures for ML model accuracy monitoring with extending Nnwdaf_MLModelProvision and proposing new service of AnLF to check accracy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR extends the parameters in Nnwdaf_MLModelProvision service to enable ML model accuracy check in MTLF by an AnLF as well as in AnLF by an MTLF. Also, this CR proposes to add the procedures for MTLF to determine ML model accuracy based on collected data and feedback from AnLF in clause 6.2E.2. and propose to define supported service to get accuracy informaiton from AnLF in clause 7.x Since key issue #1 endorses to collect data from ADRF with DataSetTag, this CR extends Nadrf_DataManagementRetrivalRequest service to inbclude DataSetTag. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 TR conclusion for key issue #1 is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1, 6.2A.2, 6.2E.2 (new), 6.2E.x (new), 7.1, 7.5.1, 7.5.2, 10.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * The 1st change * * * *

6.2A.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2A.1-1 is used by an NWDAF service consumer, i.e. an NWDAF containing AnLF to subscribe/unsubscribe at another NWDAF, i.e. an NWDAF containing MTLF, to be notified when ML Model Information on the related Analytics becomes available, using Nnwdaf_MLModelProvision services as defined in clause 7.5. The ML Model Information is used by an NWDAF containing AnLF to derive analytics. The service is also used by an NWDAF to modify existing ML Model Subscription(s). An NWDAF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.1-1: ML Model for analytics subscribe/unsubscribe

1.	The NWDAF service consumer (i.e. an NWDAF containing AnLF) subscribes to, modifies, or cancels subscription for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2A.2. The service consumer optionally indicates its support for multiple ML models if available.

When a subscription for a trained ML model associated with an Analytics ID is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the subscription; or

-	determine whether triggering further training for the existing trained ML models is needed for the subscription.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in clause 6.2, to generate the ML model.

If the service invocation is for a subscription modification or subscription cancelation, the NWDAF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the NWDAF service consumer subscribes to a (set of) trained ML model(s) associated to a (set of) Analytics ID(s), the NWDAF containing MTLF notifies the NWDAF service consumer with:

-	the trained ML Model Information (containing a (set of) file address(es) of the trained ML model(s)), when multiple ML models is not supported by the consumer; or

-	a set of pair of unique ML Model Identifier and ML Model Information associated with an Analytics ID, when multiple ML models is supported by the consumer.

NOTE 1:	The structure and format of the ML Model identifier and its uniqueness are up to stage 3.

NOTE 2:	Parameters defined for Multiple models are for Analytics accuracy enhancement.

by invoking Nnwdaf_MLModelProvision_Notify service operation. The content of trained ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify service operation to notify either an available re-trained ML model when the NWDAF containing MTLF determines that the previously provided trained ML Model required re-training at step 1, or ML model is degradaded if ML Model Accuracy Level is provided at step 1.

When the step 1 is for a subscription modification (i.e. including Subscription Correlation ID), the NWDAF containing MTLF may provide either a new trained ML model different to the previously provided one, or re-trained ML model by invoking Nnwdaf_MLModelProvision_Notify service operation.

3.	The NWDAF containing MTLF may determine to monitor accuracy of the ML model(s) selected at step 2 as described in clause 6.2D and 6.2E.

6.2A.2	Contents of ML Model Provisioning

The consumers of the ML model provisioning services (i.e. an NWDAF containing AnLF) as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	Information of the analytics for which the requested ML model is to be used, including:

-	A list of Analytics ID(s): identifies the analytics for which the ML model is used.

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML model ML model.

NOTE 1:	The NWDAF containing MTLF can use the parameter "Use case context" to select the most relevant ML model, when several ML models are available for the requested Analytics ID(s). The values of this parameter are not standardized.

-	[OPTIONAL] ML Model Interoperability Information. This is vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.

-	[OPTIONAL] ML Model Filter Information: enables to select which ML model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML model is requested, e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	ML Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

-	[OPTIONAL] Indication of supporting multiple ML models.

-	[OPTIONAL] accuracy level of Interest.

Editor's note:	It is FFS if additional parameters are needed for multiple model provisioning.

- 	Information to enhance the requested ML model accuracy, including:

-	[OPTIONAL] ML Model Accuracy Level: indicates a required level (e.g., Accuracy in Training, Mean Absolute Error, etc.) of accuracy for ML model, and enables to provide notification when the provisioned ML model accuracy is degraded under the level.

-	[OPTIONAL] Accuracy Proviosion Capability Indicator: indicates capability of the service consumer (i.e., an NWDAF containing AnLF) to support accuracy feedback (i.e., Nnwdaf_AccuracyProvision service) for the provisioned ML model.

The NWDAF containing MTLF provides to the consumer of the ML model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	ML Model Information, which includes:

-	the ML model file address (e.g. URL or FQDN) for the Analytics ID(s), when multiple ML models is not supported; or.

-	a set of pair of unique ML Model identifier and the ML model file address (e.g. URL or FQDN) for the Analytics ID(s), if multiple ML models is supported.

-	[OPTIONAL] Validity period: indicates time period when the provided ML Model Information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

NOTE 2:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

-	[OPTIONAL] ML Model Degradation: indicates the provisioned ML model is degraded.

# * * * * The 2nd change * * * *

6.2E.2	Procedures for MTLF ML Model Accuracy Monitoring

The procedure in Figure 6.2E.2-1 is used by an NWDAF containing MTLF to judge ML model degradation. If the NWDAF containing MTLF determines ML model degradation for the provisioned ML model, the NWDAF containing MTLF re-provides an ML model which is retrained or reselected by the procedures as described in clause 6.2A.1.

Figure 6.2E.2-1: ML model accuracy monitoring

- The NWDAF containing MTLF provides an ML model(s) to the NWDAF containing AnLF as described in clause 6.2A.1.

- The NWDAF containing MTLF may subscribe to the PCF events for checking changes in the policies for the UE indicated in Target of ML Model Reporting in Nnwdaf_MLModelProvision_Subscribe service operation invoked by the NWDAF containing AnLF at step 0.

- When the PCF invokes Npcf_UEPolicyControl_Update_response/ Npcf_AMPolicyControl_Update_response/ Npcf_SMPolicyControl_Update_response or Npcf_UEPolicyControl_UpdateNotify/ Npcf_AMPolicyControl_UpdateNotify/ Npcf_SMPolicyControl_UpdateNotify service operation, the PCF notifies to the NWDAF containing MTLF that the occurrence of change in the policy, if step 2 is performed.

- The NWDAF containing MTLF may subscribe to the UDM to get notification on UE subscription data (e.g., Access and Mobility Subscription data, Session Management Subscription data, Slice Selection Subscription data, etc.) change by invoking Nudm_SDM_Subscribe service operation.

- If step 3 is triggered, the UDM may subscribe to the UDR to get notifications of the modification on UE subscription data by invoking Nudr_DM_Subscribe.

- If step 4 is triggered, the UDR sends the notification of data changes from the UDR regarding on the UE subscription data as indicated in step 4 by invoking Nudr_DM_Notify.

- If step 3 is triggered, the UDM notifies the change in UE subscription data indicated in Step 3 by invoking Nudm_SDM_Notification.

- The NWDAF containing MTLF determines to check the provisioned ML model accuracy based on the internal logic or configuration which may exploit the notifications from step 2 and step 6, and/or the provided ML Model Accuracy Level in Nnwdaf_MLModelProvision_Subscribe service operation by the NWDAF containing AnLF at step 0.

When the NWDAF containing MTLF decide to check the provisioned ML model accuracy, the NWDAF containing MTLF may:

- request to check accuracy of the provisioned ML model to the NWDAF containing AnLF, if Accuracy Provision Capability Indicator is provided in Nwdaf_MLModelProvision_Subscribe service operation at step 0; and/or

- trigger to collect data from the NWDAF containing AnLF, ADRF, or other NF to monitor or to build a new test data (including input data, ground truth data and corresponding inference).

- If the NWDAF containing AnLF supports accuracy feedback (i.e., Nnwdaf_AccuracyProvision service), the NWDAF containing MTLF may request to check accuracy of the provisioned ML model to the NWDAF containing AnLF by invoking Nnwdaf_AccuracyProvision_Subscribe service operation enabling Measured Accurcay Request Indicator with notification correlation ID for Nnwdaf_MLModelProvision service as described in Step 0, and/or, the NWDAF containing MTLF may trigger to collect test data from the NWDAF containing AnLF by invoking Nnwdaf_AccuracyProvision_Subscribe service operation with enabling Accuracy related Data Request Indicator.

- The NWDAF containing AnLF determines accuracy of the provision ML model as described in clause 6.2D.

- The NWDAF containing AnLF sends Accuracy  Information and ML Model identifier to the NWDAF containing MTLF by invoking Nnwdaf_AccuracyProvision_Notify service operation.

- The NWDAF containing MTLF may subscribe to collect data to monitor the accuracy of the provisioned ML model by invoking Ndccf_DataManagement_Subscribe service operation.

- The DCCF provides the requested data to the NWDAF containing MTLF by invoking Ndccf_DataManagement_Notify.

- If the information to get data used for generating inference is provided at step 10, then the NWDAF containing MTLF retrieves the data from indicated ADRF with DataSetTag by invoking Nadrf_DataManagement_RetrievalRequest including Storage Transaction Identifier.

- The NWDAF containing MTLF may subscribe to collect data to monitor the accuracy of the provisioned ML model by invoking Nnf_EventExposure_Subscribe service operation.

- The NF provides the requested data to the NWDAF containing MTLF by invoking Nnf_EventExposure_Notify

- The NWDAF containing MTLF determine to re-train the provisioned ML model and/or to re-select ML model for the requested analytics by step 0.

6.2E.X	Contents of Accuracy Provisioning

The consumers of the accuracy provisioning service (i.e. an NWDAF containing AnLF) as described in clause 7.x  may provide the input parameters as listed below:

-	Information of the ML model for which the requested acccuracy is to be measured, including:

-	A list of ML model identifiers: identifies the ML model for which the accuracy is to be measured.

-	Measured Accuracy Request Indicator: indicates the service consumer to get measured accuracy of the ML model.

-	Accuracy related Data Request Indicator: indicates the service consumer to get the input data, ground truth data and corresponding inference for the ML model.

-	[CONDITIONAL] Notification Correlation ID of ML Model Provisioning service, which allows to correlated notifications received from the subscription of Nnwdaf_MLModelProvision service provided by the service consumer (i.e., NWDAF containing MTLF) with this subscription.

-	[OPTIONAL] Use case context: indicates the context of use of the ML model.

-	[OPTIONAL] Target of Accuracy Reporting: indicates the object(s) for which accuracy provisioning is requested, e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	[OPTIONAL] ML Model Accuracy Level: indicates a required level (e.g., Accuracy in Training, Mean Absolute Error, etc.) of accuracy for ML model, and enables to provide notification when the provisioned ML model accuracy is degraded under the level.

-	Accuracy Reporting Information: parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3].

The NWDAF provides to the consumer of the Nnwdaf_AccuracyProvision_Notify service operations described in clause 7.x, the output information listed below:

- The Notification Correlation ID,

- For each ML model identifier, the Accuracy Information:

  - Measured Accuracy Information: includes measured accuracy level (e.g., Accuracy in Use, Mean Absolute Error, etc.) of the ML model by service consumer;

  - Accuracy related Data Information: may include either the collected data used for measuring the accuracy of ML model, or information to retrieved the collected data such as ADRF ID, Storage Transaction Identifier, and DataSetTag.

- [OPTIONAL] ML Model Degradation: indicates the provisioned ML model is degraded.

# * * * * The 3rd change * * * *

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_AccuracyProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |

# * * * * The 4th change * * * *

7.5.1	General

Service Description: This service enables the consumer to receive a notification when an ML model matching the subscription parameters becomes available.

When the subscription is accepted by the NWDAF containing MTLF, the consumer NF, i.e. the NWDAF containing AnLF, receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription. The modification of ML model subscription can be enforced by NWDAF based on operator policy and configuration.

7.5.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe.

Description: Subscribes to NWDAF ML model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model subscription), ML Model Filter Information to indicate the conditions for which ML model for the analytics is requested and Target of ML Model Reporting to indicate the object(s) for which ML model is requested (e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs)), ML Model Reporting Information (including e.g. ML Model Target Period), Expiry time, Use case context, indication of support for multiple ML models, multiple ML models Filter Information to indicate the conditions for which multiple ML models are requested, ML Model Accuracy Level, Accuracy Proviosion Capability Indicator.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

# * * * * The 5th change * * * *

10.2.5	Nadrf_DataManagement_RetrievalRequest service operation

Service operation name: Nadrf_DataManagement_RetrievalRequest

Description: The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF. The Nadrf_DataManagement_RetrievalRequest response either contains the data or analytics, or provides instructions for fetching the data or analytics. The Nadrf_DataManagement_RetrievalRequest may be unsolicited (e.g. when the consumer itself has known "Storage Transaction Identifier") or sent in response to a Fetch Instructions received from the ADRF in an Nadrf_DataManagement_RetrievalNotify.

Inputs, Required: one of the following:

-	Storage Transaction Identifier; or

-	Fetch Correlation ID(s) if the RetrievalRequest is in response to a Fetch Instruction received from the ADRF in an Nadrf_DataManagement_RetrievalNotify.

Inputs, Optional: DataSetTag.

Outputs Required: Result Indication.

Outputs, Optional: Data or Analytics.

# * * * * End of changes * * * *