3GPP TSG-WG SA2 Meeting #154AH-e 	S2-2300807

Elbonia, Jan 16th – 20th, 2022	(revision of)

Source:	Samsung

Title:	Evaluation and Conclusion for FS_GMEC KI#4

Document for:	Approval

Agenda Item:	9.8.1

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing Evaluation and Conclusion for KI#4.

1. Introduction/Discussion

It is proposed to update the conclusion for KI#4 based on evaluations of solutions for KI#4.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-74.

# * * * * First change * * * *

7.4	Key Issue #4: Multiple SMFs for VN group communication

There are 6 solutions (sol#3, sol#4, sol#5, sol#16, sol#19, and sol#20) for KI#4.

The multiple SMFs serving the same 5G VN group can belong to a single SMF Set like sol#3 and sol#16 or different SMF Sets like sol#4, sol#5, sol#19, sol#20, and sol#16.

- AF can set the Single-SMF indicator for a group to indicate whether multiple-SMFs is needed for this group.

- In case when the same 5G VN group belongs to a single SMF sets, I-UPF can support local switching as instructed by A-SMF.

- In case when the same 5G VN group belongs to different SMF sets, GSMF (can collocate with existing NF e.g. NRF, UDM) is used for discovery and registration of SMFs serving a 5G VN group.

- N19 tunnels between UPFs controlled by different SMFs are set up via SMF interactions and used for traffic forwarding. SMF interactions can occur via Nx interface or via GSMF. The topology of UPFs controlled by different SMFs is determined based on network policies, e.g. Relay-SMF/UPF.

- N6 interface between UPFs controlled by different SMFs can also be used for traffic forwarding using existing/implementation mechanism with the constraint that the delay and bandwidth of the N6 interface between UPFs are not controlled by 3GPP.

8.4	Key Issue #4: Multiple SMFs for VN group communication

The following are way forward principles.

-	The multiple SMFs serving the same 5G VN group can belong to a single SMF Set or different SMF Sets

-	AF/NEF can set the Single-SMF indicator for a 5G VN group to indicate the SMF whether the procedures used for multiple SMFs is applicable for this 5G VN group and this Single-SMF indicator is stored as part of the group data in group subscription data, or

-	Single-SMF indicator can be pre-configured for the VN group in 5GC according operator’s configuration.

-	If N19-based forwarding is used between UPFs controlled by different SMFs

-	GSMF is used for discovery and registration of SMFs serving a 5G VN group

-	N19 tunnels between UPFs controlled by different SMFs are set up via SMF interactions. The SMF interactions can occur via Nx interface (between SMFs) or via GSMF.

-	The topology of UPFs controlled by different SMFs is determined based on network policies, e.g. Relay-SMF/UPF, where the network policies are determined by GSMF and indicated to the involved SMFs.

NOTE:	GSMF NF to support the above functions can collocate with other existing NFs.

-	If N6-based forwarding is used between UPFs controlled by different SMFs

-	N6 interface between UPFs controlled by different SMFs can also be used for traffic forwarding using existing/implementation mechanism

# * * * * End of changes * * * *