3GPP TSG-WG SA2 Meeting #154AH-e	S2-2300858

16 - 20 January, 2023, Electronic Meeting	(revision of S2-230xxxx)

# Title:	[DRAFT] Reply LS on NSSRG restriction across different access types over different PLMNs

# Response to:	LS on NSSRG restriction across different access types over different PLMNs (S2-2300023/C1-227101)

# Release:	Rel-17

# Work Item:	eNS_Ph2

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Kundan Tiwari

E-mail Address:	Kundan.tiwari@india.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 CR#xxxx

1. Overall Description:

SA2 would like to thank CT1 regarding the LS on NSSRG restriction across different access types over different PLMNs. SA2 provides following answers to CT1 questions.

Question #1: Shall the Requested NSSAI across different access types over different PLMNs share common NSSRG value(s)?

SA2 Answer: Yes. The UE shall include S-NSSAIs that share a common NSSRG in the Requested NSSAI when the UE is registered to different PLMNs over 3GPP access and non-3GPP access.

Question #2: If the answer of Question #1 is yes, how does the network (i.e. different AMFs in different PLMNs) enforce the common NSSRG value(s) restriction on the Requested NSSAI?

SA2 Answer: In most of the cases, the UE sends compatible slices as per the latest received NSSRG policy. The AMF enforcement is needed only for the case when the UE doesn’t have latest NSSRG policy. NSSRG enforcement on network side is not a FASMO scenario. Hence no change is needed in Rel-17. The AMF enforcement is defined in Rel-18 as per the attached CRs.

Attached Rel-17 and Rel-18 CRs clarify the UE and AMF behaviour when the UE is registered to different PLMNs over 3GPP access and non-3GPP access.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting#155	February 20 – 24, 2023	Athens, Greece

TSG-SA2 Meeting#156	April 17 – 21, 2023	E-Meeting