SA WG2 Meeting #154AHE (e-meeting)	S2-2300894

Jan 16 – 20, 2023	(revision of S2-23xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0663 | rev | 0 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add application descriptor and connection capability into output of OSE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As the conclusion in TR 23.700-81, the NWDAF assisted URSP rule is agreed to normative. And in the output of OSE, the Application service experiences of certain S-NSSAI, DNN, FQDN, destination IP address of application server can be provided by NWDAF to PCF to assist the URSP rule generation. But in the URSP rule traffic descriptor, the Application Descriptor and Connection Capability are not in the output of OSE. It is useful for the following Scenario: The PCF can request the analytic from NWDAF, and the content of analytic can be associated with the certain Traffic Descriptor, for example, the Application descriptor, Domain Descriptor and etc. And the PCF can adjust the RSD or RSCs under a certain Traffic Descriptor or generate the new URSP rule (traffic descriptor and RSD). So, in the output of OSE, that add the output of application descriptor and connection capability are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the output of OSE, that add the output of application descriptor and connection capability under the Application service experiences. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The only two traffic descriptor are missing in output of OSE in NWDAF. And the PCF can’t generate the new URSP rule for the traffic descriptor of application descriptor and connection capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID (NOTE 2) | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| Application service experiences (0..max) | List of observed service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Application Descriptor | Identify the Application(s) that is(are) running on the UE's OS |
| > Connection Capabilities | Identify the UE application when it requests a network connection with certain capabilities. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location (NOTE 1, NOTE 5) | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered. |
| > UPF Info (NOTE 4) | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) with the same application service experience. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > RAT Type (NOTE 7) | Indicating the list of RAT type(s) for which the application service experience analytics applies. |
| > Frequency (NOTE 7) | Indicating the list of carrier frequency value(s) of UE's serving cell(s) where the application service experience analytics applies. |
| > SSC Mode | SSC Mode selected for the PDU Session used to associate with the application. |
| > PDU Session Type | Type of PDU Session used to associate with the application. |
| > Access Type | Access Type when the UE establishes a PDU Session for the application. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested. NOTE 2: The NSI ID is an optional parameter. If not provided the Slice instance service experience indicates the service experience for the S-NSSAI. NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 4: If the consumer NF is an AF, the "UPF info" shall not be included. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. UE location shall only be included if the Consumer analytics request is for single UE or a list of UEs. Inclusion of UE location requires user consent. NOTE 6: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. NOTE 7: When "any" value has been provided in the request (e.g. "any" RAT type, "any" frequency, or "any" for all the RAT type and frequency indication), the NWDAF provides an instance of the Application service experience per combination of RAT Type(s) and/or Frequency value(s) having the same Service Experience. |  |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID (NOTE 2) | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| Application service experiences (0..max) | List of predicted service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Application Descriptor | Identify the Application(s) that is(are) running on the UE's OS |
| > Connection Capabilities | Identify the UE application when it requests a network connection with certain capabilities. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location (NOTE 1, NOTE 5) | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered. |
| > UPF Info (NOTE 4) | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) with the same application service experience. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| > RAT Type (NOTE 7) | Indicating the list of RAT type(s) for which the application service experience analytics applies. |
| > Frequency (NOTE 7) | Indicating the list of carrier frequency value(s) of UE's serving cell(s) where the application service experience analytics applies. |
| > SSC Mode | SSC Mode selected for the PDU Session used to associate with the application. |
| > PDU Session Type | Type of PDU Session used to associate with the application. |
| > Access Type | Access Type when the UE establishes a PDU Session for the application. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 2: The NSI ID is an optional parameter. If not provided the Slice instance service experience indicates the service experience for the S-NSSAI. NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 4: If the consumer NF is an AF, the "UPF info" shall not be included. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. UE location shall only be included if the Consumer analytics request is for single UE or a list of UEs. Inclusion of UE location requires user consent. NOTE 6: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. NOTE 7: When "any" value has been provided in the request (e.g. "any" RAT type, "any" frequency, or "any" for all the RAT type and frequency indication), the NWDAF provides an instance of the Application service experience per combination of RAT Type(s) and/or Frequency value(s) having the same Service Experience. |  |

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided as part of Analytics Reporting Information by the NWDAF Service Consumer.

* * * End of Change* * *