3GPP TSG-WG SA2 Meeting #154-AH e-meeting 	S2-2300903

Electronic, 16 January - 20 January 2023	(revision of S2-230xxxx)

Source:	Nokia

Title:	Resolution of open questions in DetNet TR

Document for:	Approval

Agenda Item:	9.27.1

Work Item / Release:	FS_DetNet / Rel-18

Abstract: Resolve the open questions that remain in the DetNet TR 23.700-46

# 1. Exposure of uplink routing information

A UPF can have multiple interfaces on N6. There can be various routes to reach the same host on N6. DetNet controller acting as SDN controller needs to know the UL routes for selecting the required forwarding path to reach the same host. As UPF is not a typical SDN-compliant router, it is not possible for DetNet Controller acting as SDN controller to collect the route information via the OpenFlow interface.

UE

RAN

UPF

SMF

AMF

PCF

TSCTSF

R

CPF:

DetNet

controller

DetNet

network

5GS

logical

DetNet

Node

DetNet

system

R

R

Figure 1-1: Multiple Routes on N6

Even in the case of static route configuration at N6, it is not practical to manually sync the route configuration between the DetNet controller and each N6 interface of every UPF in the network.

Hence it requires 3GPP standards change to expose the static or dynamic routes on the N6 interface toward the DetNet controller via TSCTSF. DetNet controller will use the route information from 5GS to influence the route selection for DetNet flows.

As stated in IETF’s “draft-ietf-detnet-controller-plane-framework-03”; in clause 3.2, “A Centralized Controller collects topology information and DetNet capabilities of the network nodes via NETCONF/YANG”.

The controller can collect the DetNet capability of the network nodes or, in our case, available UPF’s N6 forwarding paths per interface. The N6 route information can be exposed to the controller via SMF/PCF/TSCTSF using NETCONF/YANG model as per IETF RFC 8349.

The controller may use this information to choose appropriate routes/forwarding path in UL. The IETF’s “draft-ietf-detnet-controller-plane-framework-03” in clause 4.1 states, “The computed path for the DetNet service has to be sent/configured/signaled to the network device, so the corresponding DetNet flow could pass through the network domain following the specified path”.

# 3. Text Proposal

It is proposed to capture the following changes to TR 23.700-46.

# * * * * First change * * * *

## 7.2	Key Issue #1: 5GS DetNet node reporting

The 5GS is exposed by the TSCTSF to the DetNet controller as a router on a per UPF granularity. The node may be identified by a Node ID. The interfaces correspond to the PDU Sessions (and to the network side interfaces (i.e. including the interfaces of UPF and NW-TT). Each interface is identified by an interface identifier.

The following information may be reported from TSCTSF to DetNet controller for each interface.

-	Type of interface.

-	IP address.

-	subnet (prefix length).

-	Neighbour address (in case of network side interfaces).

-	MAC address (in case of network side interfaces).

-	N6 IP forwarding table entries on a per N6 interface

NOTE: How the UPF/NW-TT determines the route/forwarding information is out of the scope of 3GPP

-	MTU size.

The TSCTSF collects the information from the UPF/NW-TT and the SMF re-using the existing procedures in Rel-17 TSC, with the addition of new parameters as needed.

# * * * * End of changes * * * *