3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300924

# Elbonia, 16-20 January 2023	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3816 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing Redundant Steering Mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, InterDigital, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on conclusions of FS_ATSSS_Ph3 on KI#3 in TR 23.700-53 clause 8.2, a new redundant traffic steering mode is introduced in Rel-18. It can work with and without threshold values for PLR and RTT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new redundant steering mode is defined and the option to suspend and resume duplication of traffic via a new PMF message from UPF to UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 specifications not aligned with the Rel-18 study conclusions on redundant steering in TR 23.700-53 clause 8.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Begin of changes * * * *

### 4.22.4	Access Network Performance Measurements

The PMF of UE side or/and UPF side should be able to correlate the measurement packets with the corresponding access type in order to get the accurate measurement result for each access. The PMF of UE side correlates the sent measurement request and received measurement response messages via the same access type and the PMF of UPF side correlates the sent measurement request and received measurement response messages via the same N3 or N9 Tunnel. The PMF of UPF side shall record the relationship between the RTT measurement result and the N3 or N9 Tunnel.

NOTE:	The frequency for RTT measurement for each access is decided by the PMF of the UE and the UPF respectively.

### 4.22.5	Reporting of Access Availability

After the MA PDU session is established, if Reporting of Access Availability is required by network, the UE performs detection of the unavailability and availability of an access based on implementation. To report the availability/unavailability of the access, the UE sends the PMF-Access Report to the UPF via the user plane of any available access of the MA PDU session. The UPF shall use this report to decide which access can be used to deliver the downlink packets.

### 4.22.X	Suspend and Resume Traffic Duplication

After the MA PDU session is established and the Redundant steering mode is active, the UPF may suspend traffic duplication by sending PMF-Suspend Duplication Request to the UE via the user plane of any available access of the MA PDU session. The PMF-Suspend Duplication Request may contain the type (GBR or Non-GBR) of SDFs for which traffic duplication is suspended. Once the UE receives PMF-Suspend Duplication Request from the UPF, the UE shall stop duplicating traffic for all SDFs for which traffic duplication is suspended. The UPF may resume traffic duplication for a UE by sending PMF-Resume Duplication Request via the user plane of any available access of the MA PDU session. Once the UE receives PMF-Resume Duplication Request from the UPF, the UE may duplicate traffic for all SDFs for which traffic duplication is resumed based on the provided Redundant steering mode policies and UE implementation.

# * * * * End of changes * * * *