# 3GPP TSG-SA WG2 Meeting #154-AH-e	S2-2300932

Electronic Meeting, 16 - 20 January, 2023	(Revision of S2-220xxxx)

Title:	[Draft]Reply LS on long eDRX support for RRC_INACTIVE

Response to:	Reply LS on long eDRX support for RRC_INACTIVE

Release:	Rel-18

# Work Item:	     NR_REDCAP_Ph2

Source:	SA2

To:	RAN3, RAN2

cc:

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR 3849, 23.502 CR 3685

1. Overall Description:

SA2 thanks RAN3 for the answers provided in S2-2300055/R3-226776 and waits for further responses for issues that RAN3 indicated with no agreement yet.

SA2 would also like to inform RAN3 and RAN2 that SA2 agreed CRs related to the triggering of the RAN request for CN to handle the MT communication and related to the eDRX parameter handling for RRC_INACTIVE.

2. Actions:

To RAN3, RAN2

ACTION: 	SA2 kindly asks RAN3 and RAN2 to take above information into consideration.

3. Date of Next SA2 Meeting:

SA2#155                   	February 20 - 24, 2023                                  Athens, GR

SA2#156   	April 17 - 21, 2023 				TBD