SA WG2 Meeting #154AH	S2-2300975

# January 16th – 20th, 2023; Elbonia               			(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	23.700-85: Conclusions update for KI#3, remove 2nd EN

Document for: 	Approval

Agenda Item: 	9.25.1

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This pCR proposes conclusions updates for KI#3 to remove 2nd EN

# 1 Discussion

Support of URSPs in EPS was introduced in 3GPP Rel-16. A Rel-16 UE can use URSPs while in EPS (S1 mode) by translating the various parameters in the Traffic Descriptors and Route Selection Descriptors to corresponding EPS parameters as defined in clause 4.4.2 of TS 24.526. There are certain 5GS parameters of a Route Selection Descriptor (RSD) that do not have an exact equivalent in EPS e.g. SSC mode selection, Network Slice selection, DNN selection, Time Window, Location criteria etc. These parameters are currently marked as "not applicable in EPS"(see Table 4.4.2.2 of TS 24.526).

As per the current stage 3 specification in clause 4.4.2 of TS 24.256:

-	When the UE is registered in the EPS, the UE shall not use an RSD of a matching URSP rule if it contains at least one parameter that is marked as "Not applicable in EPS" in the table 4.4.2.2 of TS 24.526

Due to the above restriction, in order to ensure that the UEs in EPS behave as desired, the network operator has to design and provision EPS-specific RSDs that avoid any non-applicable parameter. This results to unnecessary signalling, storage and processing overheads at both UE and network sides, and network management overheads and is error-prone as by misconfiguration inconsistent URSPs between EPS and 5GS may be generated.

Observation 1:

Although the current specification allows a UE to use URSP rules in EPS, designing EPS-specific RSDs brings storage and processing overheads at both UE and network/network management sides. A solution optimisation is desirable where a single URSP rule could be used both in 5GS and EPS. This will also ensure consistent UE behaviour across 5GS and EPS.

Solution #17 in the TR 23.700-85 addresses the above issue by introducing a new "EPS applicability" parameter in the RSD. This parameter indicates how the UE shall handle the RSD component in the EPS, e.g. whether to ignore the component and use the remaining RSD components to route the traffic or to consider RSD as not applicable etc.

Notice that the issue (as mentioned in observation 1) was identified in Rel-17 and for the newly introduced parameters, PDU session pair ID and RSN, applicability was set as "ignored in EPS" (see Table 4.4.2.2 of TS 24.526), and hence the same URSP rule can be used in EPS, as per the following UE handling in clause 4.4.2 of TS 24.256:

"-	one or more parameters ignored in EPS, the UE shall evaluate the route selection descriptor without considering the one or more parameters ignored in EPS."

Mapping table for route selection descriptor parameters (from Table 4.4.2.2 of TS 24.526)

| Route selection descriptor parameter name | Description | Mapped EPS parameter description |
| --- | --- | --- |
| Route selection descriptor precedence | Determines the order in which the route selection descriptors are to be applied | Determines the order in which the route selection descriptors are to be applied |
| SSC Mode Selection | One single value of SSC mode | Ignored in EPS if set to SSC mode 1 Not applicable in EPS if set to SSC mode 2 or 3 |
| Network Slice Selection | Either a single value or a list of values of S-NSSAI(s) | Not applicable in EPS |
| DNN Selection | Either a single value or a list of values of DNN(s) | Either a single value or a list of values of APN(s). Not applicable in EPS if it contains at least one LADN DNN |
| PDU Session Type Selection | One single value of PDU Session Type | PDN type: - PDU session type "Unstructured" is mapped to PDN type "non-IP". - PDU session type "Ethernet" is mapped to PDN type "Ethernet", if supported by the UE. Otherwise PDU session type "Ethernet" is mapped to PDN type "non-IP" |
| Non-Seamless Offload indication | Indicates if the traffic of the matching application is to be offloaded to non-3GPP access outside of a PDU session | Indicates if the traffic of the matching application is to be offloaded to non-3GPP access outside of a PDN connection |
| 5G ProSe layer-3 UE-to-network relay offload indication | Indicates if the traffic of the matching application is to be offloaded to 5G ProSe layer-3 UE-to-network relay outside of a PDU session | Not applicable in EPS |
| Access Type preference | Indicates the preferred Access Type (3GPP or non-3GPP) when the UE establishes a PDU Session for the matching application | preferred Access Type (3GPP or non-3GPP) |
| Multi-Access preference | Indicates that the PDU session should be established as a multi-access PDU session, using both 3GPP access and non-3GPP access. | Not applicable in EPS |
| Time window | The time window when the matching traffic is allowed. | Not applicable in EPS |
| Location criteria | The UE location where the matching traffic is allowed. | Not applicable in EPS |
| PDU session pair ID | One single value of PDU session pair ID for redundant PDU session establishment. | Ignored in EPS |
| RSN | One single value of RSN for redundant PDU session establishment. | Ignored in EPS |

For certain features like edge computing, "Time Window" and "Location Criteria" are spatial and temporal criteria which could be used to determine the validity of URSP rules. But since these are currently marked as "Not applicable in EPS", the UE will ignore this RSD to route an application traffic in the EPS and will have to consider another RSD of lower precedence.

Solution #17 proposes to generalize this mechanism, allowing the operator to easily configure what holds for each parameter of an RSD while the UE is in EPS.

Observation 2:

The operator cannot enforce certain parameters, like temporal and spatial validity conditions for application traffic, while the UE is in EPS. Also, in this case the network operator will have to design and provision extra EPS-specific RSDs, as the validity conditions cannot be applied in EPS.

Solution #17 in the TR 23.700-85 addresses the above issue by introducing a new "EPS applicability" parameter in the RSD. This parameter indicates how the UE shall handle the RSD component in the EPS, e.g. whether to ignore RSD component and use the remaining RSD components to route the traffic or whether to apply the conditions in the RSD component in EPS also.

The proposed solution is backwards-compatible and always reduces the necessary signalling, since whenever a single applicability indication is used, a whole URSP rule (or at least an extra RSD) is saved (i.e., the EPS-specific version of the rule)

Observation 3:

Backwards compatibility is ensured by design. The lack of the indication means that the handling should be as per Table 4.4.2.2 of TS 24.526.

The interested reader can check a candidate stage-3 implementation proposal of the solution which was discussed as part of Rel-17 in CT1 and though no technical issue was identified, it was postponed and proposed to be handled as part in eUEPO in rel.18.

| C1-220513 | CR | eIMS5G2: Network-indicated applicability of URSP parameters (S-NSSAI) in EPS | Nokia, Nokia Shanghai Bell | Spec: 24.501 CR3958 | CT1#133-bis-e |
| --- | --- | --- | --- | --- | --- |
| C1-220512 | CR | eIMS5G2: Network-indicated applicability of URSP parameters (S-NSSAI) in EPS | Nokia, Nokia Shanghai Bell | Spec: 24.526 CR0135 | CT1#133-bis-e |
| C1-217443 | CR | Support of Network-indicated applicability of URSP parameters in EPS | Nokia, Nokia Shanghai Bell | Spec: 24.501 CR3828r1 | CT1#133-e |
| C1-217442 | CR | Support of Network-indicated applicability of URSP parameters in EPS | Nokia, Nokia Shanghai Bell | Spec: 24.526 CR0132r1 | CT1#133-e |

Proposal:

It is proposed to use Solution #17 as the baseline for normative work to solve the issues and gaps identified in the existing URSP mapping mechanism and thereby addressing the below aspect of the KI#3 in TR 23.700-85:

-	Identify the use cases and scenarios where the UE may need URSP that is consistent across 5GC and EPC.

-	Study whether there are any issues and gaps in the existing URSP mapping mechanism described in clause 5.17.1.2 of TS 23.501 [2], if so, identify them and propose solutions.

It is proposed conclude the below:

-	The RSD in URSP rules is enhanced to contain "EPS applicability" parameter within an RSD component as described in Sol#17. A UE that supports this additional parameter for RSD component shall include an indication for support of the feature during registration.

-	If the UE has indicated support for "EPS applicability" parameter in RSD component, the PCF may generate URSP rules including the "EPS applicability" for one or more RSD components.

- 	The "EPS applicability" parameter in an RSD component may indicate whether the UE shall ignore the parameter and use the rest of the parameters in the RSD when in EPS or whether the UE shall apply the parameter also in EPS. Absence of "EPS applicability" parameter in an RSD would mean the UE behaves as per R17 specification, which ensures backward compatibility.

# 2 Proposal

It is proposed to update 23.700-85 as follows:

FIRST CHANGE

## 8.3	Conclusion on KI#3:

-	The ePCO based URSP delivery to UE in EPS is selected as baseline mechanism for normative work.

-	In order to avoid unnecessary signalling overload for the purpose of delivery URSP to pre-Rel-18 UEs in EPS, the UE needs to indicate the URSP Provisioning Support Indication in EPS in the UE Policy Container to UE-PCF in 5GS.

-	During 5GS to EPS mobility with N26, the AMF doesn't initiate the UE Policy Association termination procedure, instead the AMF will request the UE-PCF to delay the termination of UE Policy Association for a configured period. The UE-PCF can initiate the UE Policy Association Termination procedure either after it receives the UE Policy Association Establishment from SM-PCF or after the configured period. The mobility procedure from 5GS to EPC described in clause 6.16.2.3.3 is selected as the baseline procedure for 5GS to EPS mobility with N26.

-	AMF doesn't provide any new information to MME during 5GS to EPS mobility with N26, the SM-PCF determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. The SM-PCF discovers the address of UE-PCF serving the UE by querying BSF. The UE-PCF recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS handover").

-	When the UE is attached to EPS, the SM-PCF can retrieve the PCRTs for UE Policy from UE-PCF and subscribe to the applicable PCRTs in EPC to SMF+PGW-C.

-	When the UE returns from EPS to 5GS, the old UE policy association in EPS should be terminated after the new UE policy association established in 5GS.

-	When the UE does Initial Attach to EPC, in the default PDN connection is requested to established, the UE includes the UE Policy Container ePCO in PDN Connectivity Request encapsulated in Attach Request; otherwise, the UE will include the UE Policy Container ePCO in the PDN Connectivity Request during the first request for PDN connectivity as described (i.e. as described in Sol#16). The PDN Connection used by UE and SMF/PGW-C to convey UE Policy Container ePCO shall be kept when the UE is in the CONNECTED mode.

-	The PCF triggers the re-evaluation of applicable URSPs for the UE and determines an update of URSP is needed for the UE when an event as described in clause 6.16.11 happens in PCF. The PCF selects one of the PDN connection/PDU sessions associated to EPC for the delivery of the URSP update. Then the PCF generates the corresponding UPDP message MANAGE UE POLICY COMMAND in a similar way than it is done in 5GC and then includes the message into a new IE for sending it to the SMF over N7, then further to UE as described in clause 6.16.2.2.

-	The RSD in URSP rules is enhanced to contain "EPS applicability" parameter within an RSD component as described in Sol#17. A UE that supports this additional parameter for RSD component shall include an indication for support of the feature during registration.

-	If the UE has indicated support for "EPS applicability" parameter in RSD component, the PCF may generate URSP rules including the "EPS applicability" for one or more RSD components.

- 	The "EPS applicability" parameter in an RSD component may indicate whether the UE shall ignore the parameter and use the rest of the parameters in the RSD when in EPS or whether the UE shall apply the parameter also in EPS. Absence of "EPS applicability" parameter in an RSD would mean the UE behaves as per Rel-17 specification, which ensures backward compatibility. Besides, whenever an "EPS applicability" parameter is included in an RSD, it removes the need for an EPS-specific RSD/Rule and hence saves signalling.

-	In Rel-18, the "EPS applicability" parameter can only apply to following RSD components: Location Criteria, Time Window

Editor's note:	It is FFS how to handle the EPS mobility for a UE that is dual registered (i.e. with a registration in 5GS via N3IWF and attached to EPC/E-UTRAN) and thus whether UE policy association can be maintained in both EPS and 5GS or only in 5GS or EPS.

END OF CHANGES