3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300984

# Elbonia, January 16 – 20, 2023		(revision of S2-2210617)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0194 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Communication path switching between PC5 and Uu reference points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR adds communication path switching between PC5 and Uu reference points based on the KI#3 conclusion in TR 23.700-33. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The policy and description communication path switching between PC5 and Uu reference points is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Communication path switching between PC5 and Uu reference points is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.3.1, 5.X (new), 5.X.1 (new), 5.X.2 (new), 5.X.3 (new), 6.X (new), 6.X.1 (new), 6.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 5.X referenced in clauses 5.1.1 & 5.1.3.1 is the new clause 5.X added in this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.1.1	General

In 5GS, the parameters for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, and 5G ProSe UE-to-Network Relay service may be made available to the UE in following ways:

-	provisioned in the ME; or

-	configured in the UICC; or

-	provisioned in the ME and configured in the UICC; or

-	provided or updated by the ProSe Application Server via PCF and/or PC1 reference point; or

-	provided or updated by the PCF to the UE.

If the same parameters described in clauses 5.1.2.1, 5.1.3.1 and 5.1.4.1 are provided by different sources, the UE shall consider them in the following priority order:

-	provided or updated by the PCF (including parameters determined by the PCF itself and parameters provided by the ProSe Application Server to the PCF as specified in clause 6.2.5);

-	provided or updated by the ProSe Application Server via PC1 reference point;

-	configured in the UICC;

-	provisioned in the ME.

The parameters provided or updated by the ProSe Application Server via PC1 reference point may need to be complemented with configuration data from other sources listed above.

NOTE:	The ProSe Application Server can provision the same ProSe parameters via 5GC as specified in clause 6.2.5 or directly to the UE via PC1 reference point, and can revoke (e.g. delete) the ProSe parameters via 5GC as specified in clause 6.2.5 in order for the provisioning via PC1 reference point to take effect.

The basic principles of service authorization and provisioning for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, and 5G ProSe UE-to-Network Relay service are as follows:

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorized to use 5G ProSe Direct Discovery.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to use 5G ProSe Direct Communication.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe UE-to-Network Relay. Authorisation for 5G ProSe Layer-2 UE-to-Network Relay and 5G ProSe Layer-3 UE-to-Network Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to access 5GC via 5G ProSe UE-to-Network Relay (i.e. to act as 5G ProSe Remote UE). Authorisation to access via 5G ProSe Layer-2 UE-to-Network Relay and via 5G ProSe Layer-3 UE-to-Network Relay are independent of each other.

-	The PCF in the HPLMN merges authorization information from home and other PLMNs and provides the UE with the final authorization information.

-	The PCF in the VPLMN or HPLMN may revoke the authorization (via H-PCF when roaming) at any time by using the UE Configuration Update procedure for transparent UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [5].

-	The ProSe Policy/parameters provisioning to UE is controlled by the PCF and may be triggered by UE. The PCF provisions one or more of the following ProSe Policy/parameters:

-	ProSe Policy/parameters for 5G ProSe Direct Discovery as specified in clause 5.1.2.1;

-	ProSe Policy/parameters for 5G ProSe Direct Communications as specified in clause 5.1.3.1;

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 UE-to-Network Relay as specified in clause 5.1.4.1;

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 Remote UE as specified in clause 5.1.4.1.

-	The PCF includes the 5G ProSe Policy/parameters in a Policy Section identified by a Policy Section Identifier (PSI) as specified in clause 6.1.2.2.2 of TS 23.503 [9].

In addition to the above, ProSe usage reporting configuration and rules for charging can be (pre)configured in the UE or provided by the PCF.

In addition to the above, the path selection policy can be (pre)configured in the UE or provided by the PCF as defined in clause 5.11. A path preference for ProSe Services can be provided by ProSe Application Server to UDR, and may be used by PCF for path selection policy generation and update. The path switch policy can be (pre)configured in the UE or provided by the PCF as defined in clause 5.X. A path preference for ProSe Services can be provided by ProSe Application Server to UDR, and may be used by PCF for path switch policy generation and update.

When a 5G ProSe Layer-3 Remote UE is using a 5G ProSe Layer-3 UE-to-Network Relay without involving N3IWF, the PCF based provisioning and update of 5G ProSe Policy/parameters to the 5G ProSe Layer-3 Remote UE are not supported.

# * * * * Second change * * * *

#### 5.1.3.1	Policy/Parameter provisioning for 5G ProSe Direct Communication

The following sets of information for 5G ProSe Direct Communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "served by NG-RAN".

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "not served by NG-RAN".

NOTE 1:	In this specification, [When the UE is "served by NG-RAN"] and [When the UE is "not served by NG-RAN"] are relevant to 5G ProSe Direct Communications over NR PC5 reference point.

2)	Groupcast mode 5G ProSe Direct Communication policy/parameters:

-	For each application layer group supported include the parameters that enable the UE to perform Groupcast mode 5G ProSe Direct Communication when provided by PCF or provisioned in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to.

-	ProSe Layer-2 Group ID: Destination Layer-2 ID applicable only when the Application Layer Group ID is provided by the application layer

-	ProSe Group IP multicast address

-	Indication whether the UE should use IPv4 or IPv6 for that group

-	For a specific Group configured to operate using IPv4, optionally an IPv4 address to be used by the UE as a source address. If none is provisioned, then the UE shall use Dynamic Configuration of IPv4 Link-Local Addresses RFC 3927 [18] to obtain a link local address for the Group.

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 2:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

4)	Policy/parameters related to privacy:

-	The list of ProSe services (i.e. ProSe identifiers) with Geographical Area(s) that require privacy support.

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

5)	Policy/parameters when NR PC5 is selected:

-	The mapping of ProSe services (i.e. ProSe identifiers) to radio frequencies with Geographical Area(s).

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for broadcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for groupcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 3:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one ProSe service. In the case where different ProSe services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one ProSe service, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

NOTE 4:	Security policies for Unicast mode 5G ProSe Direct Communication can be provisioned by PCF as defined in TS 33.503 [29].

-	The mapping of ProSe services (i.e. ProSe identifiers) to PC5 QoS parameters defined in clause 5.6.1 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	The mapping of ProSe services (i.e. ProSe identifiers) to the corresponding NR Tx Profiles for broadcast and groupcast (see TS 38.300 [12] and TS 38.331 [16] for further information).

-	AS layer configurations (see TS 38.331 [16]), i.e. the mapping of PC5 QoS profile(s) to radio bearer(s), when the UE is "not served by NG-RAN".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2 of TS 23.287 [2], and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1 of TS 23.287 [2] and in Table 5.6.1-1.

-	For broadcast, groupcast and initial signalling to establish unicast connection, PC5 DRX configuration (see TS 38.331 [16]), e.g. the mapping of PC5 QoS profile(s) to PC5 DRX cycle(s), default PC5 DRX configuration, when the UE is "not served by NG-RAN".

6)	Path selection policy:

-	The mapping of ProSe services (i.e. ProSe identifiers) to path preference (i.e. PC5 preferred, Uu preferred, or no preference) as defined in clause 5.11.

7)	Path switching policy:

-	The mapping of ProSe services (i.e. ProSe identifiers) to path permission (i.e. PC5 permitted, Uu permitted, or both PC5 and Uu permitted) as defined in clause 5.X.

NOTE 5:	The path selection policy and path switching policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

8)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Direct Communication.

The above parameter sets from bullet 2) to 8) may be provided or updated to the UE by the ProSe Application Server.

# * * * * Third change (all new) * * * *

## 5.X	Communication path switching between PC5 and Uu reference points

### 5.X.1	General

Communication path switching between PC5 and Uu reference points is the procedure for how two UEs switch between Uu communication path and PC5 communication path when they are communicating with each other for ProSe Services. The PC5 communication means that the communication with each other for a ProSe Service is performed by using 5G ProSe Direct Communication only. The Uu communication path means that the communication with each other is performed via the network for a ProSe Service.

NOTE 1:	The communication via a 5G ProSe UE-to-Network Relay (Layer-2 or Layer-3) can be considered as the communication path over Uu reference point, as it involves communication via the network.

A path switching policy is provided to the UE to indicate which path(s) are permitted for all or specific ProSe services (i.e. PC5 permitted, Uu permitted, or both PC5 and Uu permitted) as specified in clause 5.1.3.1.

The path switching policy for a ProSe service is as follows:

-	PC5 permitted: The UE is allowed to perform path switching from a communication path over the Uu reference point to a communication path over the PC5 reference point.

-	Uu permitted: The UE is allowed to perform path switching from a communication path over the PC5 reference point to a communication path over the Uu reference point.

-	Both PC5 and Uu permitted: The UE is allowed to perform path switching from a communication path over the PC5 reference point to a communication path over the Uu reference point, and also is allowed to perform path switching from a communication path over the Uu reference point to a communication path over the PC5 reference point.

UEs negotiate over the existing PC5 unicast link to determine whether and which ProSe services are to be switched, by one UE requesting switch for specific services and the other UE confirming which services can be switched or not. Each UE determines whether and which ProSe services are to be switched by taking into account e.g. its path switching policy or willingness to switch, availability of Uu path, under congestion control or mobility restriction.

### 5.X.2	Communication path switching from PC5 to Uu reference point

A make-before-break mechanism is used when path switching from a communication path over the PC5 reference point to a communication path over the Uu reference point. The UEs prepare the communication path over the Uu reference point, e.g., by establishing or modifying a PDU session, and then performs the path switching from a communication path over the PC5 reference point to a communication path over the Uu reference point. After that they use the communication path over the Uu reference point, and the communication path over the PC5 reference point may be released.

If the UEs maintain a PC5 unicast link after path switching, the UEs can determine to switch back to the communication path over the PC5 reference point from the communication path over the Uu reference point based on e.g. path switch policy, the PC5 signal level of the maintained unicast link.

The Uu QoS parameters of each UE are decided based on PC5 QoS parameters and negotiated via the existing PC5 connection between two UEs. Each UE performs the UE requested PDU Session Modification as defined in TS 23.502 [5] clause 4.3.3 to request the Uu QoS parameters.

The IP addresses used for a communication path over the Uu reference point can be shared between the UEs to assist them to establish communication directly with each other via the communication path over the Uu reference point.

### 5.X.3	Communication path switching from Uu to PC5 reference point

A make-before-break mechanism is used when path switching from a communication path over the Uu reference point to a communication path over the PC5 reference point. The UEs prepare the communication path over PC5 reference point, e.g., establishing or modifying a PC5 unicast link, and then perform the path switching from a communication path over the Uu reference point to a communication path over the PC5 reference point. After that they use communication path over the PC5 reference point, and communication path over the Uu reference point may be released.

To assist the UEs in discovering each other during PC5 discovery or during the Layer-2 link establishment procedure, user info may be provided by the application layer before switching.

# * * * * Forth change (all new) * * * *

## 6.x	Procedures for communication path switching between PC5 and Uu reference points

### 6.x.1	Procedure for communication path switching from PC5 reference point to Uu reference point

Figure 6.x.1-1 depicts the procedure for communication path switching from PC5 reference point to Uu reference point.

Figure 6.x.1-1: Procedure for communication path switching from PC5 reference point to Uu reference point

1.	Service authorization and provisioning are performed for the UE as described in clause 6.2 including the Path switching policy.

2.	UE1 and UE2 have an established PC5 connection for unicast mode communication as described in clause 6.4.3.6.

3.	UE1 decides to perform path switching for ProSe service(s) from a PC5 path to a Uu path, e.g. when the PC5 signal strength of the unicast link with UE2 is below configured signal strength threshold.

UE1 determines whether and which ProSe service(s) can be switched to Uu path based on, e.g., the path switching policy as specified in clause 5.1.3.1, availability of Uu path, congestion control or mobility restrictions.

Upon deciding to perform path switching, UE1 sends a Path Switch Request message to UE2. The message includes the path switching indication from a PC5 path to a Uu path, a list of ProSe service(s) to be switched, the IP address used for UE1’s Uu path if available for each ProSe service, and Uu QoS parameters used for UE2’s Uu path for each ProSe service.

Before sending the Path Switch Request message, UE1 may set up the PDU session(s) for the ProSe services to be switched by using the PDU session establishment procedure as specified in clause 4.3.2 of TS 23.502 [5]. UE1 can decides the Uu QoS parameters of each UE based on PC5 QoS parameters for each ProSe service.

4.	After receiving Path Switch Request from UE1, UE2 may establish or modify PDU Session(s) by using the PDU session establishment procedure as specified in clause 4.3.2 or the PDU session modification procedure as specified in clause 4.3.3 of TS 23.502 [5]. UE2 sets the Requested QoS as the Uu QoS parameters used for UE2’s Uu path for the specific service in the PDU Session Modification Request.

5.	UE2 responds with a Path Switch Accept message.

UE2 determines whether and which ProSe service(s) can be switched to a Uu path based on, e.g., the path switching policy as specified in clause 5.1.3.1, availability of Uu path, congestion control or mobility restrictions.

If UE2 accepts the path switch request, the Path Switch Accept message includes the accepted ProSe service(s), and the IP address used for UE2’s Uu path for each accepted ProSe service.

6.	UE1 may establish or modify PDU Session(s) by using the PDU session establishment procedure as specified in clause 4.3.2 or the PDU session modification procedure as specified in clause 4.3.3 of TS 23.502 [5]. UE1 sets the Requested QoS as the Uu QoS parameters used for UE1’s Uu path for the specific service in the PDU Session Modification Request.

7.	UE1 sends a Path Switching Ack message to UE2, which may include the IP address used for UE1’s Uu path for each accepted ProSe service if not sent during the step 3.

8.	Based on the Path Switch Ack message, UE2 and UE1 switch the traffic of the accepted ProSe service(s) from PC5 path to Uu path.

9.	The PC5 unicast link may be released, e.g. if all ProSe services over the PC5 link are switched.

NOTE:	If the UEs maintain a PC5 unicast link after path switching, the UEs can determine to switch back to the PC5 path from the Uu path based on e.g. path switch policy, the PC5 signal level of the maintained unicast link.

### 6.x.2	Procedure for communication path switching from Uu reference point to PC5 reference point

Figure 6.x.2-1 depicts the procedure for communication path switching from Uu reference point to PC5 reference point.

Figure 6.x.2-1: Procedure for communication path switching from Uu reference point to PC5 reference point

1.	Service authorization and provisioning are performed for the UE as described in clause  6.2 including the Path switching policy.

2.	UE1 and UE2 communicate with each other via Uu path, i.e. by using established PDU Session(s).

3.	UE1 may decide to perform path switching for ProSe service(s) from a Uu path to a PC5 path, e.g. when the Uu signal level is lower than the configured threshold or Uu QoS for a service cannot be satisfied.

UE1 determines whether and which ProSe service(s) can be switched to a PC5 path based on the path switching policy as specified in clause 5.1.3.1.

UE1 triggers the establishment of a PC5 unicast link by using the Layer-2 link establishment procedure as specified in clause 6.4.3.1, with the following differences and clarifications:

-	The Direct Communication Request message also includes the path switching indication from a Uu path to a PC5 path.

-	ProSe Service Info in the Direct Communication Request message is a list of ProSe service(s) which can be switched.

-	The Direct Communication Accept message includes the accepted ProSe service(s) to be switched. UE2 determines whether and which ProSe service(s) can be switched to a PC5 path based on the path switching policy as specified in clause 5.1.3.1.

4.	Traffic for the accepted ProSe service(s) is switched from its Uu path to the PC5 path.

5.	UE1 may release its PDU Session(s), e.g. if no traffic transmitted over the PDU Session.

6.	UE2 may release its PDU Session(s), e.g. if no traffic transmitted over the PDU Session.

# * * * * End of changes * * * *