3GPP TSG-WG SA2 Meeting #154AH	S2-2301026

# Elbonia, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4043 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCF Requests QoS Monitoring for Backhaul |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSATB |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | On the EN related to QoS Monitoring for backhal: “Editor’s Note: In this case, whether PCF can request QoS monitoring for the backhaul only, i.e. without receiving delay values including RAN part delay, is FFS.” According to the descriptions on QoS monitoring in clause 5.33.3.3 of TS 23.501, clause 6.1.3.21 of TS 23.503 and clause 5.24 of TS 29.244, 1) the SMF can request to activate QoS monitoring for the GTP-U path(s) between all UPF(s) and the (R)AN based on locally configured policies. While the SMF doesn’t support to report the QoS monitoring result to the PCF in the current TS. 2) The PCF sends QoS monitoring policy included in PCC rule to SMF, the SMF activates QoS monitoring of this PDU session based on GTP-U path monitoring. The UPF reports the packet delay of the requested QoS flow(s) from UE to PSA to the SMF. Based on the above description, in current specification the PCF cannot request QoS monitoring for backhaul only. This contribution proposes to introduce QoS monitoring for dynamic satellite backhaul to cover the conclusion described in clause 8.1 of TS 23.700-27. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The PCF sends a QoS monitoring policy for dynamic satellite backhaul which only requests packet delay on backhaul. - Based on the QoS monitoring policy for dynamic satellite backhaul, the SMF requests the PSA UPF to only report the packet delay between RAN and PSA UPF. - The PSA UPF calculates the packet delay between RAN and PSA UPF and reports the result to the SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoS monitoring for dynamic satellite backhaul only is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.43.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.43.5	QoS monitoring when dynamic Satellite Backhaul is used

If dynamic satellite backhaul is used, QoS monitoring can be used to obtain packet delay as specified in clause 5.33.3.

Based on the received Satellite backhaul category from SMF, the PCF may trigger QoS monitoring for backhaul if dynamic satellite backhaul is used. QoS monitoring in clause 5.33.3 of TS 23.501 [4] is reused with the following differences:

The PCF sends QoS Monitoring policy which indicates the packet delay for the backhaul is to be monitored. Based on the QoS Monitoring policy included in the PCC rule, the SMF indicates the PSA UPF to only report he packet delay between RAN and PSA UPF. In this case, the PSA UPF calculates packet delay between RAN and the PSA UPF of the QoS Flow and reports the result to the SMF. Then the SMF reports the packet delay to the PCF.

# * * * * End of changes * * * *