3GPP TSG-WG SA2 #154-AH-e 	S2-2301036

Elbonia, Online, January 16 – 20, 2023	(revision of S2-230xxxx)

# Title:	[DRAFT] LS Reply on PDU Set handling

# Response to:	LS (S2-2300053/R2-2213351) on PDU Set handling from RAN2

# Release:	Release 18

# Work Item:	XRM

Source:	Huawei [to be SA2]

To:	RAN2

Cc:	SA4

Contact Person:

Name:	Hui Ni

Tel. Number:

E-mail Address:	hui.ni@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

SA2 would like to thank RAN2 for the LS on PDU Set handling. Regarding the following issue of PDU Set mapping raised by RAN2, SA2 would like to provide the following feedback.

In order to decide how PDU sets could be mapped in radio protocols, RAN2 is wondering if different PDU sets could have different characteristics (for instance importance, PSER, and/or PSDB) and if so, which characteristics can be different and with which granularity (e.g. QoS flow, individual PDU Sets…).

RAN2 would also like to know whether different types of PDU set can be mapped to the same QoS flow and if so whether RAN should have the ability to treat those differently over the air interface. If RAN should have such an ability, RAN2 would like to know based on what information signalled to the gNB this would be based on.

Different PDU Sets can have different characteristics or requirements including e.g. PDU Set importance, PSER and PSDB.

PSER and PSDB: As other QoS parameters of QoS Flow (e.g. PDB and PER), one QoS Flow has only one PSER and PSDB for both UL and DL. For PDU Sets with different PSER and PSDB requirements, they should be transferred via different QoS Flows.

PDU Set Importance: As other PDU Set information sent via GTP-U header, different PDU Sets within a QoS Flow can be marked with different PDU Set importance, which is used to identify the importance of a PDU Set within a QoS flow and RAN may use it for PDU Set level packet discarding in presence of RAN congestion.

Different types of PDU Sets can be mapped to same QoS Flow or different QoS Flows, which depends on whether their QoS requirements are same or not. 5GC can provide RAN with QoS profiles (including e.g. PSER/PSDB) via control plane, and PDU Set information via user plane (including e.g. PDU Set Importance) for QoS handling on RAN.

2. Actions:

To RAN2 group.

ACTION: 	SA2 kindly asks RNA2 group to take the above into account and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #155		Feb 20 – 24, 2023				EU

TSG-SA2 Meeting #156   	Apr 17 – 21, 2023				TBD