3GPP TSG-SA2 Meeting #154AHE	S2-2301045

# Elbonia, Jan 16 - Jan 20

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0672 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE latency performance analytics to assist federated learning operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusion with a NOTE in TR 23.700-80 clause 8.7.2 needs to be addressed. - New latency performance analytics are introduced for NWDAF. NOTE 1: The specific input and output parameters for the latency performance analytics among the group of UEs to assist the Application AI/ML FL operation will be determined during the normative phase. In particular, specific input and output data needs to be defined as per the above agreed conclusion at the last meeting. First, input data with close relationship to latency performance from existing analtyics can be also considered as input data for the new analytics of UE latency performance. In addition, it is recommended to design the analytics to include more UE level Network Data from OAM including network performance as well as RF signal status in that analytics on per UE granularity is required due to the nature that each UE play a role as a training device. Second, information for latency performance per UE derived from traffic for application in use during each time slot is recommended to predict expected latency per UE for assistance of the Application AI/ML FL operation. Output data is desinged in this context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.7.X is updated to specify input and output data for new analytics of UE latency performance. Clause 6.7.1 and 7.1 are updated to include the new analytics for UE latency performance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing features agreed based on conclusions in TR 23.700-80. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1, 6.7.X, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.7.1	General

This clause specifies the UE related analytics which can be provided by NWDAF:

-	UE mobility analytics;

-	UE communication analytics;

-	Expected UE behavioural parameters related network data analytics;

-	Abnormal behaviour related network data analytics;

-	UE latency performance analytics.

The NWDAF service consumer may request for these analytics separately, or in a combined way. As an example, an NWDAF service consumer may learn from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3 of TS 23.502 [3] for a group of UEs or a specific UE, by requesting analytics for both UE mobility (see clause 6.7.2) and for UE communication (see clause 6.7.3).

Depending on local regulations, the NWDAF retrieves user consent for the UE with UDM prior to data collection as defined in clause 6.2.2.2 or clause 6.2.2.3. If user consent to collect data is not granted by the UE, the NWDAF rejects/cancels any analytics subscriptions to any of the UE related analytics with target for analytics set to the SUPI or GPSI of that UE. If the target for analytics is either an Internal or External Group Id or a list of SUPIs or "any UE", the NWDAF skips those SUPIs that do not grant user consent for the purpose of analytics or model training.

NOTE:	Possible uses of such analytics is for the AMF to learn about expected UE behaviour to derive appropriate MICO mode configuration, or for an AF to learn about expected UE behaviour to further provision 5GC with appropriate UE parameters.

NEXT CHANGE

6.7.X	UE latency performance Analytics

6.7.X.1	General

In order to assist the Application AI/ML operation in 5GS, an NWDAF may perform data analytics on UE latency performance and provide the analytics results to an AF.

The service consumer may be an NF (e.g. NEF, AF).

The consumer of these analytics shall indicate in the request or subsription:

-	Analytics ID = "UE latency performance".

-	Target of Analytics Reporting: one or more SUPI(s) or a group of UEs (an Internal Group ID).

-	Analytics Filter Information optionally including:

-	S-NSSAI;

-	DNN;

-	Application ID;

-	Area of Interest.

-	an optional list of analytics subsets that are requested (see clause 6.7.X.3);

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics.

-	Optional preferred order of results for the list of UE latency performance:

-	ordering criterion: "latency performance",

-	order: ascending or descending;

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics subsets (see clause 6.7.X.3) in order to be notified by the NWDAF;

-	Optionally, maximum number of objects;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

6.7.X.2	Input Data

In order to produce “UE latency performance” analytics, the NWDAF collects Performance Data from AF as defined in clauses 6.4.2-1a, Network Data from 5GC NF for Target of Analytics Reporting as defined in clause 6.4.2-2, and/or Performance measurement data from OAM as defined in clause 6.13.2-1.

Additionally, the NWDAF collects UE level Network Data from OAM as specified in Table 6.7.X.2-1 shown below.

Table 6.7.X.2-1: UE level Network Data from OAM

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | OAM | A time stamp associated with the collected information. |
| Reference Signal Received Power | OAM (see NOTE 1) | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM (see NOTE 1) | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM (see NOTE 1) | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |
| Throughput for DL and UL | OAM (see NOTE 1) | The per UE measurement of the throughput for DL and UL as specified in clauses 5.2.1.1 and 5.4.1.1 of TS 37.320 [20] |
| Packet delay for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet delay for DL and UL, including per QCI per UE packet delay as specified in clauses 5.2.1.1 of TS 37.320 [20] and per DRB per UE packet delay as specified in clauses 5.4.1.1 of TS 37.320 [20] |
| Packet loss rate for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet loss rate for DL and UL, including the per QCI per UE packet loss rate as specified in clauses 5.2.1.1 of TS 37.320 [20] and the per DRB per UE packet loss rate as specified in clauses 5.4.1.1 of TS 37.320 [20] |
| NOTE 1: Per UE measurement for a specific UE from OAM (via MDT), is as captured in clause 6.2.3.1. |  |  |

NWDAF subscribes the network data from OAM in the Table 6.4.X.2-1 by using the services provided by OAM as described in clause 6.2.3.

6.7.X.3	Output Analytics

The NWDAF supporting UE latency performance Analytics provides the analytics results to consumer NFs. The analytics results provided by the NWDAF include the UE latency performance statistics as defined in Table 6.7.X.3-1 or predictions as defined in Table 6.7.X.3-2.

Table 6.7.X.3-1: UE latency performance statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. internal group ID defined in clause 5.9.7 of TS 23.501 [2], or a list of UEs for which the statistic applies (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot (average and variance). |
| > Observed UE latency performances (1…max) | List of latency performance per UE |
| >> Application ID | Identifies the application in use during the time slot |
| >> Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| >> Serving anchor UPF info (NOTE 1) | The UPF ID/address/FQDN information for the involved anchor UPF. |
| >> DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| >> Latency Performance | Latency Performance indicators |
| >>> Average Packet Delay (NOTE 2) | Average packet delay observed for UEs communicating with the application. |
| >>> Maximum Packet Delay (NOTE 2) | Maximum packet delay for observed for UEs communicating with the application. |
| >> Spatial Validity Condition | Area where the UE latency performance analytics applies. |
| NOTE 1: The item "Serving anchor UPF info" shall not be included if the consumer NF is an AF. NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". |  |

Table 6.7.X.3-1: UE latency performance predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. internal group ID defined in clause 5.9.7 of TS 23.501 [2], or a list of UEs for which the statistic applies (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot (average and variance). |
| > Observed UE latency performances (1…max) | List of latency performance per UE |
| >> Application ID | Identifies the application in use during the time slot |
| >> Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| >> Serving anchor UPF info (NOTE 1) | The UPF ID/address/FQDN information for the involved anchor UPF. See NOTE 1. |
| >> DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| >> Latency Performance | Latency Performance indicators |
| >>> Average Packet Delay (NOTE 2) | Average packet delay observed for UEs communicating with the application. |
| >>> Maximum Packet Delay (NOTE 2) | Maximum packet delay observed for UEs communicating with the application. |
| >> Spatial Validity Condition | Area where the UE latency performance analytics applies. |
| >> Confidence | Confidence of this prediction |
| NOTE 1: The item "Serving anchor UPF info" shall not be included if the consumer NF is an AF. NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". |  |

NEXT CHANGE

7	Nnwdaf Services Description

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| UE Latency Performance | Analytics ID: UE Latency Performance | Statistics or predictions on UE latency performance |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |

END CHANGE