3GPP TSG-WG SA2 Meeting #154AHE	S2-2301050

# Elbonia, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0674 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for AnLF assisted MTLF in ML model accuracy monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce new procedure for AnLF assisted MTLF in ML model accuracy monitoring, based on the conclusions for eNA_ph3 KI#1 in TR 23.700-81. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause 6.2E.3 that describes AnLF assisted MTLF ML models accuracy monitoring procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality agreed based on conclusions in TR 23.700-81. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.3(NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2E.3	Procedure for AnLF-asisited MTLF ML Models Accuracy Monitoring

This procedure in Figure 6.2E.3-1 is used for NWDAF containing AnLF assisted NWDAF containing MTLF in ML model accuracy monitoring.

Figure 6.1E.2-1: Procedure for AnLF-asisited MTLF ML Models Accuracy Monitoring

1. The NWDAF containing MTLF subscribes to ML model accuracy information by invoking Nnwdaf_ AccuracyProvision_Subscribe service operation to get notifications of the accuracy degradation of the analytics generated by the ML model(s).

2a. The NWDAF containing AnLF subscribes accuracy feedback information that indicates whether the consumer’s actions that may affect the ground truth from the NWDAF analytics consumer by invoking Nnf_EventExposure_Subscribe service operation.

2b. The NWDAF containing AnLF subscribes ground truth data from data source by invoking Nnf_EventExposure_Subscribe service operation.

3. The NWDAF containing AnLF compares predictions and its corresponding ground truth data which is collected in step 2b and considers accuracy feedback information in step 2a, to calculate and generate model accuracy information.

4. The NWDAF containing AnLF provides ML model accuracy information to the NWDAF containing MTLF by invoking by invoking Nnwdaf_ AccuracyProvision_Notify service operation.

5a. The NWDAF containing MTLF collects the input data from data source by invoking Nnf_EventExposure_Subscribe service operation;

5b. The NWDAF containing MTLF collects input data from ADRF by invoking Nadrf_DataManagement_RetrievalSubscribe service operation;

6. The NWDAF containing MTLF reselect a new ML model for the request from NWDAF containing AnLF or retain the existing ML model.

# * * * * End of changes * * * *