3GPP TSG-WG SA2 Meeting #154AHE	S2-2301051

# Elbonia, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0675 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New NWDAF service supporting for AnLF assisted MTLF in accuracy monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce new NWDAF service to supprot AnLF assisted MTLF in accuracy monitoring, based on the conclusions for eNA_ph3 KI#1 in TR 23.700-81. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add general description about Nnwdaf_ AccuracyProvision Servie in clause 7.X.1; Add Nnwdaf_ AccuracyProvision_Subscribe service operation in clause 7.X.2; Add Nnwdaf_ AccuracyProvision_Unsubscribe service operation in clause 7.X.3; Add Nnwdaf_ AccuracyProvision_Notify service operation in clause 7.X.4; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality agreed based on conclusions in TR 23.700-81. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X (NEW), 7.X.1(NEW), 7.X.2(NEW), 7.X.3(NEW), 7.X.4(NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 7.X	Nnwdaf_ AccuracyProvision Service

### 7.X.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe for ML model accuracy information monitored by an NWDAF (i.e. NWDAF containing AnLF).

When the subscription is accepted by the NWDAF containing AnLF, the consumer NF, i.e. the NWDAF containing Model Training Logical Function, receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription. The modification of ML model subscription can be enforced by NWDAF based on operator policy and configuration.

### 7.X.2	Nnwdaf_ AccuracyProvision_Subscribe service operation

Service operation name: Nnwdaf_ AccuracyProvision_Subscribe.

Description: Subscribes to NWDAF for ML model accuracy information with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model subscription), ML Model Identifier to indicate the ML model to be monitored when NWDAF containing MTLF provides multiple ML models, ML Model accuracy information period to indicate the reporting periodicity in which the information can be reported, Accuracy threshold to indicate the reporting condition below which the accuracy information needs to be reported.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

### 7.X.3	Nnwdaf_ AccuracyProvision_Unsubscribe service operation

Service operation name: Nnwdaf_ AccuracyProvision_Unsubscribe.

Description: The NF consumer unsubscribes to the NWDAF for ML Model accuracy information.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.X.4	Nnwdaf_AccuracyProvision_Notify service operation

Service operation name: Nnwdaf_AccuracyProvision_Notify.

Description: NWDAF notifies the ML Model accuracy information to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information, Set of

- the tuple (Analytics ID, ML Model accuracy information), when multiple ML models is not supported.

- the tuple (Analytics ID, ML model identifier, ML Model accuracy information), when the ML model is specified in Nnwdaf_AccuracyProvision_Subscribe service operation.

Inputs, Optional: Validity period.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *