3GPP TSG-WG SA2 Meeting #154-AH-E e-meeting  	S2-2301055

# Elbonia, Jan. 16 – 20, 2022	(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR |  | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of IMS Data Channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Key Issue #1 of R18 FS_NG_RTC "Enhancement to support Data Channel usage in IMS network" has been concluded and the conclusion need to be normative. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a normative Annex to describe IMS data channel architecture according to the conclusion of R18 FS_NG_RTC Key Issue #1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion of R18 FS_NG_RTC Key Issue #1 will not be normative. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 3.3, and Annex XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

3.2	Symbols

For the purposes of the present document the following symbols apply:

Cr	Reference Point between an AS and an MRFC for media control.

Cx	Reference Point between a CSCF and an HSS.

DC1	Reference Point between a DCSF and an IMS AS.

DC2	Reference Point between an IMS AS and a DCMF.

DC3	Reference Point between a DCSF and a NEF.

DC4	Reference Point between a DCSF and a DC Application Server.

DC5	Reference Point between a DCSF and a DCAR.

Dx	Reference Point between an I-CSCF and an SLF.

Gi	Reference point between GPRS and an external packet data network.

Gm	Reference Point between a UE and a P-CSCF or between an IP-PBX and a P-CSCF.

ISC	Reference Point between a CSCF and an Application Server and between a CSCF and an MRB.

Iu	Interface between the RNS and the core network. It is also considered as a reference point.

Ix	Reference Point between IBCF and TrGW.

Ici	Reference Point between an IBCF and another IBCF belonging to a different IM CN subsystem network or between an IBCF and an IP-PBX.

Izi	Reference Point between a TrGW and another TrGW belonging to a different IM CN subsystem network.

Le	Reference Point between an AS and a GMLC.

Ma	Reference Point between an AS and an I-CSCF.

Mb	Reference Point used for IMS media transport to IP network services.

Mf	Reference Point between a transit function and AS.

Mg	Reference Point between an MGCF and a CSCF.

Mi	Reference Point between a CSCF and a BGCF.

Mj	Reference Point between a BGCF and an MGCF.

Mk	Reference Point between a BGCF/IMS ALG and another BGCF.

Mm	Reference Point between a IBCF/CSCF/BGCF/IMS ALG and an IP multimedia network.

Mr	Reference Point between an CSCF and an MRFC.

Mr′	Reference Point between an AS and an MRFC for session control.

Mp	Reference Point between MRFP and MRFC.

Ms	Reference point between an IBCF and Application Server

Mw	Reference Point between a CSCF and another CSCF.

Mx	Reference Point between a CSCF/BGCF and IBCF.

Rc	Reference Point between an AS and an MRB.

Sh	Reference Point between an AS (SIP-AS or OSA-CSCF) and an HSS.

Si	Reference Point between an IM-SSF and an HSS.

Ut	Reference Point between UE and an Application Server.

# * * * * Second change * * * *

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [68] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [68].

5GS	5G System

A2P	Application to Person

API	Application Program Interface

APN	Access Point Name

AS	Application Server

BCSM	Basic Call State Model

BG	Border Gateway

BGCF	Breakout Gateway Control Function

BS	Bearer Service

CAMEL	Customised Application Mobile Enhanced Logic

CAP	Camel Application Part

CDR	Charging Data Record

CN	Core Network

CS	Circuit Switched

CSCF	Call Session Control Function

CSE	CAMEL Service Environment

DCAR	Data Channel Application Repository

DCMF	Data Channel Media Function

DCSF	Data Channel Signalling Function

DHCP	Dynamic Host Configuration Protocol

DNN	Data Network Name

DNS	Domain Name System

ECN	Explicit Congestion Notification

ENUM	E.164 Number Mapping

GGSN	Gateway GPRS Support Node

GLMS	Group and List Management Server

GMLC	Gateway Mobile Location Centre

GRUU	Globally Routable User Agent URI

GUP	Generic User Profile

HSS	Home Subscriber Server

IBCF	Interconnection Border Control Function

I-CSCF	Interrogating-CSCF

IETF	Internet Engineering Task Force

IM	IP Multimedia

IMC	IMS Credentials

IMS	IP Multimedia Core Network Subsystem

IMS ALG	IMS Application Level Gateway

IMSI	International Mobile Subscriber Identifier

IN	Intelligent Network

IP	Internet Protocol

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

IP-CAN	IP-Connectivity Access Network

IP-SM-GW	IP Short Message Gateway

ISDN	Integrated Services Digital Network

ISIM	IMS SIM

ISP	Internet Service Provider

ISUP	ISDN User Part

IWF	Interworking Function

NP	Number portability

MAP	Mobile Application Part

MGCF	Media Gateway Control Function

MGF	Media Gateway Function

MRB	Media Resource Broker

MRFC	Multimedia Resource Function Controller

MRFP	Multimedia Resource Function Processor

NAI	Network Access Identifier

NAPT	Network Address Port Translation

NAT	Network Address Translation

NA(P)T-PT	Network Address (Port-Multiplexing) Translation-Protocol Translation

II-NNI	Inter-IMS Network to Network Interface

OSA	Open Services Architecture

P2A	Person to Application

P2P	Person to Person

P-CSCF	Proxy-CSCF

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCRF	Policy and Charging Rules Function

PDN	Packet Data Network

PDP	Packet Data Protocol e.g., IP

P-GRUU	Public Globally Routable User Agent URI

PLMN	Public Land Mobile Network

PSI	Public Service Identity

PSTN	Public Switched Telephone Network

QoS	Quality of Service

RAB	Radio Access Bearer

RFC	Request for Comments

SCS	Service Capability Server

S-CSCF	Serving-CSCF

SDP	Session Description Protocol

SGSN	Serving GPRS Support Node

SLF	Subscription Locator Function

SNPN	Stand-alone Non-Public Network

SSF	Service Switching Function

SS7	Signalling System 7

SIM	Subscriber Identity Module

SIP	Session Initiation Protocol

S-GW	Signalling Gateway

TAS	Telephony Application Server

T-GRUU	Temporary Globally Routable User Agent URI

THIG	Topology Hiding Inter-network Gateway

TrGW	Transition Gateway

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

URL	Universal Resource Locator

USIM	UMTS SIM

# * * * * Third change * * * *

Annex XX (normative):
Support of IMS Data Channel

XX.1	Overview

XX.1.1	General

TS 26.114 [76] introduced IMS data channel and described media negotiation and configuration of IMS data channel. This Annex specifies the network architecture options, interfaces and procedures for the support of IMS data channel.

XX.1.2	Architecture

Figure XX.1.2-1 and Figure XX.1.2-2 show two architecture options of IMS data channel. In Figure XX.1.2-1, a new Network Function, DCMF, is introduced to provide the media functionalities of IMS data channel. In Figure XX.1.2-2, the existing MRF is enhanced to provide the media functionalities of IMS data channel.

Figure XX.1.2-1: Architecture option of IMS supporting DC usage with DCMF

Figure XX.1.2-2: Architecture option of IMS supporting DC usage with enhanced MRF

XX.1.3	Network Function functional description

XX.1.3.1	DCSF

The Data Channel Signalling Function (DCSF) provides the signalling functionalities of IMS data channel as below:

- Implements data channel control logic through DC1 control API.

- Receives event notifications from the IMS AS and decides the usage of data channel.

- Manages bootstrap data channels and (if applicable) application data channels by interacting with the IMS AS.

- Enables uploading of data channel applications as specified in TS 26.114 [76], however this is out of scope.

- Controls download of IMS data channel applications to the UE.

- Optionally supports HTTP web server to terminate HTTP traffic associated with downloading data channel applications via the DCMF or the enhanced MRF.

- Interacts with the NEF for data channel capability exposure via N33.

- Interacts with the DC Application Server via DC3/DC4 for DC resource control.

XX.1.3.2	DCMF/enhanced MRF

The Data Channel Media Function (DCMF) and the enhanced MRF support the same data channel media functionalities as below:

- Manage the media resources for data channel media (bootstrap data channel and application data channel if applicable) under the control of the IMS AS.

- Terminate bootstrap data channels to/from the UE and forwards HTTP traffic between the UE and the DSCF via MDC1 or relays bootstrap data channel media between the UE and the DCSF via MDC1.

- Anchor application data channels in P2P scenarios, if required, and accordingly supports e.g. data traffic forwarding and possible media interworking between the DCMTSI client in terminal and the MTSI client in terminal.

- Relay A2P/P2A application data channels between the UE and the DC Application Server via either MDC1 or MDC2.

NOTE 1:	The definition of DC Application Server is out of scope of 3GPP.

XX.1.3.3	DCAR

The Data Channel Application Repository (DCAR)

- Supports data channel applications storage which can be internally integrated inside the DCSF or standalone.

NOTE 1:	When the DCAR is separate from the DCSF, the interface DC5 between the DCAR and the DCSF is out of scope of 3GPP.

XX.1.3.4	IMS AS

The IMS AS includes the following functionalities:

- Interworks with the DCSF via DC1 for data channel event notification and data channel control procedures.

- Interacts with the DCMF via DC2 or with the enhanced MRF via Mr'/Cr for data channel media resource management.

NOTE 1:	The IMS AS may be integrated with the DCSF in which case the DC1 interface is internal.

XX.1.4	Reference point to support IMS Data Channel

The following reference points are introduced or enhanced to support data channel usage in IMS network:

- DC1:	Reference point between the DCSF and the IMS AS.

- DC2:	Reference point between the IMS AS and the DCMF.

- DC3:	Reference point between the DCSF and the NEF.

- DC4:	Reference point between the DCSF and the DC Application Server.

- DC5:	Reference point between the DCSF and the DCAR.

NOTE 1:	DC5 is not specified in 3GPP.

The following reference points are introduced or enhanced to support DC media handling:

- Mb:		Existing reference point used for transport of data channel media between the DCMF (or the enhanced MRF) and the IMS-AGW.

- Mr'/Cr:	SIP based reference point between the IMS AS and the enhanced MRF

- MDC1:	Reference point for transport of data channel media between the data channel media function (either the DCMF or the enhanced MRF) and the DCSF.

- MDC2:	Reference point for transport of data channel media between the data channel media function (either the DCMF or the enhanced MRF) and the DC Application Server.

NOTE 2:	MDC1 and MDC2 are not specified in 3GPP.

NOTE 3:	MDC2 may be used between different domains.

XX.1.5	Service based interface to support IMS Data Channel

DC1, DC2, DC3 and DC4 are realized by service-based interfaces shown below:

- Ndcsf:	Service-based interface exhibited by the DCSF.

- Ndcmf:	Service-based interface exhibited by the DCMF.

- Nimsas:	Service-based interface exhibited by the IMS AS.

- Nnef:	Service-based interface exhibited by the NEF.

# * * * * End of changes * * * *