3GPP TSG-SA2 Meeting #154AH	S2-2301071

# 16-20 January 2023, Elbonia               		  									(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4052 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

s

| Title: | Alignment of local switching procedures with the existing 5G VN procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSATB |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G VN group principles were agreed to be re-used as part of the 5GSATB work. The following deviations from the already specified principles would cause unnecessary need to change the existing 5G VN procedures that are specified in TS 23.501 clause 5.8.2.13 (Support of 5G VN group communication): - Inter-UPF interface can be either N19 or N6-based - 5G VN groups are pre-configured - 5G VN group members are identified by the UE identifier, not their address UE-to-UE is a reserved word that is specified for ProSe and the use of an existing term in different context should be avoided. The URSP configuration of DNN/S-NSSAI is a general requirement that is not restricted to UPF on board deployment case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deviations are eliminated by means of referring to the 5G VN specification that is re-used also for local switching via UPF on board satellite. This means pre-configured UE groups as specified in clause 5.29.2 and N6 or N19 link between the UPFs. UE-to-UE is replaced by more general terminology to avoid re-using reserved word for ProSe. General URSP requirement is moved from clause 5.43.3.2 to clause 5.43.3.1. And various minor corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G VN group processing principles cannot be re-used as agreed in the TR 23.700-27 conclusions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.43.2, 5.43.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.43.3	Local switch for communication between two UEs via UPF deployed on GEO satellite

#### 5.43.3.1	General

Traffic between two UEs may be locally routed by UPF(s) deployed on satellite (i.e., through local switch) to the target UE without traversing back to the satellite gateway on the ground.

Local switching via UPF(s) deployed on satellite in this clause only applies on GEO satellite backhaul case and considers only DNNs and slices for 5G VN.

N19 tunnel may be established between two UPFs deployed on different satellites for traffic between two UEs as specified in clause 5.8.2.13. Also N6 may be used for carrying traffic between UPFs deployed on different satellites.

Only a single SMF is supported for local switching and N19 forwarding, i.e. both UEs are served by the same SMF.

NOTE:	The latency optimisation that can be gained by inter-satellite link between UPFs on different GEO satellites depends on the distance between the satellites that can be substantial, depending on the number of deployed satellites.

Clause 5.43.3.2 describes the case of PSA UPF deployed on satellite, clause 5.43.3.3 describes the case of ULCL and local PSA deployed on satellite (PSA UPF is on the ground). Selection of PSA UPF or UL CL/BP/local PSA on satellite is described in clause 6.3.3.

Combination of DNN/S-NSSAI is assigned by the operator for the 5G VN group where backhaul with UPF is deployed on board GEO satellite. URSP can be used to configure UEs to use this DNN and S-NSSAI as described in TS 23.503 [45].

#### 5.43.3.2	Local switch with PSA UPF deployed on satellite

A combination of DNN/S-NSSAI is assigned by the operator to the services deployed on GEO satellite, the URSP configuration is described in TS 23.503 [45].

If SMF selects the UPF deployed on satellite as PSA of UE's PDU Session, the SMF configures the UE's N4 session to forward/detect packet to/from the internal interface as specified for the configuration for the 5GVN group member's N4 Session in clause 5.8.2.13.1 (Support for unicast traffic forwarding of a 5G VN).

Based on pre-configuration, the SMF may establish the N19 tunnel between PSA UPFs and set corresponding group-level N4 session rules utilizing IP address(s) in UPF address pool to forward/detect packet to/from the N19 tunnel interface as specified in clause 5.8.2.13. If N6 is used, SMF configures corresponding N4 rules for processing traffic to/from N6.

For establishing N19 tunnel between the UPFs onboard the satellite, the UPFs are controlled by the same SMF.

-	To process packets between UE and servers residing in DN, SMF configures rules to route traffic via N6 as described in clause 5.8.2.13.1.

The group-level N4 session is per DNN and slice identified by the S-NSSAI. The SMF can create, update or delete the group-level N4 Session, i.e., add or delete N4 rules, allocate or release the N19 tunnel resources based on operator deployment, e.g. based on GEO satellite's planned obsolescence or new GEO satellite setup.

#### 5.43.3.3	Local switching with ULCL/BP and local PSA UPF deployed on satellite

For UPF deployed on satellite acting as ULCL/BP/local PSA case, the PSA UPF anchored by UE PDU sessions is on the ground, if the UEs are served by the same SMF, the SMF determines to activate local switching or forwarding between the UPFs serving the 5G VN group via N19 or N6 for the UEs using the GEO satellite backhaul, based on

AF request including UE identitifiers of UEs which require communication between two UEs as specified in clause 5.29.2.

In that case, if SMF determines that UEs are using the GEO satellites backhaul and UEs are allowed to access the DNAIs corresponding to the GEO satellites, for each UE communicating with target UE(s) in the communication group, the SMF selects the UPF deployed on GEO satellite according to the DNAI as UL-CL/BP and L-PSA, and configures UL-CL/BP with the following rule:

−	Route the data traffic received from the UE and destined to IP address(es) of the target UE(s) to the L-PSA.

−	Route other data traffic received from the UE to the PSA of the UE's PDU Session.

The SMF configures the Local PSA with local forwarding rules to forward the data traffic to the target UEs directly. If the selected L-PSAs are different for the UEs in the communication group, N19 tunnel is established between the L-PSAs. For establishing N19 tunnel between the UPFs onboard the satellite, the UPFs are controlled by the same SMF. If the UEs are members of the same 5G VN group,  the SMF may configure the local data forwarding rules on L-PSA(s)  using 5G VN user plane handing mechanism in clause 5.8.2.13.1 (Support for unicast traffic forwarding of a 5G VN).

Editor's note:	It is for further discussion whether or not the configurations are different and needs additional description, if the UL-CL/BP and L-PSA are combined.

NOTE:	The selected UPF deployed on satellite can be inserted as ULCL/BP/L-PSA reusing existing ULCL/BP insertion procedures defined in TS 23.502 [3] or TS 23.548 [130].