3GPP TSG-SA2 Meeting # 154AHE (e-meeting)	S2-2301135

# Elbonia, January 16th -20th , 2023																(new CR)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0679 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure updates to support FL between NWDAFs containing MTLF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2022-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion for KI#8 agreed in TR 23.700-81, this contribution introduces - the additional factors that need to be considered during NWDAF discovery and selection to support Federated Learning between NWDAFs containing MTLF in clause 5.2. The update in clause 5.2 with respect to NWDAF discovery and selection address the following EN in clause 6.2X. Hence, it is proposed to delete the following EN. Editor’s Note: Procedure of MTLF registration and discovery with respect to FL is FFS. - New Nnwdaf service operation is defined Nnwdaf_MLModel_DistributedTraining request/response between two NWDAFs containing MTLF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When NWDAF containing MTLF supports Federated Learning, NWDAF discovery and selection procedure include FL capability type when registration with the NRF is performed and used during the discovery of NWDAF containing MTLF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for Federated Learning between NWDAF(s) containing MTLF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.2X, 7.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types, the consumer may query NRF providing the NF Set IDs or NF types in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	an NWDAF containing MTLF shall include the ML model provisioning services (i.e. Nnwdaf_MLModelProvision, Nnwdaf_MLModelInfo) as one of the supported services during the registration in NRF when trained ML models are available for one or more Analytics ID(s). The NWDAF containing MTLF may provide to the NRF a (list of) Analytics ID(s) corresponding to the trained ML models and possibly the ML Model Filter Information for the trained ML model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. For each Analytics ID, the NWDAF containing MTLF may also include, in the registration to the NRF, an ML Model Interoperability indicator, FL capability type.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML models requested by the NWDAFs from the vendors in the list.

-	When an NWDAF containing MTLF is capable of supporting Federated Learning, the FL capability type indicates the role an NWDAF containing MTLF assumes either as FL Server or as FL client.

Editor's note:	It is FFS if the ML Model Interoperability indicator is per Analytics ID.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (i.e. an NWDAF containing AnLF) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and NF consumer information. The NRF returns one or more candidate for instances of NWDAF containing MTLF to the NF consumer and each candidate for instance of NWDAF containing MTLF includes the Analytics ID(s) and possibly the ML Model Filter Information for the available trained ML models, if available.

NOTE 5:	NF consumer information such as Vendor ID is defined in stage 3.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

* * * Next Changes * * *

### 6.2C.2	Procedures

#### 6.2C.2.1	General procedure for Federated Learning among Multiple NWDAF containing MTLF

Figure 6.2C.2.1-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF) sends a Nnwdaf_MLModelProvision_Subscribe/Nnwdaf_MLModelInfo request to NWDAF containing MTLF to retrieve a ML model, including Analytic ID, ML model filter information, ML model accuracy information  as described in TS 23.288 [5]. The NWDAF containing MTLF can be a FL server (Server NWDAF) with FL server capability or a NWDAF containing MTLF without FL server capability. The NWDAF discovery and selection procedure as described in clause 5.2 is applicable.

Editor's note:	Definition of FL capability is FFS.

NOTE X: Whether to initiate an FL operation for an ML model by the NWDAF contanining MTLF with FL server capability is based on operator policy and configuration.

1.	Server NWDAF sends Nnwdaf_MLModel_DistributedTraining request (Analytics ID(s), (set of) file address of the initial ML model, ML model reporting time limit) to the selected NWDAF containing MTLF(Client NWDAF) that participates in the Federated learning to perform the local ML model training for Federated Learning.

2.	Each Client NWDAF collects its local data by using the current mechanism as described in clause 6.2.

3.	During Federated Learning training procedure, each Client NWDAF containing MTLF further trains the ML model sent by the server NWDAF containing MTLF in step 1, based on its own data.  Client NWDAF containing MTLF sends Nnwdaf_MLModel_DistributedTraining response (Analytics ID(s), Result Indication, (set of) file address of the local ML model, validity period). The validity period indicates the time over which the local trained ML model is valid.

NOTE Y: The validity period sent from the Client NWDAF containing MTLF to the Server NWDAF containing MTLF may be used by the Server NWDAF to determine whether the Client NWDAF may be selected for the next iteration of the FL during the FL training procedure.

4.	The Server NWDAF containing MTLF aggregates the local ML model  received from NWDAF containing MTLF in step 3, to generate the global ML model for the given Analytics ID in step 0.

5.  The Server NWDAF containing MTLF determines whether to continue the FL training iteration or reselect the list of client NWDAF(s) containing MTLF based on either the ML model accuracy information sent by NF consumer (NWDAF containing AnLF) or based on local configuration in the Server NWDAF.

6.  [Conditional] The Server NWDAF containing MTLF sends Nnwdaf_MLModel_DistributedTraining request (Analytics ID(s), (set of) file address of the aggregated ML model) to each NWDAF containing MTLF.

7.	[Conditional] The Client NWDAF containing MTLF updates the local ML model based on the aggregated ML model sent by the Server NWDAF containing MTLF in step 6.

8a.	Based on the NF consumer (NWDAF containing AnLF) request in step 1, the Server NWDAF containing MTLF updates the training status (an accuracy level) to the consumer periodically (one or multiple rounds of training or every 10 min, etc.) or dynamically when some pre-determined status (e.g. some accuracy level) is achieved.

8b.	[Optional] The NF consumer (NWDAF containing AnLF) decides whether the current model can fulfil the requirement e.g. accuracy and time. The consumer modifies subscription if the current model can fulfil the requirement.

Editor's note:	Further details on providing the accuracy of FL model process in step 8a/8b and associated service operations and procedures is dependent on normative work progress corresponding to KI#1 conclusion of TR 23.700-81.

NOTE:	The steps 2-8b should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

9.   The Server NWDAF containing MTLF sends Nnwdaf_MLModelProvision_Notify/Nnwdaf_MLModelInfo response to the NF consumer (NWDAF containing AnLF) including Analytics ID(s), (set of) file address of the globally optimal ML model.

* * * Next Changes * * *

## 7.X	Nnwdaf_MLModel_DistributedTraining service

### 7.X.1	General

Service description: this service enables the consumer to request and get from NWDAF containing MTLF ML Model Information for distributed training of ML model.

NOTE: In this release of the specification, this service is applicable to Federated Learning between NWDAFs containing MTLF.

### 7.X.2	Nnwdaf_MLModel_DistributedTraining_Request service operation

Service operation name: Nnwdaf_MLModel_DistributedTraining_Request

Description: The consumer requests NWDAF ML Model Information required for distributed ML model training.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2, address (e.g. URL or FQDN) of Model file).

Inputs, Optional: ML Model Filter Information to indicate the conditions for which ML model for the analytics is requested, and Target of ML Model Reporting to indicate the object(s) for which ML model is requested (e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs)), ML Model Reporting Information (including e.g. ML Model Target Period), ML model accuracy information

Outputs, Required: Set of the tuple (Analytics ID, Result Indication, address (e.g. URL or FQDN) of Model file).

Outputs, Optional: Validity period.

* * * End of Changes * * *