3GPP TSG-SA WG2 Meeting #154AHE	S2-2301165

# E-meeting, 16th – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0180 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS session activation and deactivation procedures for UEs in RRC Inactive state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in the conclusion for Key Issue #1 "MBS session reception in RRC Inactive" in TR 23.700-47: - When the MBS session is activated, the UEs in RRC Inactive state in cells, where the MBS session is delivered allowing RRC-inactive reception, should be able to remain in RRC Inactive state for receiving the MBS session data. - For group paging, the network notifies which MBS session is to be activated. For the UE in RRC_INACTIVE state how the group paging is handled will be decided by RAN WGs. - How NG-RAN notifies the UE that the MBS session is re-activated and whether the MBS session is allowed to be received in RRC-inactive state will be decided by RAN WGs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add MBS session activation and deactivation procedures for UEs receiving multicast MBS session data in RRC Inactive state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MBS session activation/deactivation procedures for UEs receiving multicast MBS session data in RRC Inactive state is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.X.Z (new), 7.2.X.Z.1 (new), 7.2.X.Z.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

7.2.X.Z	MBS session activation and deactivation procedures for UEs receiving multicast MBS session data in RRC Inactive state

7.2.X.Z.1	General

When multicast MBS session delivery to UEs in CM-CONNECTED with RRC Inactive state is supported in the network, the multicast MBS session activation and deactivation procedures are performed as specified in clause 7.2.5 with the following enhancements:

-	during MBS session (re-)activation, the group-based paging is executed also for UEs in RRC Inactive state by the NG-RAN.

-	when the multicast MBS session is activated, the UEs in RRC Inactive state should be able to remain in RRC Inactive state for receiving the multicast MBS session data.

7.2.X.Z.2	MBS session activation procedure

The multicast MBS session activation procedure is performed with the following enhancements compared to the procedure defined in clause 7.2.5.2:

-	In step 10a, the NG-RAN configures UE with RRC messages if needed, which may contain the configuration parameters for multicast MBS session reception by the UE in RRC Inactive state.

-	In step 12, upon receiving the RAN paging, the UE in RRC Inactive state may decide to remain in RRC Inactive state for receiving the multicast MBS session data, e.g. based on the configuration parameters received in step 10a.

-	In step 13, the NG-RAN may decide to move some UEs to RRC Inactive state for receiving the multicast MBS session data, based on e.g. the "assistance information for multicast MBS session delivery to UEs in RRC Inactive state" provided by the 5GC.

Editor's note:	How the group paging is performed in NG-RAN to UEs in RRC Inactive state for multicast MBS session activation and how NG-RAN notifies the UEs whether the MBS session is allowed to be received in RRC Inactive state will be decided by RAN WGs. The above procedure needs to be defined aligning with RAN WGs' decision.

* * * End of Changes * * * *