3GPP TSG-SA WG2 Meeting #154AHE	S2-2301167

# E-meeting, 16th – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0681 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service definition for analytics accuracy monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Service provided by the NWDAF for analytics accuracy monitoring needs to be defined to support analytics accuracy monitoring. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add definition of the service provided by the NWDAF for analytics accuracy monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Analytics accuracy monitoring is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.X (new), 7.X.1 (new), 7.X.2 (new), 7.X.3 (new), 7.X.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_AnalyticsAccuracy | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

* * * Next Change * * * *

7.X	Nnwdaf_AnalyticsAccuracy service

7.X.1	General

Service Description: This service enables the consumer (which can be analytics consumer NF, NWDAF containing MTLF, or NWDAF containing AnLF) to receive information about the accuracy of analytics generated by a NWDAF containing AnLF or a NWDAF containing MTLF with accuracy checking capability.

7.X.2	Nnwdaf_AnalyticsAccuracy_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsAccuracy_Subscribe.

Description: Subscribes to notifications on the accuracy of analytics generated using a trained ML model by the NWDAF.

When the subscription is accepted by the NWDAF with accuracy checking capability, the consumer NF receives from the NWDAF with accuracy checking capability an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription. The modification of analytics accuracy subscription can be enforced by NWDAF based on operator policy and configuration.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the analytics accuracy subscription), ML model identifier, Reporting Thresholds.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted: An error response.

Outputs, Optional: None.

7.X.3	Nnwdaf_AnalyticsAccuracy_Unsubscribe service operation

Service operation name: Nnwdaf_AnalyticsAccuracy_Unsubscribe.

Description: unsubscribe to notifications on the accuracy of analytics from the NWDAF.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.X.4	Nnwdaf_AnalyticsAccuracy_Notify service operation

Service operation name: Nnwdaf_AnalyticsAccuracy_Notify.

Description: NWDAF notifies the accuracy of analytics generated using a trained ML model to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information, Set of:

-	the tuple (Analytics ID, the accuracy of analytics), when the consumer is analytics consumer NF or multiple ML models is not supported; or

-	the tuple (Analytics ID, one or more tuples of (ML Model identifier, accuracy level of analytics generated using the ML model, analytics deviation, data set for the detected deviation).

Inputs, Optional: Validity period, Spatial validity.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * End of Changes * * * *