3GPP TSG-SA2 Meeting #154	S2-2301255

# Electronic Meeting, 16‑20 Jan 2023											 (revision of S2-2210920)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0083 | rev | 1 | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Edge Relocation within the same hosting PLMN's EHEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2022-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-48 concluded below related to Edge Relocation in clause 8.5: For edge relocation in GSMA OPG scenario, the EAS reselection methods should be the same as the initial selection. Edge relocation within the same hosting PLMN's EHE is supported, with the principles below: 1) SMF initiates AF/EAS relocation and selection of the target EHE e.g. due to UE mobility; 2) SMF determines AF in the target EHE domain corresponding to the UE target location based on service area or EHE domain supported by AF; 3) notification of User plane management event procedure is enhanced to support AF/EAS change (Sol #23, see clause 6.23). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes changes to TS 23.548 clause 6.3.2 related to Edge Relocation within the same hosting PLMN's EHEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of Edge Relocation within the same hosting PLMN's EHEs as concluded in TR 23.700-48, clause 8.5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 6.3.2	Edge Relocation Involving AF Change

This clause is related to scenarios where distributed Edge Application Server (EAS) deployed in local part of a Data Network or a central AS are relocated, and where the (E)AS relocation also implies AF relocation i.e. AF instance change.

Application Function influence on traffic routing mechanism as described in clause 5.6.7 of TS 23.501 [2] can be applied for a relocation of the AF. In the case that AF sends AF request via NEF, the target AF may invoke Nnef_TrafficInfluence_Create to deliver the relocation related information, including notification target address based on the procedure described in clause 4.3.6.2 of TS 23.502 [3]. Also, the source AF or target AF may invoke Nnef_TrafficInfluence_Update service operation to deliver the relocation information, including AF ID and notification target address based on the procedure described in clause 4.3.6.2 of TS 23.502 [3].

Also if the AF relocation occurs during the early/late notification procedure described in clause 4.3.6.3 of TS 23.502 [3], the target AF invokes Nnef_TrafficIfluence_Create/Update at step 4e-a or Npcf_PolicyAuthorization_Create at step 4g-a to deliver the notification target address of the AF. In the case that AF directly interacts with PCF, the target AF may invoke Npcf_PolicyAuthorization _Create, or the source AF/target AF may invoke Npcf_PolicyAuthorization _Update service operation to deliver relocation information including notification target address based on the procedure described in clause 4.3.6.4 of TS 23.502 [3].

In case of Edge relocation between two EHEs, an AF relocation may be triggered by SMF, e.g., due to UE mobility. In such cases, the SMF provides the target AF ID and the transaction ID for correlation purposes during early/late notification procedure as described in TS 23.502 [3] clause 4.3.6.3. The target AF then may subsequently provide the EAS ID corresponding to the target EHE as part of late notification.

* * * End of Changes * * *