3GPP TSG-WG SA2 Meeting #154-AH-e                                                  S2-2301271

Online, January 16-20, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0690 | rev | 0 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data storage in ADRF using DataSetTag. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update TS23.288 to support the usage of a DataSetTag to store and fetch data to and from ADRF, as concluded for KI#4 in TR23.700-81. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TS23.288, clause 6.2B – introduce DataSetTag in all the ADRF procedures. Clause 10.2 – introduces the DataSetTag in the appropriate service procedures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not updated for Rel. 18 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2, 6.2B.3, 6.2B.4, 10.2.2, 10.2.3, 10.2.4, 10.2.5, 10.2.6, 10.2.9, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 6.2B.2	Historical Data and Analytics storage

The procedure depicted in figure 6.2B.2-1 is used by consumers (e.g. NWDAF, DCCF or MFAF) to store historical data and/or analytics, i.e. data and/or analytics related to past time period that has been obtained by the consumer. After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an ADRF. Whether the consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.

The consumer may include in the storage request the DataSetTag attribute which the data records are to be associated to when stored by ADRF. The DataSetTag attribute is defined in Table 6.2B-1. Data records can be associated to multiple DataSetTag attributes.

Table 6.2B-1: DataSetTag attribute

| Information | Description |
| --- | --- |
| DataSetTag | Identifies the data set. |
| DataSetDescription | Provides human-readable information about the characteristics of the data set. |

Figure 6.2B.2-1: Historical Data and Analytics storage

1.	The consumer sends data and/or analytics to the ADRF by invoking the Nadrf_DataManagement_StorageRequest (collected data with timestamp, analytics with timestamp, Service Operation, Analytics Specification or Data Specification, optionally DataSetTag) service operation.

2.	The ADRF stores the data and/or analytics sent by the consumer. The ADRF may, based on implementation, determine whether the same data and/or analytics is already stored or being stored based on the information sent in step 1 by the consumer NF and, if the data and/or analytics is already stored or being stored in the ADRF, the ADRF decides to not store again the data and/or analytics sent by the consumer. If the DataSetTag attribute is included for data and/or analytics already stored or being stored, then ADRF associates the data records with such DataSetTag.

3.	The ADRF sends Nadrf_DataManagement_StorageRequest Response message to the consumer indicating that data and/or analytics is stored, including when the ADRF may have determined at step 2 that data or analytics is already stored.

# * * * * 2nd change * * * *

### 6.2B.3	Historical Data and Analytics Storage via Notifications

The procedure depicted in figure 6.2B.3-1 is used by consumers (NWDAF, DCCF) to store received notifications in the ADRF. The consumer requests the ADRF to initiate a subscription for data and/or analytics. Data and/or analytics provided in notifications as a result of the subsequent subscription by the ADRF are stored in the ADRF.

The consumer may include in the subscription request the DataSetTag attribute defined in Table 6.2B-1.

Figure 6.2B.3-1: Historical Data and Analytics Storage via Notifications

1a-d.	Based on provisioning or based on reception of a DataManagement subscription request (e.g. see clause 6.2.6.3.2), the DCCF or the NWDAF determines that notifications are to be stored in an ADRF.

2a-b.	The DCCF or the NWDAF determines the ADRF where data and/or analytics needs to be stored and requests that the ADRF subscribes to receive notifications. The determination may be made based on configuration or information supplied by the data consumer as described in clauses 6.1.4 and 6.2.6.3. The request to the ADRF specifies the data and/or analytics to which the ADRF will subscribe by invoking the Nadrf_DataManagement_StorageSubscriptionRequest service operation. The request may include the DataSetTag attribute which the data records are to be associated to when stored by ADRF.

3.	[Optional] The ADRF may, based on implementation, determine whether the same data and/or analytics is already stored or being stored, based on the information sent in step 2 by the consumer. If the DataSetTag attribute is included for data and/or analytics already stored or being stored, then ADRF associates the data records with such DataSetTag.

4.	[Optional] If the data and/or analytics is already stored and/or being stored in the ADRF, the ADRF sends Nadrf_DataManagement_StorageSubscriptionRequest Response message to the consumer indicating that data and/or analytics is stored.

5a-b.	ADRF subscribes to the DCCF or the NWDAF to receive notifications, providing its notification endpoint address and a notification correlation ID.

6.	The DCCF, the MFAF or the NWDAF sends Analytics or Data notifications containing the notification correlation ID provided by the ADRF to ADRF notification endpoint address. The Analytics or Data notifications shall contain timestamp. The ADRF stores the notifications.

7a-b.	The DCCF or the NWDAF determines that notifications no longer need to be stored in the ADRF.

8a-b.	The DCCF or the NWDAF requests that the ADRF unsubscribes to receive notifications.

9a-b.	The ADRF sends a request to the DCCF or the NWDAF to unsubscribe to data notifications.

The NWDAF may interact with the Data Source and the DCCF may interact with the Data Source and/or MFAF. Delivery notifications from the DCCF/MFAF or NWDAF to the ADRF are subsequently halted.

# * * * * 3rd change * * * *

### 6.2B.4	Data removal from an ADRF

The procedure depicted in figure 6.2B.4-1 is used by consumers (DCCF, NWDAF) to remove data previously stored in an ADRF.

Figure 6.2B.4-1: Data Removal from an ADRF

1.	A consumer requests that specified data be deleted from the ADRF using Nadrf_DataManagement_Delete request service operations. The request may include the DataSetTag attribute which the stored data records are associated to.

2.	The ADRF deletes all copies of the stored data.

3.	The ADRF indicates the result (i.e. data deleted, data not found, data found but not deleted) using Nadrf_DataManagement_Delete response service operations.

# * * * * 4th change * * * *

### 10.2.2	Nadrf_DataManagement_StorageRequest service operation

Service operation name: Nadrf_DataManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store data or analytics. Data or analytics are provided to the ADRF in the request message.

Inputs, Required: Data with timestamp or Analytics with timestamp to be stored, Service operation, Analytics Specification or Data Specification.

"Service Operation" identifies the service used to obtain the data or analytics from a Data Source (e.g. Namf_EventExposure_Subscribe or Nnwdaf_AnalyticsSubscription_Subscribe).

"Analytics Specification or Data Specification" is the "Service Operation" specific required and optional input parameters that identify the data that was stored (e.g. Analytics ID(s) / Event ID (s), Target of Analytics Reporting or Target of Event Reporting, Analytics Filter or Event Filter, etc.). Service Operations and input parameters are defined in clause 7 for NWDAF and in clause 5.2 of TS 23.502 [3] for the other NFs.

Inputs, Optional: DataSetTag.

Outputs Required: Result Indication.

Outputs, Optional: Storage Transaction Identifier, DataSetTag(s).

# * * * * 5th change * * * *

### 10.2.3	Nadrf_DataManagement_StorageSubscriptionRequest service operation

Service operation name: Nadrf_DataManagement_StorageSubscriptionRequest

Description: The consumer (NWDAF or DCCF) uses this service operation to request the ADRF to initiate a subscription for data or analytics (see clause 6.2B.3). Data or analytics provided in notifications as a result of the subsequent subscription by the ADRF are stored in the ADRF.

This service operation provides parameters needed by the ADRF to initiate the subscription (to a DCCF or NWDAF).

Inputs, Required: Service operation, Analytics Specification or Data Specification, Target NF (or Set) to subscribe to for notifications.

"Service Operation" identifies the service used to request data or analytics from a Data Source (e.g. Namf_EventExposure_Subscribe or Nnwdaf_AnalyticsSubscription_Subscribe)

"Analytics Specification or Data Specification" is the "Service Operation" specific required and optional input parameters that identify the data to be collected (e.g. Analytics ID(s) / Event ID (s), Target of Analytics Reporting or Target of Event Reporting, Analytics Filter or Event Filter, etc.). Service Operations and input parameters are defined in clause 7 for NWDAF and in clause 5.2 of TS 23.502 [3] for the other NFs.

"Target NF (or Set) to subscribe to for notifications" may be a DCCF or NWDAF that can provide the data or analytics

Inputs, Optional: Formatting Instructions, Processing Instructions, DataSetTag.

Formatting Instructions and Processing Instructions are as defined in clause 5A.4.

Outputs Required: Transaction Reference ID, DataSetTag(s) (if any).

Outputs, Optional: None.

NOTE:	The parameters used in this service operation, including Formatting and Processing Instructions (if provided) are used by the ADRF when subscribing to a DCCF or NWDAF for Data or Analytics to be stored.

# * * * * 6th change * * * *

### 10.2.4	Nadrf_DataManagement_StorageSubscriptionRemoval service operation

Service operation name: Nadrf_DataManagement_StorageSubscriptionRemoval

Description: The consumer NF uses this service operation to request that the ADRF no longer subscribes to data or analytics it is collecting and storing.

Inputs, Required: one of the following:

-	Transaction Reference ID provided in the Nadrf_DataManagement_StorageSubscriptionRequest Output; or

-	DataSetTag.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * 7th change * * * *

### 10.2.5	Nadrf_DataManagement_RetrievalRequest service operation

Service operation name: Nadrf_DataManagement_RetrievalRequest

Description: The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF. The Nadrf_DataManagement_RetrievalRequest response either contains the data or analytics, or provides instructions for fetching the data or analytics. The Nadrf_DataManagement_RetrievalRequest may be unsolicited (e.g. when the consumer itself has known "Storage Transaction Identifier") or sent in response to a Fetch Instructions received from the ADRF in an Nadrf_DataManagement_RetrievalNotify.

Inputs, Required: one of the following:

-	Storage Transaction Identifier; or

-	Fetch Correlation ID(s) if the RetrievalRequest is in response to a Fetch Instruction received from the ADRF in an Nadrf_DataManagement_RetrievalNotify; or

-	DataSetTag.

Inputs, Optional: None.

Outputs Required: Result Indication.

Outputs, Optional: Data or Analytics.

# * * * * 8th change * * * *

### 10.2.6	Nadrf_DataManagement_RetrievalSubscribe service operation

Service operation name: Nadrf_DataManagement_RetrievalSubscribe

Description: The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF and to receive future notifications containing the corresponding data or analytics received by ADRF.

Inputs, Required: one of the following:

-	Service Operation, Analytics Specification or Data Specification, Time Window; or

-	DataSetTag.

"Service Operation" identifies the service used to obtain the data or analytics from a Data Source (e.g. Namf_EventExposure_Subscribe or Nnwdaf_AnalyticsSubscription_Subscribe).

"Analytics Specification or Data Specification" is the "Service Operation" specific required and optional input parameters that identify the data that was stored (e.g. Analytics ID(s) / Event ID (s), Target of Analytics Reporting or Target of Event Reporting, Analytics Filter or Event Filter, etc.). Service Operations and input parameters are defined in clause 7 for NWDAF and in clause 5.2 of TS 23.502 [3] for the other NFs.

"Time Window" is the start and stop time when the requested data or analytics was collected. If Time Window includes a period in the future, subsequent notifications containing the requested data or analytics received by the ADRF are sent to the notification endpoint.

"DataSetTag" indicates all data records stored or being collected and stored by ADRF which are associated to that attribute.

Inputs, Optional: Fetch flag.

Outputs Required: Result Indication.

Outputs, Optional: Subscription Correlation ID.

# * * * * 9th change * * * *

### 10.2.9	Nadrf_DataManagement_Delete

Service operation name: Nadrf_DataManagement_Delete

Description: This service operation instructs the ADRF to delete stored data.

Inputs, Required: One of the following:

-	Storage Transaction Identifier;

-	Service Operation, Analytics Specification or Data Specification and Time Window; or

-	DataSetTag.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of change * * * *