3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2301298

# Elbonia, January 16 – 20, 2023		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0892 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of MBSR Policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBSR node may be configured using existing UE Policy mechanism, with the necessary parameters for its operation. The MBSR Policy is defined under the UE Policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the definition of the MBSR Policy under UE Policy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration of the MBSR node using UE Policy mechanism is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2; (new) 6.6.5; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.2.2	UE policy control requirements

The 5GC shall be able to provide policy information from the PCF to the UE. Such UE policy information includes:

-	Access Network Discovery & Selection Policy (ANDSP): It is used by the UE for selecting non-3GPP accesses network.

-	UE Route Selection Policy (URSP): This policy is used by the UE to determine how to route outgoing traffic. Traffic can be routed to an established PDU Session, can be offloaded to non-3GPP access outside a PDU Session, can be routed via a ProSe Layer-3 UE-to-Network Relay outside a PDU session, or can trigger the establishment of a new PDU Session.

-	V2X Policy (V2XP): This policy provides configuration parameters to the UE for V2X communication over PC5 reference point or over Uu reference point or both. V2X Policies are defined in TS 23.287 [28].

-	ProSe Policy (ProSeP): This policy provides configuration parameters to the UE for ProSe Direct Discovery, ProSe Direct Communication, ProSe UE-to-Network Relay and Remote UE. ProSe Policies are defined in TS 23.304 [34].

-	MBSR Policy (MBSRP): This policy provides configuration parameters to the Mobile Base Station Relay (MBSR) node regarding its access to the serving PLMN, and the OAM servers. MBSR is defined in TS 23.501 [2] clasue 5.35A.

# * * * * Next change * * * *

### 6.6.5	MBSR Policy information

The MBSR Policy information (MBSRP) includes the following information:

-	PDU Session Parameter Information, with corresponding PLMN IDs; and

-	List of Preferred PLMN IDs, and Forbidden PLMN IDs for PLMN selection of MBSR.

The PDU Session Parameter Information reuses the Route Selection Descriptor format as defined in clasue 6.6.2.1. For a specific seving PLMN, the MBSR shall identify the PDU Session Parameter Information corresponds to the PLMN ID to establish the PDU session for accessing the OAM server.

The List of Preferred PLMN IDs and Forbiddent PLMN IDs indicates the PLMNs the MBSR is allowed or forbidden to operate as MBSR. The PLMN selection procedures for MBSR are defined in TS 23.122 [xx].

Editor's note:	further information for the MBSRP will be decided based on RAN WG inputs.

# * * * * End of changes * * * *