3GPP TSG-SA2 Meeting #154 AH-e	S2-2301369

# EMEETING, 16th – 20th January, 2023										   revision of S2-2300421

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3897 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN communication configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in TR 23.700-88, PIN traffic QoS and routing differentiation need to be supported by 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe PIN communication configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PIN is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 5.X.c.1, new 5.X.c.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Separate PDU Session Establishment and PDU Session Modification into different sub-clauses |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

5.X	Support for Personal IoT Networks

5.X.c	Session management enhancement for PIN service support

5.X.c.1	PDU Session Establishment for PIN

When PDU Sessions associated with a PIN are established by PEGCs, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The PEGC UE may use IPv6 Prefix Delegation feature as described in clause 5.8.2.2, or DHCP proxy for IP address allocation of PINEs. If IPv4 is used for allocating IP addresses for PINEs, the PEGC may act as DHCP proxy between the PINE and the SMF/UPF over User Plane.

One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN. One PIN may be served by more than one PDU sessions in the PEGC.

Editor's note:	The handling of PEMC in 5GC in relation with PIN is FFS.

5.X.c.2	Session management related policy control

For PIN traffic via PEGC/5GC, the 5GC supports the policy control. For PIN traffic of one PIN via 5GC, one or more PDU session with user plane is established by the PEGC UE. The policy control is based on session management procedures as specified in TS 23.502 [3] and TS 23.503 [45].

A PEGC or an AF may request traffic QoS and traffic routing for a PIN. The PIN traffic can be categorised into following types:

-	Between two PINEs, which is via 5G core network when the two PINEs connect to different PEGCs;

-	Between PINE and PEMC via a PEGC and 5G core network;

-	Between PINE and external DN via a PEGC and 5G core network;

-	Between PEGC and external DN via 5G core network.

Editor's note:	The handling of PIN deletion/deactivation/activation is FFS.

# * * * * End of change * * * *