3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2301448

# Elbonia, January 16 – 20, 2023		(revision of S2-2300966r04)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3822 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Triggered Connection Resume in RRC Inactive procedure alignment with RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ARCH_NR_REDCAP, TEI18 |  |  |  |  |  | Date: |  |  | 2023-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE Triggered Connection Resume in RRC Inactive procedure is not aligned with the corresponding procedure as defined in TS 38.300, clause 9.2.2.4.1, where the RRC procedure is executed before the N2 procedure. Current call flow in TS 23.502 allows them to be executed in parallel which is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align with TS 38.300, clause 9.2.2.4.1 for UE Triggered Connection Resume in RRC Inactive procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrrect order of execution from NG-RAN of RRC procedure towards UE and N2 procedure towards 5GC may lead to RRC state mismatch issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.8.2.2	UE Triggered Connection Resume in RRC Inactive procedure

The Connection Resume procedure is used by the UE to perform RRC Inactive to RRC Connected state transition. Triggers for the UE to initiate this procedure are defined in clause 5.3.3.2.5 of TS 23.501 [2].

Figure 4.8.2.2-1: Connection Resume in RRC Inactive

1.	UE to NG-RAN: RRC message (Resume ID).

The UE initiates the transition from RRC Inactive state to RRC Connected state, see TS 38.300 [9]. The UE provides its Resume ID needed by the NG-RAN to access the UE's stored Context.

2.	[Conditional] NG-RAN performs UE Context Retrieval.

UE Context Retrieval is performed when the UE Context associated with the UE attempting to resume its connection is not locally available at the accessed NG-RAN. The UE Context Retrieval procedure via NG-RAN is specified in TS 38.300 [9].

3.	NG-RAN to UE: RRC messages.

The NG-RAN confirms to the UE that the UE has entered RRC_CONNECTED state and the RRC connection is resumed, see clause 9.2.2.4.1 of TS 38.300 [9].

4a.	[Conditional] N2 Path switch procedure.

If the accessed NG-RAN is able to retrieve the UE Context, the accessed NG-RAN node initiates N2 Path Switch procedure, i.e. steps 1 to 8 of clause 4.9.1.2.2 and including Xn data forwarding.

If the Connection Resume procedure is a response to RAN paging which is triggered by 5GC due to an N2 interface procedure, NG-RAN and 5GC handle the N2 interface procedure as a collision described in clause 4.9.1.2.

4b.	[Conditional] N2 Notification,

4b.1	If the accessed NG-RAN is the same as the NG-RAN that configured RRC Inactive and still has the UE context, NG-RAN send an N2 Notification to the AMF indicating the UE is in RRC_CONNECTED, if an AMF requested N2 Notification (see clause 4.8.3) or Connection Inactive procedure with CN based MT communication handling (see clause 4.8.1.1a) has been performed previously.

4b.2	The AMF invokes Nsmf_PDUSession_UpdateSMContext Request towards SMF indicating the Downlink data delivery for each PDU session with active user plane, if the AMF has requested data buffering as described in clause 4.8.1.1a.

4b.3	N4 session modification procedure is triggered by the SMF. If data buffering is handled in the UPF, the SMF updates the UPF with appropriate rules to trigger data delivery.

4b.4	The SMF sends the Nsmf_PDUSession_UpdateSMContext response.

4b.5	The AMF sends the N2 response to NG-RAN.

Editor's note:	Whether the N2 message to be used is a new message or an existing message, e.g. N2 notification, needs coordination with RAN WGs considering also if it's NGAP class 1 or class 2 procedure.

# * * * * End of changes * * * *