3GPP TSG-SA WG2 Meeting #154AH-E	S2-2301584

# Elbonia, January 16 - 20, 2023	(revision of S2-2300468r15)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3910 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Assistance to Member Selection Functionality for Application Operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, OPPO, Toyota, Nokia, Nokia Shanghai Bell, Samsung, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution address the following aspect of the conclusion agreed in TR 23.700-80 to assist federated learning member selection: - The new member selection assistance functionality will be specified in the normative phase. The new member selection assistance functionality is defined as a functionality offered by the 5GC upon receiving a request from the AF to provide a list of candidate UEs and additional information. - The new member selection assistance functionality can be used by the AI/ML based application to select FL members. - The new member selection assistance functionality can provide a list of recommended UEs based on the parameters contained in the request from the AF. - The AF includes the following parameters as part of the request service operation to the 5GC: - UE list (a UE or a UE list). - The 5GC provides the following parameters as part of the response service operation to the AF: - List(s) of candidate UEs which can be selected by the AF. NOTE 1: Any further parameters to be contained in the request/response from/to the AF and the procedure for selecting the list of candidate UEs will be discussed in the normative. - The new member selection assistance functionality will be hosted in NEF. NOTE 2: If this new member selection assistance functionality is not used, AF can select the AI/ML members based on existing network exposure information as described in clause 4.15 of TS 23.502 [4]. NOTE 3: How to support the trusted AF with member selection assistance functionality will be addressed during the normative phase. So it’s necessary to update the related TS to specify the enhancements included in the conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce new member selection assistance functionality - Update NEF functionality for member selection assistance functionality - Update NEF Services for member selection assistance functionality |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC can not support member selection assistance functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y (new), 5.20, 6.2.5.0, 7.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.502 CR 3674 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 5.2.6.X and 4.15.X references are the clause in the companion 23.502 CR. (Note that this CR was postponed at SA2#154AH-E, so will be discussed at SA2#155) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.X.Y	UE member selection assistance functionality for application operation

5G system may support UE member selection assistance functionality to assist the AF in selecting member UE(s) that can be used in application operations such as AI/ML based applications (e.g., Federated Learning). The UE member selection assistance functionality shall be hosted by NEF, and the features of the UE member selection assistance functionality hosted by NEF include (see clause 4.15.X in TS 23.502 [3] for details of UE member selection procedures):

- Receiving a request from the AF that shall include a list of target member UE(s), optionally (a) time window(s), and one or more filtering criteria as specified in Table 5.2.6.X.1-1 of TS 23.502 [3] (e.g., number of UEs, UE location, trajectory and direction, QoS requirements, Area of Interest, DNN, S-NSSAI, expected application operation time duration, preferred access/RAT type, Desired UE transmission latency performance).

NOTE 1:	At least one of the UE member filtering criteria is required.

Editor's note:	Which parameters are mandatory in the AF request is FFS.

- Coordinatings with other relevant 5GC NFs (e.g., PCF, NWDAF and AMF) to collect the necessary information of target member UE(s) to derive (a) list(s) of candidate UE(s). Event filtering criteria are used by NEF to collect data from other 5GC NF and derive the list of candidate UEs (i.e., UEs among the list of provided target member UE(s)). Other additional information may be provided by NEF (e.g., recommended time window(s) for performing the application operation QoS of each target UE, UE(s) location, Access/RAT type).

- Providing the AF with the UE member selection assistance information.

The UE member selection assistance information provided by the NEF can be used by the AF to select member UEs.

Editor's note:	Any further parameters to be contained in the service operations from/to the AF are FFS.

NOTE 2:	AF can choose whether to use the UE member selection assistance functionality provided by NEF. AF in either trusted or untrusted domain can select the UE members based on existing network exposure information as described in clause 4.15 of TS 23.502 [3].

* * * * Start of 2nd Change * * * *

6.2.5	NEF

6.2.5.0	NEF functionality

The Network Exposure Function (NEF) supports the following independent functionality:

-	Exposure of capabilities and events:

NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing as described in clause 5.13.

NEF stores/retrieves information as structured data using a standardized interface (Nudr) to the Unified Data Repository (UDR).

-	Secure provision of information from external application to 3GPP network:

It provides a means for the Application Functions to securely provide information to 3GPP network, e.g. Expected UE Behaviour, 5G-VN group information, time synchronization service information and service specific information. In that case the NEF may authenticate and authorize and assist in throttling the Application Functions.

-	Translation of internal-external information:

It translates between information exchanged with the AF and information exchanged with the internal network function. For example, it translates between an AF-Service-Identifier and internal 5G Core information such as DNN, S-NSSAI, as described in clause 5.6.7.

In particular, NEF handles masking of network and user sensitive information to external AF's according to the network policy.

-	Redirecting the AF to a more suitable NEF/L-NEF e.g. when serving an AF request for local information exposure and detecting there is a more appropriate NEF instance to serve the AF's request.

-	The Network Exposure Function receives information from other network functions (based on exposed capabilities of other network functions). NEF stores the received information as structured data using a standardized interface to a Unified Data Repository (UDR). The stored information can be accessed and "re-exposed" by the NEF to other network functions and Application Functions, and used for other purposes such as analytics.

-	A NEF may also support a PFD Function: The PFD Function in the NEF may store and retrieve PFD(s) in the UDR and shall provide PFD(s) to the SMF on the request of SMF (pull mode) or on the request of PFD management from NEF (push mode), as described in TS 23.503 [45].

-	A NEF may also support a 5G-VN Group Management Function: The 5G-VN Group Management Function in the NEF may store the 5G-VN group information in the UDR via UDM as described in TS 23.502 [3].

-	Exposure of analytics:

NWDAF analytics may be securely exposed by NEF for external party, as specified in TS 23.288 [86].

-	Retrieval of data from external party by NWDAF:

Data provided by the external party may be collected by NWDAF via NEF for analytics generation purpose. NEF handles and forwards requests and notifications between NWDAF and AF, as specified in TS 23.288 [86].

-	Support of Non-IP Data Delivery:

NEF provides a means for management of NIDD configuration and delivery of MO/MT unstructured data by exposing the NIDD APIs as described in TS 23.502 [3] on the N33/Nnef reference point. See clause 5.31.5.

-	Charging data collection and support of charging interfaces.

-	Support of UE member selection assistance functionality:

-		NEF may provide list(s) of candidate UEs and additional information to the AF based on the parameters contained in the request from the AF as described in clause 5.X.Y.

-	Support of UAS NF functionality:

Details are defined in TS 23.256 [136].

-	Support of EAS deployment functionality:

Details are defined in TS 23.548 [130].

-	Support of SBI-based MO SM transmit for MSISDN-less MO SMS:

Details are defined in TS 23.540 [142].

A specific NEF instance may support one or more of the functionalities described above and consequently an individual NEF may support a subset of the APIs specified for capability exposure.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

The services provided by the NEF are specified in clause 7.2.8.

For external exposure of services related to specific UE(s), the NEF resides in the HPLMN. Depending on operator agreements, the NEF in the HPLMN may have interface(s) with NF(s) in the VPLMN.

When a UE is capable of switching between EPC and 5GC, an SCEF+NEF is used for service exposure. See clause 5.17.5 for a description of the SCEF+NEF.

* * * * Start of 3rd Change * * * *

7.2.8	NEF Services

The following NF services are specified for NEF:

Table 7.2.8-1: NF Services provided by NEF

| Service Name | Description | Reference in TS 23.502 [3] or other TS |
| --- | --- | --- |
| Nnef_EventExposure | Provides support for event exposure. | 5.2.6.2 |
| Nnef_PFDManagement | Provides support for PFDs management. | 5.2.6.3 |
| Nnef_ParameterProvision | Provides support to provision information which can be used for the UE in 5GS. | 5.2.6.4 |
| Nnef_Trigger | Provides support for device triggering. | 5.2.6.5 |
| Nnef_BDTPNegotiation | Provides support for background data transfer policy negotiation and optionally notification for the renegotiation. | 5.2.6.6 |
| Nnef_TrafficInfluence | Provide the ability to influence traffic routing. | 5.2.6.7 |
| Nnef_ChargeableParty | Requests to become the chargeable party for a data session for a UE. | 5.2.6.8 |
| Nnef_AFsessionWithQoS | Requests the network to provide a specific QoS for an AS session. | 5.2.6.9 |
| Nnef_MSISDN-less_MO_SMS | Used by the NEF to send MSISDN-less MO SM to the AF. | 5.2.6.10 |
| Nnef_ServiceParameter | Provides support to provision service specific information. | 5.2.6.11 |
| Nnef_APISupportCapability | Provides support for awareness on availability or expected level of a service API. | 5.2.6.12 |
| Nnef_NIDDConfiguration | Used for configuring necessary information for data delivery via the NIDD API. | 5.2.6.13 |
| Nnef_NIDD | Used for NEF anchored MO and MT unstructured data transport. | 5.2.6.14 |
| Nnef_SMContext | Provides the capability to create, update or release the SMF-NEF Connection. | 5.2.6.15 |
| Nnef_AnalyticsExposure | Provides support for exposure of network analytics. | 5.2.6.16 |
| Nnef_UCMFProvisioning | Provides the ability to configure the UCMF with dictionary entries consisting of UE manufacturer-assigned UE Radio Capability IDs, the corresponding UE radio capabilities, the corresponding UE Radio Capability for Paging and the (list of) associated IMEI/TAC value(s) via the NEF. The UE radio capabilities the NEF provides for a UE radio Capability ID can be in TS 36.331 [51] format, TS 38.331 [28] format or both formats. Also used for deletion (e.g. as no longer used) or update (e.g. to add or remove a (list of) IMEI/TAC value(s) associated to an entry) of dictionary entries in the UCMF. | 5.2.6.17 |
| Nnef_ECRestriction | Provides support for queuing status of enhanced coverage restriction, or enable/disable enhanced coverage restriction per individual UEs. | 5.2.6.18 |
| Nnef_ApplyPolicy | Provides the capability to apply a previously negotiated Background Data Transfer Policy to a UE or a group of UEs. | 5.2.6.19 |
| Nnef_Location | Provides the capability to deliver UE location to AF. | 5.2.6.21 |
| Nnef_AMInfluence | Provides the ability to influence access and mobility management related policies for one or multiple UEs. | 5.2.6.22 |
| Nnef_AMPolicyAuthorization | Provides the ability to provide inputs that can be used by the PCF for deciding access and mobility management related policies. | 5.2.6.23 |
| Nnef_AKMA | AKMA Application Key derivation service. | TS 33.535 [124] |
| Nnef_Authentication | This service enables the consumer to authenticate and authorize the Service Level Device Identity as described in TS 23.256 [136]. | TS 23.256 [136] |
| Nnef_TimeSynchronization | Provides the ability to support for (g)PTP or 5G access stratum based time synchronization service. | 5.2.6.25 |
| Nnef_EASDeployment | EAS deployment service. | 5.2.6.26 |
| Nnef_UEId | UE Identifier service, which supports to retrieve AF specific UE Identifier based on UE address. | 5.2.6.27 |
| Nnef_MBSTMGI | Allows AF to request allocation/deallocation of TMGI(s) for MBS Session. | TS 23.247 [129] |
| Nnef_MBSSession | Allows AF to create, update and delete MBS Session. | TS 23.247 [129] |
| Nnef_ASTI | Provides the ability to influence 5G access stratum based time distribution configuration. | 5.2.6.28 |
| Nnef_SMService | Used for SBI-based MO SM transmit through NEF for MSISDN-less MO SMS. | 5.2.6.29 |
| Nnef_PDTQPolicyNegotiation | Provides support for negotiation for Planned Data Transfer with QoS requirements policy and optionally notification for the renegotiation. | 5.2.6.30 |
| Nnef_UEMemberSelectionAssistance | Provides list(s) of candidate UE(s) and additional information based on the parameters contained in the request from the AF | 5.2.6.X |

* * * * Start of 4th Change * * * *

5.20	External Exposure of Network Capability

The Network Exposure Function (NEF) supports external exposure of capabilities of network functions. External exposure can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability, Analytics reporting capability and UE Member selection capability. The Monitoring capability is for monitoring of specific event for UE in 5G System and making such monitoring events information available for external exposure via the NEF. The Provisioning capability is for allowing external party to provision of information which can be used for the UE in 5G System. The Policy/Charging capability is for handling access and mobility management, QoS and charging policies for the UE based on the request from external party. The Analytics reporting capability is for allowing an external party to fetch or subscribe/unsubscribe to analytics information generated by 5G System (this is further defined in TS 23.288 [86]). The UE Member selection capability is for allowing an external party to acquire list(s) of candidate UE(s) and additional information that is based on the assistance information generated by 5G System based on some defined filtering criteria, the details are explained in clause 4.15.X in TS 23.502 [3].

Monitoring capability is comprised of means that allow the identification of the 5G network function suitable for configuring the specific monitoring events, detect the monitoring event, and report the monitoring event to the authorised external party. Monitoring capability can be used for exposing UE's mobility management context such as UE location, reachability, roaming status, and loss of connectivity. AMF stores URRP-AMF information in the MM context to determine the NFs that are authorised to receive direct notifications from the AMF. UDM stores URRP-AMF information locally to determine authorised monitoring requests when forwarding indirect notifications. The Monitoring capability also allows AF to subscribe to the group status changes for a group, either a 5G VN group as described in clause 5.29.2, as well as a group configured by OA&M. In this case the AF is notified if the group member list is updated or a group member is no longer subscribed to the group.

Provisioning capability allows an external party to provision the Expected UE Behaviour or the 5G-VN group information or ECS Address Configuration Information or service specific information to 5G NF via the NEF. The provisioning comprises of the authorisation of the provisioning external third party, receiving the provisioned external information via the NEF, storing the information, and distributing that information among those NFs that use it. The externally provisioned data can be consumed by different NFs, depending on the data. In the case of provisioning the Expected UE Behaviour, the externally provisioned information which is defined as the Expected UE Behaviour parameters in clause 4.15.6.3 of TS 23.502 [3] or Network Control parameter in clause 4.15.6.3a of TS 23.502 [3] consists of information on expected UE movement, Expected UE Behaviour parameters or expected Network Configuration parameter. The provisioned Expected UE Behaviour parameters may be used for the setting of mobility management or session management parameters of the UE. In the case of provisioning the 5G-VN group information the externally provisioned information is defined as the 5G-VN group parameters in clause 4.15.6.7 of TS 23.502 [3] and it consists of some information on the 5G-VN group. In the case of provisioning ECS address, the externally provisioned information is defined as the ECS Address Configuration Information in clause 4.15.6.3d of TS 23.502 [3]. The affected NFs are informed via the subscriber data update as specified in clause 4.15.6.2 of TS 23.502 [3]. The externally provisioned information which is defined as the Service Parameters in clause 4.15.6.7 of TS 23.502 [3] consists of service specific information used for supporting the specific service in 5G system. The provisioned Service Parameters may be delivered to the UEs. The affected NFs are informed of the data update.

Policy/Charging capability is comprised of means that allow the request for session and charging policy, enforce QoS policy, apply accounting functionality and requests to influence access and mobility management policies. It can be used for specific QoS/priority handling for the session of the UE, and for setting applicable charging party or charging rate.

Analytics reporting capability is comprised of means that allow discovery of type of analytics that can be consumed by external party, the request for consumption of analytics information generated by NWDAF.

UE Member selection capability is comprised of means that allows filtering and providing UE list(s) and additional information that can be consumed by external party, the request for consumption of UE list generated by external party.

An NEF may support CAPIF functions for external exposure as specified in clause 6.2.5.1.

An NEF may support exposure of NWDAF analytics as specified in TS 23.288 [86].

The NEF may support exposure of 5GS and/or UE availability and capabilities for time synchronization service as specified in clause 5.27.1.8.

An NEF may support exposure of event based notifications and reports for NSACF as specified in clause 5.15.11.

An AF may only be able to identify the target UE of an AF request for external exposure of 5GC capabilities (e.g. Data Provisioning or for Event Exposure for a specific UE) by providing the UE's address information. In this case the NEF first needs to retrieve the Permanent identifier of the UE before trying to fulfil the AF request. The NEF may determine the Permanent identifier of the UE, as described in clause 4.15.3.2.13 of TS 23.502 [3], based on:

-	the address of the UE as provided by the AF; this may be an IP address or a MAC address;

-	the corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or determined by the NEF based on the requesting AF; this is needed if the UE address is an IP address.

The NEF may provide an AF specific UE Identifier to the AF:

-	that has explicitly requested a translation from the address of the UE to a unique UE identifier (via Nnef_UEId service); or

-	that has implicitly requested a translation from the address of the UE to a AF specific UE Identifier by requesting external exposure about an individual UE identified by its address.

The AF may have its own means to maintain the AF specific UE Identifier through, e.g. an AF session. After the retrieval of an AF specific UE Identifier the AF shall not keep maintaining a mapping between this identifier and the UE IP address as this mapping may change.

The AF specific UE Identifier shall not correspond to a MSISDN; it is represented as a GPSI in the form of an External Identifier. When used as an AF specific UE identifier, the External Identifier provided by the 5GCN shall be different for different AF.

NOTE 1:	This is to protect user privacy.

NOTE 2:	How to enforce that the AF specific UE Identifier is different for different AFs is defined in TS 33.501 [29].

* * * * End of Changes * * * *