3GPP TSG-SA2 Meeting #154-AH-e	S2-2301988

# Electronic Meeting, January 16 - 20, 2023								(revision of S2-2300454r02)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3727 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PFD Determination by NEF (PFDF) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, China Mobile, Samsung, SK Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NWDAF-assisted PFD determination was implemented based on CR0584. The CR is to add the procedure for PFD Determination by NEF (PFDF) as concluded in TR 23.700-81 KI #2 NWDAF-assisted application detection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to: - add the procedure for PFD Determination by NEF (PFDF). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for PFD Determination by NEF (PFDF) will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** Start of Changes ***

4.18.2	PFD management via NEF (PFDF)

4.18.2.1	PFD management triggered by AF

Figure 4.18.2.1-1 procedure for PFD management via NEF (PFDF) triggered by AF

1.	The AF invokes the Nnef_PFDManagement_Create/Update/Delete service. The Allowed Delay is an optional parameter. If the Allowed Delay is included, it indicates that the list of PFDs in this request should be provisioned within the time interval indicated by the Allowed Delay to the SMF(s) that have subscribed to the PFD management service using Nnef_PFDManagement_Subscribe service operation.

2.	NEF checks whether the AF is authorized to perform this request and NEF (PFDF) checks if the AF is authorised to provision this PFD data based on the operator policies. Besides the information from the AF, the NEF may subscribe to the NWDAF to receive application detection analytics defined in clause 6.16.4 in TS 23.288 [50].

3.	The NEF (PFDF) invokes the Nudr_DM_Create/Update/Delete (Application Identifier, one or more sets of PFDs, Allowed Delay) to the UDR.

4.	The UDR updates the list of PFDs for the Application Identifier.

5.	The UDR sends a Nudr_DM_Create/Update/Delete Response to the NEF (PFDF).

6.	The NEF sends Nnef_PFDManagement_Create/Update/Delete Response to the Application Function.

4.18.2.2	PFD management based on PFD Determination analytics

Figure 4.18.2.2-1 shows the procedure that, based on PFD information subscribed from the NWDAF. The procedure enables the NEF(PFDF) to determine whether to create/update/delete PFD information corresponding to the Application Identifier.

Figure 4.18.2.2-1 Procedure for PFD management based on PFD Determination analytics

1.	The NWDAF notifies PFD determination analytics to the NEF(PFDF) as Consumer NF with PFD Information defined in clause 6.16.4 in TS 23.288 [50].

2.	The NEF (PFDF) fetches the PFD information currently in use from UDR as described from step 2 to step 3 of clause 4.18.3.1.

3.	The NEF (PFDF) compares the PDF information from UDR with PFD information from the NWDAF to determine whether to create/update/delete PFD information corresponding to the Application Identifier.

4.	The NEF (PFDF) invokes the Nudr_DM_Create/Update/Delete (Application Identifier, one or more sets of PFDs) to the UDR to create/update/delete PFD information corresponding to the Application Identifier, i.e. from step 3 to step 5 of clause 4.18.2 are executed. The NEF(PFDF) may forward new/updated PFD information to UPF via SMF to detect a known application described in clause 4.4.3.5.

# *** End of Changes ***