3GPP TSG-WG SA2 Meeting #154-AH-e 	S2-2302061

Elbonia, January 16 – 20, 2023

Title:	Reply LS on Issues on historical analytics and Slice Load Level Analytics

Reply to:	LS on Issues on historical analytics and Slice Load Level Analytics (C3-225599 / S2-2300094)

Release:	Rel-17

Work Item:	eNA_Ph2

Source:	SA2

To:	CT3

Cc:

Contact person:	Susan

susan.shishufeng@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2302068 (CR 0648), S2-2302069 (CR 0697)

# Overall description

SA2 thanks CT3 for their LS on Issues on historical analytics and Slice Load Level Analytics (C3-225599 / S2-220xxxx) and provides the following answers to CT3 questions.

CT3 noticed that clause 6.1.4.3 of 3GPP TS 23.288 indicates that the historical analytics may be computed and stored in an NWDAF, while the procedure of Figure 6.1.4.3-1 also includes invocations of the Nnwdaf_DataManagement service. However, clause 5A.1 of 3GPP TS 23.288 indicates that the Nnwdaf_AnalyticsSubscription service can be used to collect analytics and the Nnwdaf_DataManagement service can be used to collect historical data or runtime data.

Question 1: Why is the Nnwdaf_DataManagement service shown in the procedure for collecting historical analytics?

SA2 answer to question 1:

SA2 believes that the Nnwdaf_DataManagement service shall not be used in the Historical Analytics Exposure via DCCF/Messaging Framework procedures. The Nnwdaf_AnalyticsSubscription service shall be used when collecting the historical analytics from the NWDAF. The related clauses were updated in the attached CR xxxx.

Question 2: How may the consumer collect historical analytics from the NWDAF?

SA2 answer to question 2:

See the answer to Question 1.

Clause 7.4.2 of 3GPP TS 23.288 includes the Analytics Specification as a possible input parameter to Nnwdaf_DataManagement (as an alternative to the DataSpecification) and says that:

“When the required data is a bulked data for an Analytics ID, the Data Specification includes Target of Reporting with the Analytics ID to generate bulked data, Target of Analytics reporting and Analytics Filter.”

Clause 6.2.6.2 (Procedure for Data Collection from NWDAF) also repeatedly includes the Analytics ID as an Alternative to the Event ID when requesting data from an NWDAF using the Nnwdaf_DataManagement service.

Question 3: Shall the Analytics Specification (including the Analytics ID mentioned in clause 6.2.6.2 of 23.288) be used by the NWDAF when collecting the data? If yes, what is the Use Case where the NF service consumer cannot perform this determination itself in order to provide the Data Specification directly to the NWDAF?

SA2 answer to question 3:

The answer is Yes. The Analytics Specification may be used when the consumer requests the bulked data from an NWDAF.

The scenario is for data collection optimization among NWDAFs, when NWDAF#1 alone is not able to generate the analytics, because part of the data that needs to be collected is under the control of a different NWDAF#2. Or when the NWDAF#1 with MTLF needs to collect data for ML model training and the required data from an AoI is covered by different NWDAF#2. In these scenarios, the NWDAF#1 does not need to request every individual event that it needs to collect from the other NWDAF#2, it requests only the bulked data for the analtyics ID, and then the NWDAF#2 knows what are the event types that need to be bulked and sends the bulked data to the NWDAF.

# 2	Actions

To CT3:

ACTION: 	SA2 kindly asks CT3 to take SA2 answers into account.

# 3	Dates of next TSG SA WG 2 meetings

3GPP TSG SA2#155	20th - 24th February 2023	Athens, Greece