3GPP TSG-SA2 Meeting #154AH (e-meeting)	S2-2300117r01

# Elbonia, January 16th – 20th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0605 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support MTLF Discovery and Model Information Exchange for Federated Learning in 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support MTLF registration and discovery, and model information exchange for Federated Learning in 5GC, three editor’s notes need to be solved. - Solving the EN “Procedure of MTLF registration and discovery with respect to FL is FFS” in clause 6.2C.2.1 of TS 23.288 to support FL among multiple NWDAFs. - Solving the EN “The ML model information exchanged between the Client NWDAF(s) and the Server NWDAF is FFS” in clause 6.2C.2.1 of TS 23.288 to support FL model information exchanging between the Client NWDAF(s) and the Server NWDAF. - Solving the EN “The services in step 1, 3, 5a and 6 to enable FL based ML model training is for FFS, and the Figure 6.2X.2.1-1 will be updated accordingly.” to add details and make the FL procedure complete and clearer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add steps for MTLF registration and discovery to clause 6.2C.2.1 to support FL among multiple NWDAFs. Extend the existing service Nnwdaf_MLModelProvision in TS 23.288 to support FL model information exchanging between the Client NWDAF(s) and the Server NWDAF. Add descriptions on how to use the extended service for FL in clause 6.2C.2.2. Define a new service Nnwdaf_MLPreparation to enable the consumer to request NWDAF containing MTLF to prepare or perform ML Model Training. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for MTLF registration and discovery, and for FL model information exchanging between the Client NWDAF(s) and the Server NWDAF are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.1, 6.2C.2.X (new), 6.2A.1, 6.2A.2, 7.1, 7.5.2, 7.5.3, 7.5.4, 7.Y (new), 7.Y.1 (new), 7.Y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 6.2C.2.1	General procedure for Federated Learning among Multiple NWDAFs

Figure 6.2C.2.1-1: General procedure for Federated Learning among Multiple NWDAFs

0a. NWDAFs register into NRF.

0b.	The consumer (NWDAF containing AnLF) sends a subscription request to NWDAF containing MTLF to retrieve a ML model, including Analytic ID and ML model filter information as described in clause 7.5.2, the NWDAF containing MTLF can be a FL server (Server NWDAF) with FL server capability or a MTLF without FL server capability.

0c. Server NWDAF discovers Client NWDAFs via NRF. NRF provides list of Client NWDAD(s) to the Server NWDAF based on FL capability, Analytics ID, Interoperability Indicator, Time interval supporting FL, etc.

Editor's note:	Definition of FL capability is FFS.

1.  Server NWDAF sends Federated Learning preparation request to the Client NWDAF(s) by invoking an Nnwdaf_MLPreparation_Request service operation with Interoperability information. In the preparation request, the following parameters may be added: Available data requirement, availability time requirement.

2.  Client NWDAF(s) decides whether to join the Federated Learning process based on its availability, computation and communication capability, and Interoperability information.

3.  Client NWDAF(s) sends response to Server NWDAF indicating if it will join the FL procedure.

4.  Server NWDAF conducts selection of Client NWDAF(s).

5a.	Server NWDAF informs the selected NWDAF containing MTLF(Client NWDAF) to perform the Federated Learning by invoking an Nnwdaf_MLPreparation_Request service operation with initial ML Model Information, a Federated Learning execution flag. Server NWDAF includes FL Training Informationin the preparation message.

5b. Server NWDAF and Client NWDAF(s) subscribe to each other for exchanging ML model information as described in clause 6.2C.2.2 by invoking Nnwdaf_MLModelProvision_Subscribe service operation.

6.	Each Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

7.	During Federated Learning training procedure, each Client NWDAF further trains the retrieved ML model from the server NWDAF based on its own data, and reports interim local ML model information to the Server NWDAF as defined in clause 6.2C.2.2 by invoking Nnwdaf_MLModelProvision_Notify service operation.

The ML model information are exchanged between the Client NWDAF(s) and the Server NWDAF during the FL training process.

8.	The Server NWDAF aggregates all the local ML model information retrieved at step 7, to update the global ML model.

9a.	Based on the consumer request, the Server NWDAF provides the training status (i.e., an accuracy level/information) to the consumer by invoking Nnwdaf_MLModelProvision_Notify service operation periodically (one or multiple rounds of training or every 10 min, etc.) or dynamically when some pre-determined status (e.g. some accuracy level) is achieved.

9b.	[Optional] Consumer decides whether the current model can fulfil the requirement e.g. accuracy and time. The consumer modifies ML model subscription if the current model can fulfil the requirement.

Editor's note:	Further details on providing the accuracy of FL model process is for FFS.

9c.	According to the request from the consumer, Server NWDAF updates or terminates the current FL training process.

10.	If the FL procedure continues, Server NWDAF sends the aggregated ML model information to each Client NWDAF for next round model training as defined in clause 6.2C.2.2 by invoking Nnwdaf_MLModelProvision_Notify service operation.

11.	Each Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the Server NWDAF at step 10.

NOTE:	The steps 7-11 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is complete, the Server NWDAF may send the globally optimal ML model information to the consumer.

* * * Next Changes (all new) * * *

#### 6.2C.2.X	Model Information Exchange for Federated Learning among Multiple NWDAFs

The Nnwdaf_MLModelProvision service is used for model sharing/parameter exchanging in the execution phase of a ML training process among multiple NWDAFs.

Server NWDAF subscribes to Client NWDAF(s) for getting local model information by invoking an Nnwdaf_MLModelProvision_Subscribe service operation, Client NWDAF(s) subscribes to Server NWDAF for getting global model informaiton by invoking an Nnwdaf_MLModelProvision_Subscribe service operation as in 6.2A.1 with the following difference:

The Server NWDAF and Client NWDAF(s) include the following parameters in the Nnwdaf_MLModelProvision_Subscribe request:

-	Identity of the current ML process, i.e., FL Correlation ID.

The Server NWDAF and Client NWDAF(s) include the following parameters in the Nnwdaf_MLModelProvision_Notify message:

-	Identity of the current ML process, i.e., FL Correlation ID.

-	Identity of the current iteration round, i.e., IR ID (e.g., IR ID = 1, 2, 3, ...).

NOTE: Whether Server NWDAF and Client NWDAFs sharing model or model parameters depending on the initial information in step 5 in clause 6.2C.2.1.

* * * Next Changes * * *

### 6.2A.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2A.1-1 is used by an NWDAF service consumer, i.e. an NWDAF containing AnLF or MTLF to subscribe/unsubscribe at another NWDAF, i.e. an NWDAF containing MTLF, to be notified when ML Model Information on the related Analytics becomes available or for model sharing/parameter exchanging in the execution phase of a Federated Learning training process among multiple NWDAFs, using Nnwdaf_MLModelProvision services as defined in clause 7.5. The ML Model Information is used by an NWDAF containing AnLF to derive analytics or by an NWDAF containing MTLF to update (global or local) model. The service is also used by an NWDAF to modify existing ML Model Subscription(s). An NWDAF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.1-1: ML Model for analytics subscribe/unsubscribe

1.	The NWDAF service consumer (i.e. an NWDAF containing AnLF or MTLF) subscribes to, modifies, or cancels subscription for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2A.2. The service consumer optionally indicates its support for multiple ML models if available.

When a subscription for a trained ML model associated with an Analytics ID is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the subscription; or

-	determine whether triggering further training for the existing trained ML models is needed for the subscription.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in clause 6.2, to generate the ML model.

If the service invocation is for a subscription modification or subscription cancelation, the NWDAF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

If the service invocation is for model sharing/parameter exchanging in the execution phase of a Federated Learning training process among multiple NWDAFs, the NWDAF service consumer includes Identity of the current ML process (FL Correlation ID) in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the NWDAF service consumer subscribes to a (set of) trained ML model(s) associated to a (set of) Analytics ID(s), the NWDAF containing MTLF notifies the NWDAF service consumer with:

-	the trained ML Model Information (containing a (set of) file address(es) of the trained ML model(s)), when multiple ML models is not supported by the consumer; or

-	a set of pair of unique ML Model Identifier and ML Model Information associated with an Analytics ID, when multiple ML models is supported by the consumer.

NOTE 1:	The structure and format of the ML Model identifier and its uniqueness are up to stage 3.

NOTE 2:	Parameters defined for Multiple models are for Analytics accuracy enhancement.

by invoking Nnwdaf_MLModelProvision_Notify service operation. The content of trained ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify service operation to notify an available re-trained ML model when the NWDAF containing MTLF determines that the previously provided trained ML Model required re-training at step 1.

When the step 1 is for a subscription modification (i.e. including Subscription Correlation ID), the NWDAF containing MTLF may provide either a new trained ML model different to the previously provided one, or re-trained ML model by invoking Nnwdaf_MLModelProvision_Notify service operation.

When the step 1 is for model sharing/parameter exchanging in the execution phase of a Federated Learning training process among multiple NWDAFs, FL Correlation ID and Identity of the current iteration round (IR ID (e.g., IR ID =1, 2, …)) should be contained in the Nnwdaf_MLModelProvision_Notify message.

* * * Next Changes * * *

### 6.2A.2	Contents of ML Model Provisioning

The consumers of the ML model provisioning services (i.e. an NWDAF containing AnLF or MTLF) as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	Information of the analytics for which the requested ML model is to be used, including:

-	A list of Analytics ID(s): identifies the analytics for which the ML model is used.

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML model.

NOTE 1:	The NWDAF containing MTLF can use the parameter "Use case context" to select the most relevant ML model, when several ML models are available for the requested Analytics ID(s). The values of this parameter are not standardized.

-	[OPTIONAL] ML Model Interoperability Information. This is vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.

-	[OPTIONAL] ML Model Filter Information: enables to select which ML model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML model is requested, e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	ML Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

-	(Only for model sharing/parameter exchanging in the execution phase of a Federated Learning training process among multiple NWDAFs) Identity of the current ML process, i.e., FL Correlation ID.

-	[OPTIONAL] Indication of supporting multiple ML models.

-	[OPTIONAL] accuracy level of Interest.

Editor's note:	It is FFS if additional parameters are needed for multiple model provisioning.

The NWDAF containing MTLF provides to the consumer of the ML model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	ML Model Information, which includes:

-	the ML model file address (e.g. URL or FQDN) for the Analytics ID(s), when multiple ML models is not supported; or.

-	a set of pair of unique ML Model identifier and the ML model file address (e.g. URL or FQDN) for the Analytics ID(s), if multiple ML models is supported.

-	[OPTIONAL] Validity period: indicates time period when the provided ML Model Information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

NOTE 2:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

- (Only for model sharing/parameter exchanging in the execution phase of a Federated Learning training process among multiple NWDAFs) Identity of the current ML process, i.e., FL Correlation ID.

- (Only for model sharing/parameter exchanging in the execution phase of a Federated Learning training process among multiple NWDAFs) Identity of the current iteration round (e.g., IR ID = 1, 2, … for Nnwdaf_MLModelProvision_Notify).

* * * Next Changes * * *

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_MLPreparation | Request | Request / Response | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |

* * * Next Changes * * *

### 7.5.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe.

Description: Subscribes to NWDAF ML model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model subscription), ML Model Filter Information to indicate the conditions for which ML model for the analytics is requested and Target of ML Model Reporting to indicate the object(s) for which ML model is requested (e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs)), ML Model Reporting Information (including e.g. ML Model Target Period), Expiry time, Use case context, indication of support for multiple ML models, multiple ML models Filter Information to indicate the conditions for which multiple ML models are requested, FL Correlation ID (when for model sharing/parameter exchanging in the execution phase of a Federated Learning training process among multiple NWDAFs).

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

* * * Next Changes * * *

### 7.5.3	Nnwdaf_MLModelProvision_Unsubscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Unsubscribe.

Description: unsubscribe to NWDAF ML model provision.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: FL Correlation ID when for model sharing/parameter exchanging in the execution phase of a Federated Learning training process among multiple NWDAFs.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * Next Changes * * *

### 7.5.4	Nnwdaf_MLModelProvision_Notify service operation

Service operation name: Nnwdaf_MLModelProvision_Notify.

Description: NWDAF notifies the ML model information to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information, Set of:

-	the tuple (Analytics ID, address (e.g. URL or FQDN) of Model file), when multiple ML models is not supported; or

-	the tuple (Analytics ID, one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file).

Inputs, Optional: Validity period, Spatial validity, FL Correlation ID and Identity of the current iteration round (IR ID) when for model sharing/parameter exchanging in the execution phase of a Federated Learning training process among multiple NWDAFs.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * Next Changes (all new) * * *

## 7.Y	Nnwdaf_MLPreparation service

### 7.Y.1	General

Service description: this service enables the consumer (e.g., Server NWDAF in FL process) to request NWDAF containing MTLF (e.g., Client NWDAF in FL process) to prepare or perform ML Model Training.

### 7.Y.2	Nnwdaf_MLPreparation_Request service operation

Service operation name: Nnwdaf_MLPreparation_Request

Description: The consumer requests NWDAF to prepare or perform ML Model Training.

Inputs, Required:

Interoperability information, available data requirement, and availability time requirement, when the Federated Learning execution is not provided.

Initial ML Model Information (One of the following three types: ADRF ID with ML model identifier, or address (e.g., URL or FQDN) of Model file,), FL Training Information when the Federated Learning execution flag is provided.

FL Training Information include FL Correlation ID, and guideline information (e.g., maximum response time for FL client to provide interim local ML model informationin)..

Inputs, Optional: Federated Learning execution flag.

Outputs, Required: None.

Outputs, Optional: Indication of join Federated Learning when the Federated Learning execution flag is not provided.

* * * End of Changes * * *