3GPP TSG-SA2  Meeting #154AH  	S2-23-00128r02

# January 16 – 20, 2023	(revision of S2-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3823 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: Support for HPLMN admission mode while Roaming. |
| --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Fulfill the GSMA requirement to enable an HPLMN to handle admission related to number of registered UEs and the number of LBO PDU sessions for its outbound roamers in any VPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support two modes to fulfill GSMA requirement: First mode, HPLMN delegated model where HPLMN delegates to VPMLN admission with the actual quota information fetched by the VPLMN from the HPLMN. HPLMN can update the quota information real-time. Additionally VPLMN sends any additional requests beyond the quota for the HPLMN to decide on their handling, with the result conveyed to the VPLMN. In the second mode, all admission requests are sent to the HPLMN for enforcement and handling. Both modes apply to centralized and hierarchal NSACF architectures. In addition to the above, the Release 17 VPLMN admission mode is enhanced to enable the VPLMN to fetch the quota to be administered by the VPLMN to avoid configuring the VPLMN with the information. This also enables the HPLMN to update this information real-time. Real-time update of quota information by HPLMN is achieved through the use of the Subscribe/Notify NSACF framework extended with a new event. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No Support for roaming as required by eNS-PH3 for KI#4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4, 4.2.7, 5.15.11.3. 5.25.11.3.1, New Clause (5.15.11.3.2, 5.15.11.3.3), 6.2.28, 6.3.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ….. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 4.2.7	Reference points

The 5G System Architecture contains the following reference points:

N1:	Reference point between the UE and the AMF.

N2:	Reference point between the (R)AN and the AMF.

N3:	Reference point between the (R)AN and the UPF.

N4:	Reference point between the SMF and the UPF.

N6:	Reference point between the UPF and a Data Network.

N9:	Reference point between two UPFs.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

N5:	Reference point between the PCF and an AF or TSN AF.

N7:	Reference point between the SMF and the PCF.

N8:	Reference point between the UDM and the AMF.

N10:	Reference point between the UDM and the SMF.

N11:	Reference point between the AMF and the SMF.

N12:	Reference point between AMF and AUSF.

N13:	Reference point between the UDM and Authentication Server function the AUSF.

N14:	Reference point between two AMFs.

N15:	Reference point between the PCF and the AMF in the case of non-roaming scenario, PCF in the visited network and AMF in the case of roaming scenario.

N16:	Reference point between two SMFs, (in roaming case between SMF in the visited network and the SMF in the home network).

N16a:	Reference point between SMF and I-SMF.

N17:	Reference point between AMF and 5G-EIR.

N18:	Reference point between any NF and UDSF.

N19:	Reference point between two PSA UPFs for 5G LAN-type service.

N22:	Reference point between AMF and NSSF.

N23:	Reference point between PCF and NWDAF.

N24:	Reference point between the PCF in the visited network and the PCF in the home network.

N27:	Reference point between NRF in the visited network and the NRF in the home network.

N28:	Reference point between PCF and CHF.

N29:	Reference point between NEF and SMF.

N30:	Reference point between PCF and NEF.

NOTE 1:	The functionality of N28 and N29 and N30 reference points are defined in TS 23.503 [45].

N31:	Reference point between the NSSF in the visited network and the NSSF in the home network.

NOTE 2:	In some cases, a couple of NFs may need to be associated with each other to serve a UE.

N32:	Reference point between a SEPP in one PLMN or SNPN and a SEPP in another PLMN or SNPN; or between a SEPP in a SNPN and a SEPP in a CH/DCS, where the CH/DCS contains a UDM/AUSF.

NOTE 3:	The functionality of N32 reference point is defined in TS 33.501 [29].

N33:	Reference point between NEF and AF.

N34:	Reference point between NSSF and NWDAF.

N35:	Reference point between UDM and UDR.

N36:	Reference point between PCF and UDR.

N37:	Reference point between NEF and UDR.

N38:	Reference point between I-SMFs and between V-SMFs.

N40:	Reference point between SMF and the CHF.

N41:	Reference point between AMF and CHF in HPLMN.

N42:	Reference point between AMF and CHF in VPLMN.

NOTE 4:	The functionality of N40, N41 and N42 reference points are defined in TS 32.240 [41].

N43:	Reference point between PCFs.

NOTE 5:	The functionality of N43 reference point is defined in TS 23.503 [45].

NOTE 6:	The reference points from N44 up to and including N49 are reserved for allocation and definition in TS 32.240 [41].

N50:	Reference point between AMF and the CBCF.

N51:	Reference point between AMF and NEF.

N52:	Reference point between NEF and UDM.

N55:	Reference point between AMF and the UCMF.

N56:	Reference point between NEF and the UCMF.

N57:	Reference point between AF and the UCMF.

NOTE 7:	The Public Warning System functionality of N50 reference point is defined in TS 23.041 [46].

N58:	Reference point between AMF and the NSSAAF.

N59:	Reference point between UDM and the NSSAAF.

N60:	Reference point between AUSF and NSWOF.

NOTE 8:	The functionality of N60 reference point is defined in TS 33.501 [29].

N80:	Reference point between AMF and NSACF.

N81:	Reference point between SMF and NSACF.

N82:	Reference point between NSACF and NEF.

N83:	Reference point between AUSF and NSSAAF.

N84:	Reference point between TSCTSF and PCF.

N85:	Reference point between TSCTSF and NEF.

N86:	Reference point between TSCTSF and AF.

N87:	Reference point between TSCTSF and UDM.

N88:	Reference point between SMF and EASDF.

Nyy:	Reference point between two NSACFs in different PLMNs.

NOTE 9:	The reference points from N90 up to and including N95 are reserved for allocation and definition in TS 23.503 [45].

The reference points to support SMS over NAS are listed in clause 4.4.2.2.

The reference points to support Location Services are listed in TS 23.273 [87].

The reference points to support SBA in IMS (N5, N70 and N71) are described in TS 23.228 [15].

The reference points to support AKMA (N61, N62 and N63) are described in TS 33.535 [124].

The reference points to support 5G ProSe are described in TS 23.304 [128].

The reference points to support 5G multicast-broadcast services are described in TS 23.247 [129].

The reference points to Support Uncrewed Aerial Systems (UAS) connectivity, identification and tracking are described in TS 23.256 [136].

The reference points to support SBA in GBA and GBA push (N65, N66, N67 and N68) are described in TS 33.220 [140] and TS 33.223 [141].

The reference points to support SMS delivery using SBA are described in TS 23.540 [142].

# * * * * 3rd change * * * *

#### 5.15.11.3	Network Slice Admission Control for Roaming

##### 5.15.11.3.0	General

In the case of roaming, depending on operator's policy, a roaming agreement or an SLA between the VPLMN and the HPLMN, NSAC of roaming UEs is performed by one of the following modes:

- VPLMN NSAC Admission; or

- VPLMN with HPLMN assistance NSAC Admission; or

- HPLMN NSAC Admission.

The VPLMN (AMF and SMF) identifies the mode to apply from the AMF subscription data at UE registration, and from the SMF subscription data at PDU session establishment.

For all the above modes, for NSAC of roaming UEs for maximum number of UEs per network slice and/or maximum number of PDU Sessions per network slice managed by the VPLMN, each S-NSSAI of the HPLMN that is subject to NSAC is mapped to a corresponding S-NSSAI of the VPLMN subject to NSAC.

For both VPLMN NSAC Admission and VPLMN with HPLMN assistance NSAC Admission modes, each configured S-NSSAI that is subject to NSAC and that is mapped from the HPLMN S-NSSAI,  the SMF performs NSAC for PDU home routed sessions according to the principles described in Clause 5.15.11.2

##### 5.15.11.3.1 	VPLMN NSAC Admission Mode

The following principles apply:

For NSAC of roaming UEs for maximum number of UEs per network slice and/or maximum number of PDU Sessions per network slice managed by the VPLMN, the following principles shall be used:

-

-	For NSAC for the maximum number of UEs for S-NSSAI of the HPLMN, a NSACF in the VPLMN can be configured with the maximum number of allowed roaming UEs per mapped S-NSSAI of the HPLMN for a S-NSSAI of the HPLMN that is subject to NSAC. In such a case, the AMFs trigger a request to a NSACF of the VPLMN.

-	For NSAC for the maximum number of PDU Sessions for S-NSSAI of the HPLMN, a NSACF in the VPLMN can be configured with the maximum number of allowed PDU Sessions in LBO mode per mapped S-NSSAI of the HPLMN for a S-NSSAI of the HPLMN that is subject to NSAC. In such a case, the anchor SMF in the VPLMN triggers a request to a NSACF of the VPLMN.

-	For NSAC for the maximum number of UEs for S-NSSAI of the VPLMN, AMFs trigger a request to a NSACF of the VPLMN to perform NSAC based on the S-NSSAI of the VPLMN subject to NSAC. The NSACF of the HPLMN is not involved.

-	For NSAC for the maximum number of PDU Sessions for S-NSSAI of the VPLMN in the LBO roaming case, the SMF triggers a request to a NSACF of the VPLMN to perform NSAC based on the S-NSSAI of the VPLMN subject to NSAC. The NSACF of the HPLMN is not involved.

‐	The AMF or SMF (in LBO roaming case) in the VPLMN provides both the S-NSSAI in the VPLMN and the corresponding mapped S-NSSAI in the HPLMN to the NSACF in the VPLMN. The NSACF in the VPLMN performs NSAC for both S-NSSAI of the VPLMN and the corresponding mapped S-NSSAI of the HPLMN based on the SLA between the VPLMN and the HPLMN.

- In addition to configuring the VPLMN NFs with the maximum number of allowed roaming UEs per mapped S-NSSAI of the HPLMN subject to NSAC, and the maximum number of allowed PDU Sessions in LBO mode per mapped S-NSSAI of the HPLMN subject to NSAC, the VPLMN can optionally fetch this information from the HPLMN primary NSACF in a hierarchal architecture or centralized NSACF in a centralized architecture, via the Subscribe/Notify framework dedicated to that.  However, in this case, the VPLMN rejects any additional requests exceeding the received information.

NOTE:	If the NSAC does not have quota configured or received through a Subscribe/Notify, the NSAC in VPLMN may receive quota in its initial interaction with HPLMN.

##### 5.15.11.3.2 	VPLMN with HPLMN assistance NSAC Admission

In this admission mode HPLMN delegates NSAC for NSSAIs subject to NSAC to the VPLMN, both for number of registered UEs and the number of LBO PDU sessions.

Every NSACF performing admission in the VPLMN for each S-NSSAI of the HPLMN that is subject to NSAC and mapped to a corresponding S-NSSAI of the VPLMN, fetches from the VPLMN primary NSACF in a hierarchal architecture the maximum number of registered UEs to be admitted and/or the maximum number of LBO PDU sessions to be allowed via the Subscribe/Notify framework using an event dedicated to that.  The VPLMN primary or central NSACF, in turn, acquires the information from the HPLMN central or primary NSACF depending on the deployed architecture. If re-distribution of quota is required in the VPLMN in a hierarchal architecture, amongst multiple NSACFs than this is handled by the primary NSACF in VPLMN with no involvement from the HPLMN.  The VPLMN NSACF discovers the HPLMN primary or central NSACF or be configured with the needed information.

NOTE:	If the NSAC does not have quota configured or received through a SUBSCRIBE/NOTIFY, the NSAC in VPLMN may receive quota in its interaction with HPLMN.

For any request(s) received in any NSACF in the VPLMN exceeding the received maximum number information, the NSACF interacts with the VPLMN primary NSACF which in turn interacts with HPLMN primary or central NSACF to receive an updated roaming quota for the corresponding mapped S-NSSAI, which is used to determine whether admission request is accepted or rejected, unless forbidden by the SLA. If an admission request is accepted the UE entry is stored in the NSACF performing admission in the VPLMN. This applies to the number of registered UEs as well as the number of LBO PDU sessions. The primary  NSACF in VPLMN may re-distribute the received updated roaming quota to the NSACFs in VPLMN to perform NSAC for Roaming UEs according to the principles described in Clause 5.15.11.1 and 5.15.11.2.

NOTE:	In a VPLMN centralized architecture the NSACF performing admission is the central NSACF.

##### 5.15.11.3.3 	HPLMN NSAC Admission Mode

In this admission mode the AMF or SMF in VPLMN interacts with HPLMN for admission both for number of registered UEs and the number of LBO PDU sessions.

For each S-NSSAI of the HPLMN that is subject to NSAC and mapped to a corresponding S-NSSAI of the VPLMN, AMF performs NSAC admission for the number of registered UEs with the HPLMN central or primary NSACF for all inbound roamers from that HPLMN when they register in this VPLMN. The AMF discovers the HPLMN primary or central NSACF or optionally be configured with the needed information.

For each S-NSSAI of the HPLMN that is subject to NSAC and mapped to a corresponding S-NSSAI of the VPLMN, every SMF in this VPLMN performs NSAC admission for the number of LBO PDU sessions with the HPLMN central or primary NSACF for all inbound roamers from that HPLMN when they initiate an LBO PDU session. The SMFs discovers the HPLMN primary or central NSACF or be configured with the needed information.

NOTE:	In a VPLMN centralized architecture the NSACF performing admission is the central NSACF.

For each S-NSSAI of the HPLMN that is subject to NSAC, the SMF performs NSAC according to the principles described in Clause 5.15.11.2, for PDU home routed sessions

In the HPLMN NSAC admission mode, the primary NSACF or central NSACF in HPLMN determines whether the NSAC admission request for a roaming UE is accepted or rejected.

# * * * * 4th change * * * *

### 6.2.28	NSACF

The Network Slice Admission Control Function (NSACF) supports the following functionality:

-	Support of monitoring and controlling the number of registered UEs per network slice.

-	Support of monitoring and controlling the number of established PDU Sessions per network slice.

-	Support of event based Network Slice status notification and reports to a consumer NF.

-	Support of different type of NSAC modes for roaming UEs for the number of UEs per network slice.

-	Support of different type of NSAC modes for roaming UEs for the number of PDU Sessions per network slice.

The details of the NSACF functionality are defined in clause 5.15.11.

# * * * * 5th change * * * *

### 6.3.22	NSACF discovery and selection

The NF consumers shall utilise the NRF to discover NSACF instance(s)unless NSACF information is available by other means, e.g. locally configured in NF consumers.

If the NSACF NF consumer is the AMF, the NSACF selection function in the AMF selects an NSACF instance based on the available NSACF instances, which are obtained from the NRF or locally configured in the AMF.

The following factors may be considered by the NF consumer for NSACF selection:

-	S-NSSAI(s).

-	NSACF Serving Area information. The NSACF service area is related to the location of the NF consumer

NOTE:	Each Serving Area is unique and unambiguously identified.

-	NSACF service capabilities:

-	Support monitoring and controlling the number of registered UEs per network slice for the network slice that is subject to NSAC.

-	Support monitoring and controlling the number of established PDU Sessions per network slice for the network slice that is subject to NSAC.

-	PLMN ID information in the case of roaming.

In the case of delegated discovery and selection in SCP, the NSACF NF consumer shall send all available factors to the SCP.

# * * * * End of changes * * * *