SA WG2 Meeting #154-AH-e	S2-2300134r01

E-meeting,  2023-01-16 – 2023-01-20

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3665 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data Exposure by UPF via SBI aligned with TS 23.502 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UPF event expoure update according to UPEAS conclusions and new PFD Determination Analytic in Rel-18 TS 23.288 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is removing the Editor Notes and completing the specification of UPF Event Exposure for UPF Data Collection by NWDAF, including the updates related to identifying in SMF the QoS Flows bound to an application Identifier for Analytics. In addition, the User data Usage event is enhaced to include the UPF Data Collection needs of Rel’18 3GPP TS 23.288 [50] new PFD Determination analytic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This new UPF capability defined in UPEAS is not supported. NWDAF can not collect from UPF the information it needs for “PFD Determination” analytic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4, 4.15.11, 5.2.8.3, 5.2.26.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.503 CR 0798 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR removes totally clause 4.15.11 added last plenary not to waste clause numbering.Changes compared to current text in 4.15.11 are highlighted below to assist tracking actual impact from this CR |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

The text below illustrates the changes proposed compared to text in TS clause 4.15.11. The CR proposes to move the

contents in 4.15.11 into a completely new subclause within 4.15.4, and all text is then shown as new.

4.15.4.x	Exposure of Events from UPF for UPF Data Collection

4.15.4.x.1	General

This clause contains the detailed description and the procedures for how the UPF event exposure service (see 5.2.26.2) is used for UPF data collection.

The NWDAF can subscribe to the UPF exposure events for data collection via the SMF, and the UPF will directly send the collected UPF data to NWDAF. Or, the NWDAF can subscribe directly to UPF.

The Subscription to UPF events via SMF is the rule though there are some exceptions listed below; Subscription via SMF means the final consumer of UPF event notifications sends the subscription request to the SMF and then the SMF is doing a third-party subscription onto UPF on behalf of this final consumer. Table 4.15.4.x.1-1 details which subscription parameters apply to direct subscription to UPF events and to subscription to UPF via SMF. Conversely the notifications are directly sent by the UPF to the final consumer of UPF notifications.

Exceptions to subscriptions via SMF are subscriptions for data collection where UPF is the source as defined in TS 23.288 [5] in the following cases:

-	Data collected by NWDAF is for UPF load analytics described in Table 6.5.2-2 of TS 23.288 [5].

-	Data Collected by NWDAF is targeting "any UE" (possibly for specific DNN and or slices) and not related with an AoI or BSSID/SSID or application.

The UPF event exposure events are described in clause 5.2.26.2.  In this release of the specification, following events are used for UPF Data collection:

-	QoS Monitoring. This event provides QoS Flow performance information

-	UserDataUsageMeasures. This event provides information of user data usage of the User PDU Session

-	UserDataUsageTrends. This event provides statistics related to user data usage of the User PDU Session

NWDAF can subscribe to QoS monitoring event via SMF only and UPF sends the QoS Flow Performance information directly to NWDAF. For this event, the interaction between SMF and UPF is over PFCP (TS 29.244 [69]). NWDAF subscription request can trigger in SMF end to end UL/DL delay measurements but only for QoS flows that have already been established. Clause 5.33.3 describes how end to end UL/DL delay measurements for QoS flows are activated and measured triggered by a PCC received in SMF. By default, the Subscription request for QoS monitoring event refers to the QoS Flow associated to the default QoS rule. The subscription can target the QoS flows bound to an application by including an application identifier. In SMF, that identifier is compared to the UPF Exposure Correlation Identifier in the PCC rule (s) that control QoS for the application traffic (see TS 23.503 [20] clause 6.1.3.22). This procedure allows SMF to determine the PCC rules relevant for this request and the QoS flows to monitor.

NOTE 1:	Extensive usage of QoS Monitoring has significant impact on load and signalling.

NWDAF can subscribe to User Data Usage events directly to UPF or via SMF, and UPF sends the event notifications directly to NWDAF. For this event, the interaction between SMF and UPF is over SBI. For User Data Usage events, the subscription request may target specific service data flows (e.g. a specific application traffic) by including a traffic description (e.g. an Application Id). Else, the scope of the subscription is all the traffic in the PDU Session. The subscription request may indicate certain granularity for the information e.g. how many differentiated measurements are requested: one measurement that is considering all the target traffic, measurements per PDR that are based on the service data flows matching the PDR, measurements per service data flow.

NOTE 2:	PDR Provisioning and PDR matching have an impact on signalling and load in UPF.

Table 4.15.4.x.1-1: Input parameters in subscription to UPF event Exposure events

| Information | To SMF | To UPF |
| --- | --- | --- |
| UE ID | Y | Y |
| GroupID | Y | N |
| AnyUE | Y | Y |
| DNN | Y | Y |
| S-NSSAI | Y | Y |
| AOI | Y | N |
| BSSID/SSID | Y | N |
| DNAI (NOTE3) | Y | N |
| UPFId (NOTE 3) | Y | N |
| Type of Measurement | Y | Y |
| Granularity of Measurement (NOTE 2) | Y | Y |
| Application ID (NOTE 1) | Y | Y |
| Traffic Filtering (NOTE 1), (NOTE 2) | Y | Y |
| NOTE 1: Application ID and Traffic Filtering are exclusive NOTE 2: This parameter does not apply to QoS monitoring event NOTE 3: These parameters are used to indicate a UP path |  |  |

4.15.4.x.2	Information flow for UPF event exposure service to NWDAF for certain UE(s)

Figure 4.15.4.x.2-1: UPF Information Exposure to NWDAF

In the case of a group of UEs, the NWDAF issues an Nnrf_NFDiscovery_Request service operation to find the UDM providing the NF type and Group ID and gets the NF profile of the UDM that currently meets the NWDAF discovery request. Then, NWDAF obtains the list of SUPIs that correspond to the Group ID from UDM using Nudm_SDM_Get

NOTE 1:	It is assumed that all members of a Group ID belong to the same UDM.

Then, for each SUPI,

1.	The NWDAF invokes Nudm_UECM_Get service operation to retrieve the appropriate SMF by providing UE ID and NF type, to request UPF information Exposure for a certain UE.

2.	The UDM provides a Nudm_UECM_Get response to the NWDAF with the corresponding SMF.

3.	The NWDAF sends the request to the SMF to subscribe UPF data, including the following information:

-	Notification Target Address (NWDAF address).

-	Indication of UPF Event Exposure Service.

-	Event Filter Information: S-NSSAI, DNN, DNAI, UPFId, Traffic Description for the target traffic (e.g. Application Id), Area of Interest, SSID/BSSID.

-	Target of Event Reporting: a UE.

-	Target Subscription information: event, Type of Measurement and granularity of the Information requested.

4.	The SMF selects the PDU session(s) and the UPFs it has to send the request to. SMF sends the request to the UPF including the NWDAF address, Event Filter Information, Target of Event Reporting, and Target Subscription Information as required. Target of Event Reporting is certain UE.. The interaction mechanism used between SMF and UPF depends on UPF exposure event (see clause 5.2.26.2.1). It could be as in 4a with N4 Session Modification with PFCP (TS 29.244 [69]) or as in 4b with Nupf_event exposure subscribe request.

NOTE 2:	Some events may require SMF interacts with RAN at this stage.

5.	The UPF responds with the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the NWDAF.

4.15.4.x.3	Information flow for UPF event exposure service to NWDAF for anyUE

When the NWDAF subscription request does not meets the criteria for direct subscription to UPF as described in 4.15.4.x.1, NWDAF shall subscribe via the SMF. NWDAF send a Nnrf_NFDiscovery_Request to NRF including the related discovery parameters and the NRF returns the SMF(s) which meet(s) the discovery request. NWDAF executes then step 3, including in this case as subscription target “AnyUE” and SMF selects the UEs to which the request applies and executes steps 4-5 in figure 4.15.4.x.2-1 for each of them.

When the NWDAF subscription request meets the criteria for direct subscription to UPF as described in 4.15.11.1, NWDAF queries the NRF including the related discovery parameter. The NRF returns the UPF(s) which meet(s) the discovery request, and NWDAF sends the Nupf_EventExposure_Subscription directly to UPF.

NOTE 1:	Requests for “Any UE” may cause significant load and signalling increase. This could be mitigated via utilization of some event filters (e.g. Area of Interest), Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information.

FIRST CHANGE

4.15.4.x	Exposure of Events from UPF for UPF Data Collection

4.15.4.x.1	General

This clause contains the detailed description and the procedures for how the UPF event exposure service (see 5.2.26.2) is used for UPF data collection.

The NWDAF can subscribe to the UPF exposure events for data collection via the SMF, and the UPF will directly send the collected UPF data to NWDAF. Or, the NWDAF can subscribe directly to UPF.

The Subscription to UPF events via SMF is the rule though there are some exceptions listed below; Subscription via SMF means the final consumer of UPF event notifications sends the subscription request to the SMF and then the SMF is doing a third-party subscription onto UPF on behalf of this final consumer. Table 4.15.4.x.1-1 details which subscription parameters apply to direct subscription to UPF events and to subscription to UPF via SMF. Conversely the notifications are directly sent by the UPF to the final consumer of UPF notifications.

Exceptions to subscriptions via SMF are subscriptions for data collection where UPF is the source as defined in TS 23.288 [5] in the following cases:

-	Data collected by NWDAF is for UPF load analytics described in Table 6.5.2-2 of TS 23.288 [5].

-	Data Collected by NWDAF is targeting "any UE" (possibly for specific DNN and or slices) and not related with an AoI or BSSID/SSID or application.

The UPF event exposure events are described in clause 5.2.26.2.  In this release of the specification, following events are used for UPF Data collection:

-	QoS Monitoring. This event provides QoS Flow performance information

-	UserDataUsageMeasures. This event provides information of user data usage of the User PDU Session

-	UserDataUsageTrends. This event provides statistics related to user data usage of the User PDU Session

NWDAF can subscribe to QoS monitoring event via SMF only and UPF sends the QoS Flow Performance information directly to NWDAF. For this event, the interaction between SMF and UPF is over PFCP (TS 29.244 [69]). NWDAF subscription request can trigger in SMF end to end UL/DL delay measurements but only for QoS flows that have already been established. Clause 5.33.3 describes how end to end UL/DL delay measurements for QoS flows are activated and measured triggered by a PCC received in SMF. By default, the Subscription request for QoS monitoring event refers to the QoS Flow associated to the default QoS rule. The subscription can target the QoS flows bound to an application by including an application identifier. In SMF, that identifier is compared to the UPF Exposure Correlation Identifier in the PCC rule (s) that control QoS for the application traffic (see TS 23.503 [20] clause 6.1.3.22). This procedure allows SMF to determine the PCC rules relevant for this request and the QoS flows to monitor.

NOTE 1:	Extensive usage of QoS Monitoring has significant impact on load and signalling.

NWDAF can subscribe to User Data Usage events directly to UPF or via SMF, and UPF sends the event notifications directly to NWDAF. For this event, the interaction between SMF and UPF is over SBI. For User Data Usage events, the subscription request may target specific service data flows (e.g. a specific application traffic) by including a traffic description (e.g. an Application Id). Else, the scope of the subscription is all the traffic in the PDU Session. The subscription request may indicate certain granularity for the information e.g. how many differentiated measurements are requested: one measurement that is considering all the target traffic, measurements per PDR that are based on the service data flows matching the PDR, measurements per service data flow.

NOTE 2:	PDR Provisioning and PDR matching have an impact on signalling and load in UPF..

Table 4.15.4.x.1-1: Input parameters in subscription to UPF event Exposure events

| Information | To SMF | To UPF |
| --- | --- | --- |
| UE ID | Y | Y |
| GroupID | Y | N |
| AnyUE | Y | Y |
| DNN | Y | Y |
| S-NSSAI | Y | Y |
| AOI | Y | N |
| BSSID/SSID | Y | N |
| DNAI (NOTE3) | Y | N |
| UPFId (NOTE 3) | Y | N |
| Type of Measurement | Y | Y |
| Granularity of Measurement (NOTE 2) | Y | Y |
| Application ID (NOTE 1) | Y | Y |
| Traffic Filtering (NOTE 1), (NOTE 2) | Y | Y |
| NOTE 1: Application ID and Traffic Filtering are exclusive NOTE 2: This parameter does not apply to QoS monitoring event NOTE 3: These parameters are used to indicate a UP path |  |  |

4.15.4.x.2	Information flow for UPF event exposure service to NWDAF for certain UE(s)

Figure 4.15.4.x.2-1: UPF Information Exposure to NWDAF

In the case of a group of UEs, the NWDAF issues an Nnrf_NFDiscovery_Request service operation to find the UDM providing the NF type and Group ID and gets the NF profile of the UDM that currently meets the NWDAF discovery request. Then, NWDAF obtains the list of SUPIs that correspond to the Group ID from UDM using Nudm_SDM_Get

NOTE 1:	It is assumed that all members of a Group ID belong to the same UDM.

Then, for each SUPI,

1.	The NWDAF invokes Nudm_UECM_Get service operation to retrieve the appropriate SMF by providing UE ID and NF type, to request UPF information Exposure for a certain UE.

2.	The UDM provides a Nudm_UECM_Get response to the NWDAF with the corresponding SMF.

3.	The NWDAF sends the request to the SMF to subscribe UPF data, including the following information:

-	Notification Target Address (NWDAF address).

-	Indication of UPF Event Exposure Service.

-	Event Filter Information: S-NSSAI, DNN, DNAI, UPFId, Traffic Description for the target traffic (e.g. Application Id), Area of Interest, SSID/BSSID.

-	Target of Event Reporting: a UE.

-	Target Subscription information: event, Type of Measurement and granularity of the Information requested.

4.	The SMF selects the PDU session(s) and the UPFs it has to send the request to. SMF sends the request to the UPF including the NWDAF address, Event Filter Information, Target of Event Reporting, and Target Subscription Information as required. Target of Event Reporting is certain UE.. The interaction mechanism used between SMF and UPF depends on UPF exposure event (see clause 5.2.26.2.1). It could be as in 4a with N4 Session Modification with PFCP (TS 29.244 [69]) or as in 4b with Nupf_event exposure subscribe request.

NOTE 2:	Some events may require SMF interacts with RAN at this stage.

5.	The UPF responds with the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the NWDAF.

4.15.4.x.3	Information flow for UPF event exposure service to NWDAF for anyUE

When the NWDAF subscription request does not meets the criteria for direct subscription to UPF as described in 4.15.4.x.1, NWDAF shall subscribe via the SMF. NWDAF send a Nnrf_NFDiscovery_Request to NRF including the related discovery parameters and the NRF returns the SMF(s) which meet(s) the discovery request. NWDAF executes then step 3, including in this case as subscription target “AnyUE” and SMF selects the UEs to which the request applies and executes steps 4-5 in figure 4.15.4.x.2-1 for each of them.

When the NWDAF subscription request meets the criteria for direct subscription to UPF as described in 4.15.11.1, NWDAF queries the NRF including the related discovery parameter. The NRF returns the UPF(s) which meet(s) the discovery request, and NWDAF sends the Nupf_EventExposure_Subscription directly to UPF.

NOTE 1:	Requests for “Any UE” may cause significant load and signalling increase. This could be mitigated via utilization of some event filters (e.g. Area of Interest), Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information.

END of CHANGE

NEXT CHANGE

END of CHANGE

NEXT CHANGE

5.2.8.3	Nsmf_EventExposure Service

5.2.8.3.1	General

Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on PDU Sessions. The following are the key functionalities of this NF service.

-	Allow consumer NFs to Subscribe and unsubscribe for an Event ID on PDU Session(s);

-	Allow the NWDAF to collect data for network data analytics from SMF as specified in TS 23.288 [50] and from UPF as specified in 4.15.4.x;

-	Notifying events on the PDU Session to the subscribed NFs; and

-	Allow consumer NFs to acknowledge or respond to an event notification.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	UE IP address / Prefix allocation/change: The event notification may contain a new UE IP address / Prefix or an indication of which UE IP address / Prefix has been released.

-	PDU Session Establishment and/or PDU Session Release.

The event notification may contain following information:

-	PDU Session Type.

-	DNN.

-	UE IP address/Prefix.

-	UP path change: a notification corresponding to this event is sent when the UE IP address / Prefix and / or DNAI and /or the N6 traffic routing information has changed.

The event notification may contain following information:

-	the type of notification ("EARLY" or "LATE").

-	for both the source and target UP path between the UE and the DN, the corresponding information is provided when it has changed:

-	DNAI.

-	UE IP address / Prefix.

-	N6 traffic routing information.

NOTE 1:	UP path change notification, DNAI and N6 traffic routing information are further described in clause 5.6.7 of TS 23.501 [2].

-	QoS Monitoring for URLLC: the event notification may contain the QoS Monitoring report as described in clause 5.33.3.2 of TS 23.501 [2]. Implicit subscription of the PCF on behalf of the NEF/AF as part of setting PCC rule(s) may trigger SMF to send this event notification.

-	Change of Access Type; The event notification contains the new Access Type for the PDU Session.

-	Change of RAT Type; the event notification contains the new RAT Type for the PDU Session.

-	PLMN change; The event notification contains the new PLMN Identifier for the PDU Session.

-	Downlink data delivery status. The event notification contains the status of downlink data buffering in the core network including:

-	First downlink packet per source of the downlink IP traffic in extended buffering and Estimated maximum wait time.

-	First downlink packet per source of the downlink IP traffic discarded.

-	First downlink packet per source of the downlink IP traffic transmitted after previous buffering and/or discarding of corresponding packet(s).

-	QFI allocation: The event notification is sent when a new QoS flow is established within a PDU session and contains:

-	If the Target of Event Reporting is a PDU session, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The DNN, S-NSSAI corresponding to the PDU session are also sent.

-	If the Target of Event Reporting is a SUPI, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set) for each PDU session ID established for this SUPI. The DNN, S-NSSAI corresponding to each PDU session are also sent.

-	If the Target of Event Reporting is an Internal-Group-Id or any UE, multiple instances of the tuple (allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). PDU session ID, SUPI). The DNN, S-NSSAI corresponding to each PDU session are also sent.

-	Total number of Session Management transactions:

-	The total number of Session Management transaction is used to collect the number of SM transactions of a SUPI or Internal Group ID, for example Dispersion Analytics as specified in TS 23.288 [50]. The transaction count is incremented when the NAS transactions from PDU Session Establishment, PDU Session Authentication, PDU Session Modification and PDU Session Release procedures is concluded. Only the periodic reporting mode applies.

-	Information on PDU Session for WLAN (i.e. Access Type is Non-3GPP and RAT Type is TRUSTED_WLAN).

NOTE 2:	When the consumer NF is the NWDAF, the event QFI allocation is used to collect data for Observed Service Experience analytics and UE communication analytics as specified in TS 23.288 [50].

-	User plane status information: The event notification contains:

-	PDU Session ID.

-	User Plane Inactivity Timer (as specified in TS 29.244 [65]).

-	PDU Session status (activated, deactivated).

NOTE 3:	When the consumer NF is the NWDAF, the event user plane status information is used to collect data for UE Communication analytics as specified in TS 23.288 [50].

-	Session Management Congestion Control Experience for PDU Session: The event notification contains the data related to Session Management Congestion Control experience per PDU Session as described in TS 23.288 [50].

-	UE session behaviour trends (see clause 4.15.4.3);

-	UE communications trends (see clause 4.15.4.3);

-	UP with redundant transmission: the event notification indicates if redundant transmission (see clause 5.33.2.2 of TS 23.501 [2]) has been activated or not for the PDU session; and

-	User Data Usage Measures and User Data Usage Trends (see clause 4.15.4.x): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF.

When the consumer NF is the NWDAF, the event Information on PDU Session for WLAN is used to collect data for WLAN performance analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the event Session Management Congestion Control Experience for PDU Session is used to collect data for Session Management Congestion Control Experience analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the events QoS Monitoring, User Data Usage Measures and User Data Usage Trends are used to collect data from UPF for analytics as specified in clause 4.15.4.x and in TS 23.288 [50].  SMF conveys the subscription to UPF on behalf of the NWDAF.

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for SMF exposure.

Table 5.2.8.3.1-1: Example of Event Filters for SMF exposure events

| Event ID for SMF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| DNAI Change | None |
| PDU Session Release | <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| PDU Session Establishment | <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QoS Monitoring for URLLC | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = Flow Info, Value = Packet Filter Set1> <Parameter Type = AoI, value = AoI1> <Parameter Type = SSID/BSSID, Value = SSID/BSSID1> |
| QFI allocation | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QFI allocation | <Parameter Type = Application Identifier, Value = Application Identifier1> |
| Transaction Count | <Parameter Type = TAI, Value = TA1> (NOTE) <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| User plane status information | <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = SUPI, Value = SUPI1> |
| Information on PDU Session for WLAN | <Parameter Type = Access Type, Value = Non-3GPP> && <Parameter Type = RAT Type, Value = TRUSTED_WLAN> |
| Session Management Congestion Control Experience for PDU Session | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| UP with redundant transmission | <Parameter Type = DNN, Value = DNN1> |
| NOTE: Optionally the SMF can fetch the location information from the AMF but transaction information correlation at the location can also be achieved without it and through transaction information associated with the requested time period, which corresponds to the UE's time span at the location of interest. |  |

The target of SMF event reporting may correspond to a PDU Session ID, an UE ID (SUPI), an Internal Group Identifier, an indication that any UE is targeted (e.g. on a specific DNN), or an indication that any PDU session is the target.

When acknowledgment is expected the SMF also provides Notification Correlation Information to the consumer NF in the event notification.

The consumer NF may provide the following event-specific information when acknowledging an event notification:

-	For UP path change event:

-	N6 traffic routing information related to the target DNAI.

NOTE 4:	Acknowledgement to a UP path change event notification is further described in clause 5.6.7 of TS 23.501 [2].

5.2.8.3.2	Nsmf_EventExposure_Notify service operation

Service operation name: Nsmf_EventExposure_Notify

Description: Report UE PDU Session related event(s) to the NF which has subscribed to the event report service.

Input Required: Event ID, Notification Correlation Information, UE ID(s) (SUPI(s) and if available GPSI(s)), PDU Session ID(s), time stamp.

SMF reports multiple PDU Sessions events when those happen at the same time and as indicated in the time stamp. The SMF reports the PDU Session ID, SUPI and if available GPSI(s) per each PDU Session event.

Input, Optional: Event specific parameter list as described in clause 5.2.8.3.1, capability of supporting EAS IP replacement in 5GC.

Output Required: Result Indication.

Output, Optional: Redirection information.

When the SMF detects the event subscribed by the NF consumer, the SMF reports the subscribed event together with the Notification Target Address (+ Notification Correlation ID) to the Event Receiving NF.

The optional event specific parameter list provides the values that matched for generating the event notification. The parameter values to match are specified during the event subscription (see clause 5.2.8.3.3).

See clause 4.3.6.3 for details on usage of this service operation toward Application Function.

If the NF consumer is AMF and the result of the service operation fails, the AMF shall set corresponding cause value in result indication which can be used by the SMF for further action. If the related UE is not served by the AMF and the AMF knows which AMF is serving the UE, the AMF provides redirection information which can be used by the SMF to resend UE related message to the AMF that serves the UE.

NOTE:	In the case of UP plane path, as described in clause 4.3.6.2, this notification can be the result of an implicit subscription of the NEF/AF by the PCF as part of setting PCC rule(s) via the Npcf_SMPolicyControl service (see clause 5.2.5.4).

5.2.8.3.2A	Nsmf_EventExposure_AppRelocationInfo service operation

Service operation name: Nsmf_EventExposure_AppRelocationInfo

Description: Acknowledge the notification from the SMF regarding UE PDU Session related event(s).

Input Required: Notification Correlation Information, cause code.

The Notification Corrrelation Information is provided by the SMF in the event notification.

Cause code indicates this acknowledgement is positive or negative.

Input, Optional: Event specific parameter list as described in clause 5.2.8.3.1, Indication that buffering of uplink traffic should start, Information for EAS IP Replacement in 5GC.

Output Required: None.

Output, Optional: None.

See clause 4.3.6.3 for details on usage of this service operation for example for the usage of the Indication that buffering of uplink traffic should start.

5.2.8.3.3	Nsmf_EventExposure_Subscribe service operation

Service operation name: Nsmf_EventExposure_Subscribe.

Description: This service operation is used by an NF to subscribe or modify a subscription for event notifications on a specified PDU Session or for all PDU Sessions of one UE, group of UE(s) or any UE.

Input, Required: NF ID, Target of Event Reporting as defined in clause 5.2.8.3.1, (set of) Event ID(s) defined in clause 5.2.8.3.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Input, Optional: Event Filter(s) associated with each Event ID; Event Filter(s) are defined in clause 5.2.8.3.1, Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, DNN, S-NSSAI, traffic filtering information, Type of measurement, granularity of measurement as described in clause 4.15.4.x.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1).

Notification Target Address (+ Notification Correlation ID) and  is used to correlate Notification sent by SMF or UPF  with this subscription.

5.2.8.3.4	Nsmf_EventExposure_UnSubscribe service operation

Service operation name: Nsmf_EventExposure_UnSubscribe.

Description: This service operation is used by an NF to unsubscribe event notifications.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

END of CHANGE

NEXT CHANGE

5.2.26.2	Nupf_EventExposure Service

5.2.26.2.1	General

Service description: This service can expose UPF related information to other NFs. There are several operations for this service:

-	Notifying events on the PDU Session to the NFs.

-	Allow consumer NFs to subscribe and unsubscribe for an Event ID on UPF.

-	Allow consumer NFs to acknowledge an event notification.

The following events can be notified to a NF consumer:

-	QoS Monitoring. This event provides QoS Flow level performance information. For example, it can be used for URLLC: the event notification may contain the QoS Monitoring report as described in clause 5.33.3.2 of TS 23.501 [2], or it may be for UPF Data collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.x.

Subscription to this event is always indirect via SMF. The subscription specifies the type of measurement that is being requested. A combination of the information listed below can be requested.

UPF and SMF interact using Session Reporting Rules as defined in TS 23.501 [2] clause 5.8.2.11.11

The event notification may contain following information:

-	QoS monitoring result e.g. end to end delay for specific QoS flow or for specific PDU session.

-	QoS Flow Bandwidth measurements: It provides bitrate measurements (UL, DL and/or overall) for a PDU Session and QoS Flow.

QoS Monitoring event can also be notified in case of an implicit subscription of the PCF on behalf of the NEF/AF as part of setting PCC rule(s) via the Npcf_SMPolicyControl service. See TS 23.548 [130] clause 6.4.

-	UserDataUsageMeasures. This event provides information of user data usage of the User PDU Session. It can be used for UPF Data Collection by NWDAF for analytics (see TS 23.288 [50]) as described in 4.15.4.x.

SMF and other allowed direct consumers shall use SBI subscription operation to subscribe this UPF event. The subscription specifies type of measurement that is being requested and for which traffic, so as the required granularity for the information reported. A combination of the information listed below can be requested.

The event notification may contain following information:

-	Volume Measurement: measures of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) with or without application granularity. This measurement can also include number of packets transmitted for applications where that is possible to differentiate.

-	Throughput Measurement: measures of data throughput (UL and DL) measures aggregated for the PDU Session or per application.

-	Application related Information: URL/s and/or Domain name/s detected in the PDU Session for the target traffic.

When the subscription requests that information is provided per service data flow, the notification includes the packet filter set and the Applications Identifier if available.

-	UserDataUsageTrends. This event provides statistical measurements. It can be used for UPF Data Collection by NWDAF for analytics (see TS 23.288 [50]) as described in 4.15.4.x.

SMF and other allowed direct consumers shall use SBI subscription operation to subscribe this UPF event. The subscription specifies for which traffic the information is requested, so as the required granularity for the information reported.

The event notification may contain following information:

-	Throughput Statistic Measurement (average and/or peak throughput) over the measurement period for the PDU Session or per application.

When the subscription requests that information is provided per service data flow, the notification includes the packet filter set and the Applications Identifier if available.

-	TSC management information (UMIC, PMIC, NW-TT port number) as defined in clause 5.8.2.11.14 of TS 23.501 [2].

TSC management information event can be the result of an implicit subscription of the PCF on behalf of the TSNAF/TSCTSF via the Npcf_SMPolicyControl service as described in TS 23.501 [2] clause 5.28.

If the consumer of UPF service is NWDAF and the target of UE is any UE, according to the Analytic ID from consumer, the NWDAF can decide which kind of information should be collect from UPF and event ID to use. Subscription may be performed directly towards UPF or via SMF as described in clause 4.15.11. The UPF collects the data according to the event ID, and exposes the related information directly regardless of whether the subscription has ben relayed by the SMF.

5.2.26.2.2	Nupf_EventExposure_Notify service operation

Service operation name: Nupf_EventExposure_Notify

Description: This service operation reports the event and information to the consumer that has subscribed implicitly.

Input Required: Event ID, UE address (i.e. IP address or MAC address). Notification Correlation Information.

Input, Optional: UE ID, Event specific parameter as described in clause 5.2.26.2.1, time stamps for the measures,  Application Id and Packet Filter Set.

Output Required: Result Indication.

Output, Optional: None.

5.2.26.2.3	Nupf_EventExposure_Subscribe service operation

Service operation name: Nupf_EventExposure_Subscribe

Description: This service operation is used by an NF to subscribe to UPF event exposure notifications e.g. for the purpose of UPF data collection on a specified PDU Session or for all PDU Sessions of one UE, group of UE(s) or any UE.

Input, Required: NF ID, Target of Event Reporting, (set of) Event ID(s) defined in clause 5.2.26.2.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, DNN, S-NSSAI, Application ID (s), traffic filtering information, Type of measurement, granularity of measurement, Reporting suggestion information.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1). Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by UPF with this subscription.

5.2.26.2.4	Nupf_EventExposure_UnSubscribe service operation

Service operation name: Nupf_EventExposure_UnSubscribe

Description: The NF consumer uses this service operation to unsubscribe for a specific event.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

The NF consumer or the SMF on behalf of other NF unsubscribes the event notification by invoking Nupf_EventExposure_Unsubscribe (Subscription Correlation ID) to the UPF.

END of CHANGES